3GPP TSG-RAN WG4 Meeting #94bis-e	R4-2004627

Online, 20th April – 30th April 2020

Source:	Ericsson, US Cellular

Title:	TP for TR 37.716-11-11: DC_12_n71

Agenda item:			8.3.2

Document for:	Approval

# 1. Introduction

Rel-16 LTE-NR DC combinations LTE 1 band + NR 1 band combination are defined in the WID [1]. This contribution is a text proposal for TR 37.716-11-11 to include DC_12_n71.

# 2. Text Proposal

# ---Start of changes---

### 6.1.x	DC_12A_n71A

#### 6.1.x.1	Operating bands for DC

Table 6.1.x.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_12_n71 | 12 | n71 | Yes1 |
| NOTE 1: Only single switched UL is supported |  |  |  |

#### 6.1.x.2	Configuration for DC

Table 6.x.2.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_12A_n71A2 | DC_12A_n71A1 | 12A | n71A |
| NOTE 1: Only single switched UL is supported NOTE 2: This combination is applicable only for non-form-factor constrained device use cases (e.g. customer premise or vehicle-mounted devices) |  |  |  |

#### 6.1.x.3	Maximum output power for DC

Note that only Single Tx Switched UL mode is supported for this combination, no additional Maximum output power for DC requirement is needed.

#### 6.1.x.4	Spurious emission band UE co-existence for DC

Note that only Single Tx Switched UL mode is supported for this combination, no additional spurious emission band UE co-existence requirement is needed

#### 6.1.x.5	MSD analysis for DC

Note that only Single Tx Switched UL mode is supported for this combination, no MSD requirement is needed

### 6.1.x.6	∆TIB and ∆RIB values

Triplexer is assumed where the UL’s of band 12 and n71 are combined.

For DC_12_n71, the TIB,c and RIB,c values are reused from CA_8-20 and are given in the tables below.

# Table 6.1.x.6-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12_n71 | 12 | 0.4 |
|  | n71 | 0.4 |

# Table 6.1.x.6-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_12_n71 | 12 | 0 |
|  | n71 | 0 |

### 6.1.x.7	MSD

Note that only Single Tx Switched UL mode is supported for this combination, no MSD requirement is needed

---End of changes---

# Reference

[1]		RP-192742, “Revised WID on EN-DC for 2 bands DL with 2 bands UL (1 LTE band + 1 NR band)”, CHTTL