3GPP TSG-RAN WG4 Meeting #94bis	R4-2004803

E-meeting, 20 April – 1 May, 2020

Agenda item:	6.5.3.5

Source: 	Qualcomm Incorporated

Title: 	IAB-MT Link Recovery requirements

Document for:	Discussion

# Introduction

During the last meeting, RAN4 made a couple of agreements regarding candidate beam detection. Several issues are still open for discussion. We discuss those issues and provide our opinions in this contribution.

# Sharing factor of CBD evaluation period

During the last meeting, there was a proposal to remove the sharing factor P from CBD evaluation periods [2]

Open Issues from [1] Issues: Sharing factor P. Proposals: Remove the sharing factor P in Evaluation Period calculation for IAB BFD/CBD requirement.

The motivation of this proposal is the following: since requirements for intra-frequency and inter-frequency measurements have not been configured for IAB-MTs, configuration of measurement gap is not necessary. This means that the sharing factor P will not be necessary, too.

However, although RAN4 has not allowed measurement requirements, the signalling for intra and inter-frequency measurement is still allowed by RAN1/RAN2. As a result, network can configure these measurement gaps and the IAB-MTs that support this feature can follow those configurations. Hence, sharing factor P does not need to be removed from the spec. If network doesn’t configure measurement gap due to the lack of RAN4 requirements, value of P will be 1, in any case.

Observation 1: Although RAN4 has not defined cell level measurement requirements, network can still configure these measurement gaps and the IAB-MTs that support this feature can follow those configurations.

Proposal 1: Sharing factor P does not need to be removed from the BFD/CBD evaluation period of IAB-MTs.

# Beam Sweeping Factor in CBD evaluation period

During the last meeting, there was a proposal to reduce the beam sweeping factor in FR2.

Open Issues from [1] Issues: Beam sweeping factor Proposal: For IAB CBD requirement, beam sweeping factor N=8 in Evaluation Period calculation for FR2 should be reduced.

Rel-16 IAB-MTs are fixed nodes. Once they lose their BFD-RS, they may have better understanding about the angle of arrival for the CBD-RS. But, IAB-MTs may need to use narrower RX beams to obtain higher link budget. Besides, the beam sweeping factor for fixed UEs (power class 1) was also decided to be 8 in Rel-15. Hence, same beam sweeping factor (N = 8) should be used to evaluate CBD-RS for IAB-MTs.

Observation 2: IAB-MTs may need to use narrower RX beams to obtain higher link budget.

Observation 3: the beam sweeping factor for fixed UEs (power class 1) was also decided to be 8 in Rel-15.

Proposal 2: For IAB CBD requirement, use beam sweeping factor N=8 in Evaluation Period calculation.

# Evaluation Period for CBD-RS

Proposal 3: Reuse the SSB and CSI-RS based CBD evaluation period of Rel-15 UEs to define the requirements for IAB-MT nodes.

# Other Requirements for CBD

Other CBD requirements (e.g. scheduling availability, measurement restrictions, etc.), that have been defined in 38.133 for Rel-15 UEs, can be reused for IAB-MTs.

Proposal 4: The following CBD related requirements, that have been defined in 38.133 for Rel-15 UEs, can be reused for IAB-MTs.

- Measurement restrictions for SSB based CBD and CSI-RS based CBD

- Scheduling availability during CBD

# Conclusion

Observation 1: Although RAN4 has not defined cell level measurement requirements, network can still configure these measurement gaps and the IAB-MTs that support this feature can follow those configurations.

Observation 2: IAB-MTs may need to use narrower RX beams to obtain higher link budget.

Observation 3: the beam sweeping factor for fixed UEs (power class 1) was also decided to be 8 in Rel-15.

Proposal 1: Sharing factor P does not need to be removed from the BFD/CBD evaluation period of IAB-MTs.

Proposal 2: For IAB CBD requirement, use beam sweeping factor N=8 in Evaluation Period calculation.

Proposal 3: Reuse the SSB and CSI-RS based CBD evaluation period of Rel-15 UEs to define the requirements for IAB-MT nodes.

Proposal 4: The following CBD related requirements, that have been defined in 38.133 for Rel-15 UEs, can be reused for IAB-MTs.

- Measurement restrictions for SSB based CBD and CSI-RS based CBD

- Scheduling availability during CBD

# Reference

- R4-2002307, “Email discussion summary for RAN4#94e_#53_NR_IAB_RRM”, Qualcomm Incorporated.

- R4-2000889, Samsung.