3GPP TSG-RAN WG4 Meeting #94-e-Bis	R4-2004870

Online Meeting, 20-30 Apr 2020

Title:	FR2 DL Intra-band CA BW Enhancement Feature Completion

Source:	Qualcomm Incorporated

Agenda item:	6.14.1.3

Release:	Rel-16

Work Item:	NR_RF_FR2_req_enh

Responsible WG:	RAN4

Document for:	Discussion

# 1.	Introduction

RAN4 established that the DL CA BW enhancement feature for Rel-16 would be captured as a UE capability to support DL-only coverage spectrum [1]. ‘DL Coverage Spectrum’ here means the range of DL frequencies that can be populated with a DL CC in CA mode. RAN4 has also partially converged on signalling framework to support this feature [2]

In this contribution, we propose completion of the feature by converging on the last remaining details and initiating LS to RAN2.

# 2. 	Discussion

For Rel-16, the DL CA BW enhancement feature establishes two distinct types of UE DL spectrum capability [1]:

- Carried over from Rel-15: Bidirectional spectrum, where a UE can support a DLCC with the option of a same-frequency UL CC

- New to Rel-16: DL-only spectrum where a UE can support a DLCC but cannot support a matching UL CC

For productive discussion, it helps to review the foundation (i.e. existing agreements) established in RAN4 for this feature.

## 2.1 	Review of Existing Agreements

### 2.1.1	Nature of enhancement

The enhancement is the UE’s ability to support DL-only spectrum in addition to the its combined bidirectional spectrum capability which carries over from Rel-15. As captured in [1]:

### 2.1.2	UE’s DL spectrum capability is contiguous

The UE’s DL spectrum capability is its combined capability to support a bidirectional spectrum and a separate DL-only spectrum. There is agreement that this combined DL capability is contiguous for this feature. As captured in [1]:

### 2.1.3	UE’s DL-only spectrum capability

In [2], the DL-only spectrum capability of the UE was narrowed down to one of 2 alternatives (and the inclusive option, which represents a third possibility). All others are precluded per the agreement

This DL-only spectrum type represents the only remaining UE capability detail to be agreed in RAN4 Based on infra preference, and flexibility in real-world use [3], we propose selection of the single sided DL-only spectrum. The down-selection is shown graphically in figure 2.1.3-1

Figure 2.1.3-1: DL-only spectrum choices from [2], and proposed down-selection

## 2.2 Signalling requirement

The down-selection above leaves just one parameter that the UE must declare as its capability, the width of the spectrum covered with DL-only capability. This parameter can adopt signalling very similar to that used by the frequency separation parameter. The WF [2] leaves RAN4 with 2 options to signal the UE’s capability.

Given the simplicity of the capability parameter, ‘Alt 2’ represents unnecessary complexity. Alt 2 also has numerous disadvantages in context of the agreements in place for this feature, as explained in [3]. We hence propose the simplest solution that works, Alt1. Note that the suitability of Alt1 does not change even if any of the other allowed DL-only spectrum capability configurations were to be chosen (both-sided, or inclusive)

Figure 2.2-1: DL-only spectrum capability signalling choices from [2], and proposed down-selection

The proposals above are incorporated into a companion feature draft CR. An LS to RAN2 in draft form, with the down-selections discussed above, is in a companion contribution.

# 3.	Conclusion

For completion of the Rel-16 DL CA enhancement feature, RAN4 need to only down-select the nature of the supported DL-only spectrum capability. Based on previous agreements, two independent options were identified, with a third inclusive option. Based on infra preference, and flexibility in real-world use, we propose down-selecting DL-only spectrum capability to the ‘single-side option’. The choice of signalling parameter is simpler yet, with (Alt. 2) per-chain capability signalling providing no extra useful information to the network, at the expense of increased complexity.

# 4. 	References

[1]	R4-1913042, ‘WF on DL Intra-band CA BW Enhancement’, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #92-Bis, Chongqing, CN, Oct 2019

[2] 	R4-1916021, ‘WF on FR2 DL Intra-band CA BW Enhancement’, Qualcomm et al, 3GPP TSG-RAN WG4 Meeting #93, Reno, NV, USA, Nov 2019

[3] 	R4-2000207, ‘FR2 DL Intra-band CA BW Enhancement Feature Parameters’, Qualcomm Incorporated, RAN4#94-e