3GPP TSG-RAN WG4 Meeting #94bis-e                                                                                 R4-2004924

Electrical Meeting, 20th April– 30th April, 2020

Source:	Samsung

Title:	View on performance requirements for URLLC enhancement with multi-TRP/panel transmission

Agenda item:			6.11.3.2

Document for:	Approval

- Introduction

Rel-16 NR eMIMO WI is a RAN1 leading WI with below major enhancement in RAN1 area:

- Enhancements on MU-MIMO support

- Enhancements on multi-TRP/panel

- Enhancements on multi-beam operation

  - L1-SINR measurement

  - Beam failure recovery for SCell

  - DL/UL beam indication with reduced latency and overhead

- Enhancement on low PAPR RS

- Enhancement on full Tx power uplink transmission

As agreed in RAN#85, Rel-16 NR eMIMO WI has below work objectives for RAN4 core part and performance part:

- Specify core requirements associated with the items specified by RAN1 [RAN4-Core]

  - Specify RRM requirements

  - Identify impact on RF requirements for the reduced PAPR pi/2-BPSK DMRS and, if needed, specify RF requirements

  - Identify impact on RF requirements for the uplink full power transmission and, if needed, specify RF requirements

- Specify necessary performance requirements for the specified enhancements. [RAN4-Performance]

During the last RAN4 meeting, the test scope for demodulation requirement of NR eMIMO was discussed and the related agreement was captured in WF [1].

In this contribution, the review on performance requirements impact for Rel-16 eMIMO WI was provided.

# Discussion

In Rel-16, based on an integrated framework of NR system, the detailed objectives for enhancements on multi-TRP/panel transmission are as follows:

- Enhancements on multi-TRP/panel transmission including improved reliability and  robustness with both ideal and non-ideal backhaul:

- Specify downlink control signalling enhancement(s) for efficient support of non-coherent joint transmission

- Specify enhancements on uplink control signalling and/or reference signal(s) for non-coherent joint transmission

- Multi-TRP techniques for URLLC requirements are included in this WI.

Based on RAN1 agreements, below multi-TRP/panel transmission supported in Rel-16:

- Multi-PDCCH based multi-TRP/multi-panel transmission (eMBB)

- Single-PDCCH based multi-TRP/multi-panel transmission (eMBB)

- URLLC enhancements with 5 schemes

URLLC enhancements schemes

Based on RAN1 agreement, 5 transmission schemes for URLLC enhancement based on multi-TRP transmission scheduled by single-PDCCH were introduced to improve the reliability and robustness with following

- Schemes 1(SDM):  2 TCI states within the single slot, with overlapped time and frequency resource allocation

- Schemes 2a/2b(FDM): 2 TCI states within the single slot, with non-overlapped frequency resource allocation

  - Scheme 2a: 	Single codeword with one RV is used across full resource allocation. From UE perspective, the common RB mapping (codeword to layer mapping as in Rel-15) is applied across full resource allocation.

  - Scheme 2b: Single codeword with one RV is used for each non-overlapped frequency resource allocation. The RVs corresponding to each non-overlapped frequency resource allocation can be the same or different. Additional UE capability is specified to inform gNB whether the UE can support CW soft combining.

- Schemes 3 (TDM within slot): 2 TCI states within the single slot, with non-overlapped time resource allocation

- Scheme 4 (TDM with slots): 2 TCI states with K (n<=K) different slots, K can be configured up to 16.

Scheme 1, 2a/2b have the similar structure with eMBB NC-JT, such as TCI indication framework for singe PDCCH.  Among these transmission schemes, scheme 1a is more typical scenario with considering overlapped time and frequency resource with two TCI states, it can be regarded as the enhancement of SFN JT transmission i based on Rel-15 from multi-TRP with single TCI state.

Scheme 3/4 has similar structure with mini-slot based repetition/ slot based repetition for single TRP transmission, discussed in Rel-16 URLLC enhancement and Rel-15 URLLC, separately.

For URLLC enhancement, considering a parallel discussion on introducing Rel-15/Rel-16 URLLC performance requirement is still on-going under Rel-16 URLLC enhancement WI. The test methodology and proper metric is still FFS, we can consider to deprioritize URLLC enhancement requirements under eMIMO WI in initial stage.

As agreed in the last meeting, there is no requirement for URLLC multi-TRP transmission schemes with reliability transmission with lower BLER test metric (e.g., lower than 1% BLER) in Rel-16 NR eMIMO WI. Regarding the requirements for URLLC multi-TRP transmission schemes with eMBB test metric, the current conclusion is still FFS.

Generally, the schemes1, 2a/2b and 3 can be covered with single-PDCCH based multi-TPR/panel transmission. Considering the tight scheduling and RAN4 workload for NR eMIMO WI, if requirements for URLLC multi-TRP transmission schemes with eMBB metric needed, we prefer to focus on the requirement of SDM (scheme 1a) firstly.

Proposal 1: If requirements for URLLC multi-TRP transmission schemes with eMBB metric needed, prioritize with transmission scheme with SDM (scheme 1a), FFS on scheme 2a/2b

Proposal 2: Deprioritize URLLC enhancement with multi-TRP/multi-panel transmission for transmission schemes with TDM (scheme 3/4), pending on progress on performance requirements of Rel-16 URLLC WI.

Regarding details test case of URLLC transmission, most of test parameters can be reused for multi-TRP/panel NC-JT transmission. From the schedule and RAN4 workload, we suggest to deprioritize detailed test case design for URLLC enhancement with NCJT Multi-TRP/Panel transmission after we have stable test case design for multi-TRP/panel NC JT transmission.

# 3. Conclusion

In this contribution, the view on PDSCH performance requirements based on multi-TRP/Panel transmission for Rel-16 eMIMO was provided.

Proposal 1: If requirements for URLLC multi-TRP transmission schemes with eMBB metric needed, prioritize with transmission scheme with SDM (scheme 1a), FFS on scheme 2a/2b

Proposal 2: Deprioritize URLLC enhancement with multi-TRP/multi-panel transmission for transmission schemes with TDM (scheme 3/4), pending on progress on performance requirements of Rel-16 URLLC WI.

# 4. Reference

[1] R4-1911404, “Work plan for Rel-16 NR eMIMO WI“, Samsung

[2] R4-2002419, WF on demodulation and CSI requirement for NR eMIMO. Samsung