# 3GPP TSG-RAN WG4 Meeting #94-e-Bis		                                                    R4-2004959

# Electronic Meeting, 20 – 30 Apr, 2020

Title:                      Proposal on power class 2 UE requirements

Source: 	CMCC

Agenda Item:	4.4.2.1

Document for:	Approval

# 1    Introduction

RAN4 discussed the Power class 2 UL-MIMO clarification and other related requirements for 5G NR, and no conclusion was reached in last RAN4 /RAN meeting. Considering this controversial topic, the purpose of this contribution is to identify which issues need to be fixed and which are not necessary to discuss in Rel-15.

# 2    Discussion

# 2.1 Power class ambiguity for PC2 UE with 2Tx

As stated in WF [1], RAN4 reach a consensus on clarify in the Rel-15 specification on ambiguous requirements for UE supporting UL MIMO.

- Clarify in the Rel-15 specification on ambiguous requirements for UE supporting UL MIMO

  - Maximum output power

    - Clarity in the spec that if PC2 UE is configured for transmission on single-antenna port, the requirements of the same power class in subclause 6.2.1 apply for the UE.

    - How to configure for transmission on single-antenna port is up to UE implementation

  - Out of band emission

    - Clarity in the spec that for PC2 UE supporting UL MIMO, the ACLR requirement for PC3 UE shall apply to each antenna connector with the MIMO configuration defined in current specification.

    - Further consider UE capability signalling in Rel-16.

First of all, the definition of Power class 2 UE is clear and should not be disputed. This should be a fundamental understanding that the UE can be defined as PC2 only if the maximum output power is +26dBm in both UL-MIMO and single antenna port mode. Power class 2 should be the same for both UL-MIMO and single antenna port mode.

Secondly, we need to ensure that the MOP of PC2 UE is still +26dBm when UL-MIMO mode fallback to single antenna port mode. In order to ensure the network coverage performance, operators need the type of PC2 UE which has the same power class in the initial access and connection states. For the MOP of UE which cannot meet PC2 in single antenna port mode, such UE should be indicated as PC3.

Proposal 1: It is no necessary for RAN4 to clarify the definition of PC2 UE. There is no ambiguity in the definition of power class in the current spec, and UE can be defined as PC2 only if the maximum output power is +26dBm in both UL-MIMO and single antenna port mode.

# 2.2 PA Architecture for PC2 UE with 2Tx

As we know, RAN4 specification does not explicitly define the RF architecture implementation of PC2 UE with 2Tx. From CMCC perspective, we definitely want to see UEs that can declare PC2 through 26dBm PA including 23+26dBm or 26+26dBm, and it's easy for such PC2 UE to support PC2 when it backs up to a single antenna port mode. However, according to current spec and WF [2], such UE 23+23dBm PA architecture cannot be excluded in Rel-15.

The following PA architectures for PC2 UE with 2TX are included in Rel-15:

- 23dBm+26dBm

- 26dBm+26dBm

- 23dBm+23dBm

The controversial issue is whether this type of 23+23 UE with 2Tx can transmit +26dBm in single antenna port mode. For the implementation of TxD, We agree that “No specific requirements for TX diversity is written in RAN4 specification” in WF [3], but this does not mean that RAN4 does not support TXD implementation, as stated in WF [2], How to configure for PC2 transmission on single-antenna port is up to UE implementation.

Proposal 2: It is no necessary for RAN4 to clarify the PA architecture of PC2 UE. The PA Architecture for PC2 UE with 2Tx including 23dBm+26dBm, 26dBm+26dBm and 23dBm+23dBm in Rel-15.

# 2.3 MOP, SEM and MPR for PC2 UE with 2Tx

Considering the above two aspects, we do not think that RAN4 needs further clarification on this issue, but we can consider fixing the definition of the MOP, SEM and MPR for PC2 UE with 2Tx.

For UE with two transmit antenna connectors, the sum requirements of the two transmit antenna connectors should apply. As stated in Draft CR [4], in this case, it is possible to fix these requirements for PC2 UE with 2Tx to meet regulations without big changes in specification.

Proposal 3: For UE with two transmit antenna connectors, the sum requirements of the two transmit antenna connectors should apply.

# 3   Conclusions

In this contribution, we get the following proposals:

Proposal 1: It is no necessary for RAN4 to clarify the definition of PC2 UE. There is no ambiguity in the definition of power class in the current spec, and UE can be defined as PC2 only if the maximum output power is +26dBm in both UL-MIMO and single antenna port mode.

Proposal 2: It is no necessary for RAN4 to clarify the PA architecture of PC2 UE. The PA Architecture for PC2 UE with 2Tx including 23dBm+26dBm, 26dBm+26dBm and 23dBm+23dBm in Rel-15.

Proposal 3: For UE with two transmit antenna connectors, the sum requirements of the two transmit antenna connectors should apply.

# 4   References

R4-1816615, WF on PC2 UL MIMO, Huawei , HiSilicon

R4-1803259, Proposal on NR HPUE definition for PC2, CMCC

R4-1913067, Summary of Tx diversity and eMIMO full power transmission, Ericsson

R4-2003012, Draft CR for UL MIMO requirements, CMCC