3GPP TSG-RAN WG4 Meeting #94-e-Bis	R4-2004993

Electronic Meeting, 20 – 30 Apr., 2020

Title:	Discussion on FR2 intra-band DL CA

Source:	Samsung

Agenda item:	6.14.1.3

Document for:	Discussion

# 1.	Introduction

In the RAN#87e plenary meeting, FR2 intra-band contiguous DL CA is removed from R16 FR2 UE RF WID, so only non-contiguous scenario needs further discussion for intra-band DL CA. In RAN4#93 meeting, good progress is achieved by introducing DL-only spectrum, and then the work for intra-band NC DL CA is stuck to the DL-only spectrum type UE supported. In this paper we analyse the DL-only spectrum type issue and propose our view on it and on breaks in frequency for EIS relaxation table as well.

# 2. 	Discussion

## 2.1	DL-only spectrum type

It was agreed in the WF [1] in RAN4#93 that the DL-only spectrum type will be one of two options: either one-sided extension or two-sided extension relative to bidirectional spectrum:

- The spectrum covered by the DL-only frequency separation shall be located relative to bidirectional spectrum(*) in one of two configurations:

  - Extends on one side

  - Extends equally on both sides, half on each side

    - FFS if both-sided DL-only spectrum can be dropped for rel-16.

    - Single bit can be used to indicate configuration if both possibilities retained

Previous papers [2][3][4] has analysed the drawbacks of two-sided extension. If UE only supports two-sided extension, the UL spectrum allocation will be limited in the center of the whole frequency separation and side spectrum will always be unavailable for uplink when maximum cumulative aggregated BW is configured; Complicated network management and more network resource is expected for two-sided extension; the usage scenario is quite few, etc. It can be concluded that two-sided extension is not feasible unless the UE supports both one-sided extension and two-sided extension.

In RAN4#94e meeting, some company proposed the new option that UE supports both extension types (both one-sided and two-sided). However, the new option is not included in option list in the agreed WF [1]. Moreover, new signalling will be needed for UE to support both types, and network will consume more resources, while there is no obvious benefit compared with one-sided extension.

The typical implementation is that common LO for both UL and DL corresponding to Fs and dedicated LO for DL-only corresponding to Fsd. The one-sided extension is aligned with practical implementation. Though it is named as one-sided extension, in fact it can cover two-sided spectrum (not simultaneously) for DL CC based on network scheduling. Considering the agreement of “Fs>Fsd”, the one-sided extension is enough to cover all practical usage scenarios. It is not necessary for UE to support two-sided extension which will make things more complicated without benefits.

Proposal 1: DL-only spectrum (Fsd) shall be extended on one side relative to bidirectional spectrum (Fs).

## 2.2	breaks in frequency for EIS relaxation table

After the frequency separation is enhanced up to 2400MHz, the EIS relaxation table for ΔRIB is need to be revised to accommodate wider frequency separation.

In RAN4#94e meeting, two options were discussed:

  - Option 1: Treat 1400 to 2000 MHz range separately from 2000 to 2400 MHz

| Cumulative Aggregated Channel BW (MHz) | ΔRIB (dB) |
| --- | --- |
| ≤ 800 | 0.0 |
| > 800 and ≤ 1400 | 0.5 |
| > 1400 and ≤ 2000 | TBD |
| > 2000 and ≤ 2400 | TBD |

  - Option 2: Treat 1400 to 2400 MHz as one range

| Cumulative Aggregated Channel BW (MHz) | ΔRIB (dB) |
| --- | --- |
| ≤ 800 | 0.0 |
| > 800 and ≤ 1400 | 0.5 |
| > 1400 and ≤ 2400 | TBD |

Different from FR1, FR2 requirements are all OTA requirements.  Therefore the CA relaxation factors comprise conduction aspect and radiation aspect, moreover beam correspondence under CA status also degrades EIS performance as beam squint occurs for wider frequency separation [5]. Given above consideration, it seems too large range to treat 1400 to 2400 MHz as one range. So it is better to treat 1400 to 2400MHz as two separate ranges

Proposal 2: apply two separate ranges from 1400 to 2400MHz to EIS relaxation table for ΔRIB.

# 3. 	Conclusion

In this paper we propose our view on DL-only spectrum type and breaks in frequency for EIS relaxation table:

Proposal 1: DL-only spectrum (Fsd) shall be extended on one side relative to bidirectional spectrum (Fs).

Proposal 2: apply two separate ranges from 1400 to 2400MHz to EIS relaxation table for ΔRIB.

# 4. 	References

- R4-1916021 	“WF on FR2 DL Intra-band CA BW Enhancement”, Qualcomm, Apple, Nokia, Samsung

- R4-2001044 	“Discussion on FR2 intra-band DL CA enhancement”, Nokia

- R4-2000207 	“FR2 DL Intra-band CA BW Enhancement Feature Parameters”, Qualcomm

- R4-2000759 	“On FR2 DL intra-band CA cumulative aggregated BW enhancement”, MediaTek

- R4-2002826 	“WF on radiative degradation mechanisms for larger frequency separation”, Qualcomm