3GPP TSG-RAN WG4 Meeting #94bis-e	R4-2005074

Online, 20th April – 30th April 2020

Source:	Ericsson

Title:	TP to TR 37.716.21-21: Addition of DC configurations for DC_1_n28-n40

Agenda item:	8.7.2

Document for:	Approval

# Background

This text proposal for TR 37.716-21-21 to add DC_1_n28-n40 configurations as defined in WID [1].

# Text Proposal

##### ---Start of changes---

## 6.97	DC_1_n28-n40

### 6.97.1	Operating bands for DC

Table 6.97.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n28-n40 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

### 6.97.2	Configuration for DC

Table 6.97.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A_n28A-n40A | DC_1A_n28A DC_1A_n40A | 1A | CA_n28A-n40A |

### 6.97.3	Co-existence studies

Co-existence analysis for DC_1_n28 shows that there is IMD4 impact from DC_1_n28 UL to NR Band n40 DL.

Co-existence analysis for DC_1_n40 shows that there is IMD4 impact from DC_1_n40 UL to NR Band n28 DL.

### 6.97.4	∆TIB and ∆RIB values

For DC_1_n28-n40 the TIB,c and RIB,c values are same as for CA_1-28-40 and are given in the tables below.

Table 6.97.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n28-n40 | 1 | 0.6 |
|  | n28 | 0.3 |
|  | n40 | 0.5 |

Table 6.97.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n28-n40 | 1 | 0 |
|  | n28 | 0.2 |
|  | n40 | 0 |

### 6.97.5	MSD

Based on MSD analysis in [2], there is IMD4 produced by Band 1 and NR band n28 that impact the reference sensitivity of NR band n40. Also, there is IMD4 produced by Band 1 and NR band n40 that impact the reference sensitivity of NR band n28. These required MSD are shown in the following Table.

Table 6.97.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A_n28A-n40A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n40 | 2374 | 5 | 25 | 2374 | 10.1 | IMD4 |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n40 | 2314 | 5 | 25 | 2314 | N/A | N/A |
|  | n28 | 713 | 5 | 25 | 768 | 8.6 | IMD4 |

##### ---End of changes---

# Reference

[1]	RP-200080, “Revised WID on EN-DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)”, LG Electronics

[2]	R4-2002654, “MSD results for new EN-DC LTE(xDL/1UL)+ NR(2DL/1UL) DC in rel-16” , LG Electronics