3GPP TSG-RAN WG4 Meeting #94bis-e	R4-2005721

Online, 20th April – 30th April 2020

Source:	Ericsson, T-Mobile US, MediaTek

Title:	TP for TR 38.716-02-00: CA_n25-n46

Agenda item:			6.1.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.716-02-00 to include CA_n25-n46 as defined in WID [1].

# 2. Text Proposal

# ---Start of changes---

## 6.x	n25-n46

### 6.x.1	Common for 1 band UL and 2 bands UL CA

#### 6.x.1.1		Operating bands for CA

Table 6.x.1.1-1: CA band combination of band n25 and n46

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
| n46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |

#### 6.x.1.2		Channel bandwidths per operating band for CA

Table 6.x.1.2-1: Supported bandwidths per CA band combination of band n25 and n46

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n46A | - | n25 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n46 | 15 |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |
|  |  |  | 60 |  |  |  | Yes |  |  | Yes |  | Yes | Yes |  |  |  |

#### 6.x.1.3		UE co-existence studies

Table 6.x.1.3-1 lists up to 7th harmonics for n25A-n46A.

Table 6.x.1.3-1: Impact of UL/DL Harmonic

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  | 6th Harmonic |  | 7th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n25 | 1850 | 1915 | 3700 | 3830 | 5550 | 5745 | 7400 | 7660 | 9250 | 9575 | 11100 | 11490 | 12950 | 13405 |
| n46 | 5150 | 5925 | 10300 | 11850 | 15450 | 17775 | 20600 | 23700 | 25750 | 29625 | 30900 | 35550 | 36050 | 41475 |

#### 6.x.1.4		∆TIB and ∆RIB values

For CA_n25-n46, the TIB,c and RIB,c values are derived from LTE combination CA_25-46 and are given in the tables below.

# Table 6.x.1.4-1: ΔTIB,c

| NR CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n25-n46 | n25 | 0 |
|  | n46 | 0 |

# Table 6.x.1.4-2: ΔRIB,c

| NR CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n25-n46 | n25 | 0 |
|  | n46 | 0 |

#### 6.x.1.5 		REFSENS requirements

As can be seen in the co-existence studies in 6.x.1.3 there are 3rd harmonics issues from n25 UL into the n46 DL.

MSD for CA_n25-n46 is proposed to be defined like in table below, where Note 1 in Table 6.x.1.5-1 is derived from Note 5 in TS36.101 Table 7.3.1A-0eA.

The Table 6.x.1.5-2 is derived from TS36.101 Table 7.3.1A-0eC.

Table 6.x.1.5-1: Reference sensitivity QPSK PREFSENS (CA with n46)

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n25 | n46 |  |  |  | N/A |  | N/A |  | N/A | N/A |  |  |
| n46 | n25 | 26.5 | 26.5 | 26.5 | 26.5 |  |  |  |  |  |  |  |
| NOTE 1: These requirements do not apply when there is at least one individual RE within the downlink (victim) transmission bandwidth which falls into the reference sensitivity exclusion region as specified in Table 6.x.1.5-2. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.x.1.5-2: n46 Reference sensitivity measurement exclusion region in MHz

| Licensed Component Carriers / E-UTRA Band / Harmonic order / Channel BW in UL |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Band | Harmonic order | 5MHz | 10MHz | 15MHz | 20MHz | 40MHz |
| n25 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 | +/- 90 |
| NOTE 1: Even though UL harmonic does not fall directly into n46 the exclusion region still applies. NOTE 2: The center of the exclusion region is obtained by multiplying the uplink channel center frequency by the harmonic order. |  |  |  |  |  |  |

---End of changes---

# Reference

[1] 		RP-192926, “Revised WID on NR-based Access to Unlicensed Spectrum”, Qualcomm