3GPP TSG-RAN WG4 Meeting # 95-e	R4-2006109

Electronic Meeting, 25 May – 5 June, 2020

Agenda Item:	11.1.3

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Indoor Downlink Coexistence Simulation Results for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz

Document for:	Discussion

# 1.	Introduction

The study item on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz was approved at TSG RAN#87-e [1]. The purpose of this study item is to study the IMT parameters relevant for sharing and compatibility for the following frequencies:

-	6425-7025MHz and 7025-7125MHz.

-	10000-10500MHz.

This study item aims as answering requests from ITU-R WP5D regarding NR in these frequencies for IMT [2]. Two set of parameters requested by ITU-R WP5D are the ACLR and ACS of BS and UE in these frequencies. Currently, different sets of ACLR and ACS are specified in RAN4 specifications [3, 4, 5] for NR BS and UE in FR1 and FR2 based on coexistence studies as recorded in TR 38.803 [6]. Therefore, coexistence study will need to be carried out to provide answers to ITU-R WP5D on these parameters with sound technical justifications from coexistence perspective. The WF on the simulation assumptions for the coexistence study was agreed in RAN4#94-e-Bis [7].

This contribution provides the indoor downlink coexistence simulation results according to the agreed assumptions in [7].

# 2.	Discussion

All simulation assumptions for the indoor downlink scenario were agreed in [7], except the AAS BS antenna characteristics which point to the WF agreed in [8], where some parameters were FFS. The following assumptions were used for these FFS parameters to obtain the simulation results in this contribution.

- Passive antenna

  - Horizontal/vertical 3 dB beamwidth: 65 degrees

  - Horizontal/vertical front-to-back ratio: 30dB

  - Antenna gain (minus cable loss): 15dB

  - Mechanical down-tilt angle: 9 degrees

The indoor simulation results of the downlink SINR of the victim UE at 7GHz and 10GHz carrier frequency are provided in Figures 1 and 2 below. Here the downlink ACIR is obtained using 45dB BS ACLR and 33dB UE ACS (currently defined for BS and UE in FR1). It can be seen from the figures that in both cases enough downlink SINR (up to 40dB) can be provided using 24dBm BS conducted output power (as agreed in [7]) even without AAS BS beamforming.

Figure 1: DL SINR of victim UE at 7GHz

Figure 2: DL SINR of victim UE at 10GHz

The indoor simulation results of the downlink throughput loss of the victim UE with different ACIR offsets (compared to the ACIR using 45dB BS ACLR and 33dB UE ACS) at 7GHz and 10GHz carrier frequency are provided in Table 1 below. It can be seen from the table that the downlink throughput loss of the victim UE in the indoor scenario can still be limited to 5% with a downlink ACIR offset of -9dB at 7GHz and 10GHz (i.e. 9dB less stringent ACIR).

| ACIR offset X [dB] | 0 | -2 | -4 | -6 | -8 | -10 |
| --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (7GHz) | 0.92% | 1.39% | 2.06% | 3.00% | 4.27% | 5.94% |
| 5%-tile throughput loss (7GHz) | 0.05% | 0.08% | 0.13% | 0.21% | 0.33% | 0.55% |
| Average throughput loss (10GHz) | 0.91% | 1.38% | 2.04% | 2.97% | 4.23% | 5.90% |
| 5%-tile throughput loss (10GHz) | 0.07% | 0.11% | 0.14% | 0.19% | 0.42% | 0.71% |

Table 1: DL throughput loss of victim UE at 7GHz and 10GHz

# 3.	Conclusion

This contribution has provided the indoor downlink coexistence simulation results according to the agreed assumptions.

The simulation results have shown that the downlink throughput loss of the victim UE in the indoor scenario can still be limited to 5% with a downlink ACIR offset of -9dB at 7GHz and 10GHz (i.e. 9dB less stringent ACIR).

# References

[1]	RP-200513, “New SI proposal: Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz”, Ericsson, Huawei, HiSilicon.

[2]	RP-200042, “LS on parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23”, ITU-R WP5D.

[3]	3GPP TS 38.101-1 v16.3.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[4]	3GPP TS 38.101-2 v16.3.0, “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

[5]	3GPP TS 38.104 v16.3.0, “NR; Base Station (BS) radio transmission and reception”.

[6]	3GPP TR 38.803 v14.2.0, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”.

[7]	R4-2005174, “WF on Simulation Assumptions for the SI on 6.425-7.125GHz and 10.0-10.5GHz”, Nokia, Ericsson, ZTE.

[8]	R4-2005173, “WF on BS Antenna parameters for the SI on 6.425-7.125GHz and 10.0-10.5GHz”, Huawei.