3GPP TSG-RAN4 Meeting #95-e	R4-2006182

# Online, 25th May - 5th Jun 2020

Agenda item:	6.1.5.11

Source:	Apple

Title:		Remaining issues on serving cell evaluation in RRC connected mode for NR-U

Document for:	Discussion

# Introduction

In RAN4 #94ebis meeting, one WF on NR-U RRM was approved in [1], and the open issues on serving cell evaluation are as below,

Issue 2-7-1: UE behaviour in RRC_CONNECTED mode when the serving cell is unavailable for consecutive SSB bursts - Candidate options: - Option 1: In RRC_CONNECTED mode, UE shall initiate measurements on neighbour cells indicated by the serving cell if it is unable to measure on the serving cell for at least X consecutive number of SSB bursts not available at the UE, where the value of X is TBD. - Option 2: Don’t introduce new UE behaviors as the problem can be taken care of by existing mechanisms.

The UE behavior and max number of DL LBT failure is still open by the end of last meeting. In this contribution, we have further analysis on the UE behavior for serving cell evaluation for NR-U.

# Discussion on serving cell measurement for NR-U

Upon exceeding the maximum acceptable number of DL LBT failures over the corresponding period of time, the UE has to restart the corresponding procedure, e.g., time index detection and measurements. In our understanding, this rule is applied for both serving and neighbour cell measurement. However, serving cell measurement is a little more special than neighbour cell, i.e. beside the restarting of the measurement, serving cell also need to consider whether it’s necessary to trigger neighbour cell measurement. In serving cell evaluation in IDLE mode, it was agreed in RAN4 #92 that,

UE shall initiate measurements on neighbour cells indicated by the serving cell if it is unable to measure on the serving cell for at least Mp consecutive number of DRX cycles not available at the UE, where the value of Mp is TBD UE shall initiate the measurements on neighbour cells of any intra-frequency or inter-frequency if it is unable to measure on serving cell during at least consecutive Mq number of DRX cycles not available at the UE, regardless of any condition of SnonIntraSearchP and SnonIntraSearchQ

Similar as IDLE mode, in RRC_CONNECTED mode, it’s also necessary to consider the mobility impact if LBT failure result into multiple times of restarting serving cell measurements. If UE only wait to resume serving cell measurement from LBT failure, that may cause potential problem on the mobility performance and eventually it may result into RLF. So if UE could smartly initialize the neighbor cell measurement in case LBT failure lasted too long time during serving cell measurement period, it would help UE to handover to a proper neighbor cell before RLF is triggered.

In last meeting, companies commented that in Connected mode UE is configured with MOs and will do the measurement all the time, however, it’s not true based on the measurement mechanism design. In Measurement configuration, UE will be configured with s-MeasureConfig.

So UE will perform the non-serving cell measurement when the serving cell signal strength is lower than the s-MeasureConfig, and it is similar as IDLE mode (if UE reach max LBT failure number we require UE to do neighbour cell measurement regardless of the actual serving cell evaluation results or S thresholds). If LBT failure interrupts the serving cell evaluation consistently, we shall also require UE to do neighbour cell measurement regardless of s-MeasureConfig. Compared with IDLE mode, the mobility performance is even more critical to be guaranteed.

Based on the discussion above, for serving cell measurement in RRC_CONNECTED mode, we would like to reuse the similar mechanism of serving cell measurement from IDLE mode, i.e., UE will initiate neighbour cell measurement if it is unable to measure on the serving cell for at least certain consecutive number of SSB bursts and the s-MeasureConfig is configured.

In IDLE mode it was specified that,

UE shall initiate measurements on neighbour cells indicated by the serving cell if it is unable to measure on the serving cell for at least Mp consecutive number of DRX cycles not available at the UE, where Mp=[4] when DRX cycle length <1.28 s, Mp=[2] when DRX cycle length ≥1.28 s.

Since we have the agreement for intra-frequency measurement as shown in the following table, and we think the amount threshold of consistent LBT failure to the serving cell evaluation could be as same as the current Lmeas,max.

| Procedure | Rel-15 samples | Maximum number of DL LBT failures |  |  |
| --- | --- | --- | --- | --- |
|  |  | Parameter name | Parameter value | Condition |
| Measurement, no gaps | 5 | Lmeas,max | 7 | Max(TDRX, TSMTC)≤40 |
|  |  |  | 5 | 40< Max(TDRX, TSMTC)≤320 |
|  |  |  | 3 | TDRX>320 |
| Measurement, with gaps | 5 | Lmeas,gaps,max | 7 | Max(TDRX, TSMTC, MGRP)≤40 |
|  |  |  | 5 | 40< Max(TDRX, TSMTC, MGRP)≤320 |
|  |  |  | 3 | TDRX>320 |

So we propose that: in RRC_CONNECTED mode,

    - when the s-MeasureConfig is configured and serving cell measurement needs MG, UE shall initiate measurements on neighbour cells indicated by the serving cell if it is unable to measure on the serving cell for at least Mp_connected consecutive number of SSB bursts not available at the UE, where Mp_connected = 7 when Max(TDRX, TSMTC)≤ 40ms, Mp_connected = 5 when 40ms< Max(TDRX, TSMTC)≤320ms, Mp_connected = 3 when DRX cycle length >320ms.

    - when the s-MeasureConfig is configured and serving cell measurement does not needs MG, UE shall initiate measurements on neighbour cells indicated by the serving cell if it is unable to measure on the serving cell for at least Mp_connected_gaps consecutive number of SSB bursts not available at the UE, where Mp_connected_gaps = 7 when Max(TDRX, TSMTC, MGRP)≤ 40ms, Mp_connected_gaps = 5 when 40ms< Max(TDRX, TSMTC, MGRP)≤320ms, Mp_connected_gaps = 3 when DRX cycle length >320ms.

Proposal: in RRC_CONNECTED mode,

    - when the s-MeasureConfig is configured and serving cell measurement needs MG, UE shall initiate measurements on neighbour cells indicated by the serving cell if it is unable to measure on the serving cell for at least Mp_connected consecutive number of SSB bursts not available at the UE, where,

      - Mp_connected = 7 when Max(TDRX, TSMTC)≤ 40ms,

      - Mp_connected = 5 when 40ms< Max(TDRX, TSMTC)≤320ms,

      - Mp_connected = 3 when TDRX >320ms.

    - when the s-MeasureConfig is configured and serving cell measurement does not needs MG, UE shall initiate measurements on neighbour cells indicated by the serving cell if it is unable to measure on the serving cell for at least Mp_connected_gaps consecutive number of SSB bursts not available at the UE, where,

      - Mp_connected_gaps = 7 when Max(TDRX, TSMTC, MGRP)≤ 40ms,

      - Mp_connected_gaps = 5 when 40ms< Max(TDRX, TSMTC, MGRP)≤320ms,

      - Mp_connected_gaps = 3 when TDRX >320ms.

# Conclusion

The UE behavior and max number of DL LBT failure is still open by the end of last meeting. In this contribution, we have further analysis on the UE behavior for serving cell evaluation for NR-U.

Proposal: in RRC_CONNECTED mode,

    - when the s-MeasureConfig is configured and serving cell measurement needs MG, UE shall initiate measurements on neighbour cells indicated by the serving cell if it is unable to measure on the serving cell for at least Mp_connected consecutive number of SSB bursts not available at the UE, where,

      - Mp_connected = 7 when Max(TDRX, TSMTC)≤ 40ms,

      - Mp_connected = 5 when 40ms< Max(TDRX, TSMTC)≤320ms,

      - Mp_connected = 3 when TDRX >320ms.

    - when the s-MeasureConfig is configured and serving cell measurement does not needs MG, UE shall initiate measurements on neighbour cells indicated by the serving cell if it is unable to measure on the serving cell for at least Mp_connected_gaps consecutive number of SSB bursts not available at the UE, where,

      - Mp_connected_gaps = 7 when Max(TDRX, TSMTC, MGRP)≤ 40ms,

      - Mp_connected_gaps = 5 when 40ms< Max(TDRX, TSMTC, MGRP)≤320ms,

      - Mp_connected_gaps = 3 when TDRX >320ms.

# References

[1] R4-2005375, Agreements and topics discussed in email thread:[94e Bis][106] NR_unlic_RRM_Core_Part_3, Nokia, Nokia Shanghai Bell, RAN4 #94ebis meeting