3GPP TSG-RAN WG4 Meeting #95-e                                                 R4-2006317

Electrical Meeting, 25th May-5th June, 2020

Source:	Samsung

Title:	Simulation results for PMI test cases with Rel-16 Type II codebook

Agenda item:			6.11.3.3

Document for:	Discussion

- Introduction

In RAN4 94bis-e meeting, a WF for Rel-16 enhanced type II was approved in [1], with the following agreements achieved:

- Test setup:

  - Option 1: Use SU-MIMO test setup as baseline scenario

  - Option 2: MU-MIMO test setup

    - Option 2a: if needed, discuss under TEI-15

- Number of CSI-RS ports:

  - Option 1: 16 ports with (N1,N2) = (4,2) and (O1,O2)=(4,4)

  - Option 2 : 32 ports with (N1,N2) = (4,4) and (O1,O2)=(4,4)

- Number of PMI Sub-bands per CQI Sub-band

  - Option 1: R = 1

  - Option 2: R = 2

- Codebook parameter configuration

  - Option 1: paramCombination-r16: 6, with L =4, pν =1/2, β=1/2 as baseline

  - Other options not precluded

- Define requirements only for sub-band PMI reporting.

- Sub-band Size:

  - Option 1:

    - 4 for FDD with 15kHz SCS, 10MHz CBW

    - 8 for TDD with 30kHz SCS, 40MHz CBW

  - Option 2: FFS

- Take beam steering approach as specified in B.2.3B.4A of TS 36.101 and

  - Option 1: Extend it to L > 2 beams

  - Option 2: Use it as it is with L = 2 beams.

- Channel Model

  - TDLA30-5 as baseline

  - Other options not precluded

- MIMO Correlation

  - Option 1: XP High

  - Option 2: XP Medium

  - Down-select to one option based on simulation results in the next meeting

- MCS and Rank

  - MCS 20 (64QAM Table), Rank 2 as baseline

  - Other options not precluded

- Test Metric for SU-MIMO

  - Relative Throughput ratio between following PMI and random PMI

- Test Metric for MU-MIMO

  - Option 1: Relative Throughput ratio between following PMI for Rel-16 enhanced Type II and Rel-15 Type II codebook for MU-MIMO based test setup

  - Other options not precluded

- For initial simulations:

  - Use the parameters listed in previous slides.

  - The remaining parameters will be same as for Rel-15 Type II codebook simulation assumptions in R4-2005550

In last meeting, we showed enhanced Type II simulation results focus on wideband reporting case, in this contribution, we will present our simulation results on enhanced Type II with subband reporting case and provide our opinions on them.

- Simulation results

In this section, we simulate all the cases on condition that the channel varies in multi-beam steering model including Type I CSI.

- 16x2 rank2

  - XP High

Figure 1: Throughput performance for 16x2 rank2 under XP High correlation channel

  - XP Medium

Figure 2: Throughput performance for 16x2 rank2 under XP Medium correlation channel

- 32x2 rank2

Figure 3: Throughput performance for 32x2 rank2

In table 1, we summarize the SNR point and relative throughput ratio between the following PMI and random PMI under 16Tx/32Tx cases.

Table 1: The TP ratio and SNR point for Enhanced Type II with “paramCombination-r16=6”

| Test case | SNR in dB at 90% of peak throughput | TP ratio |
| --- | --- | --- |
| 16x2, XP High | 13.6 | 3.98 |
| 16x2, XP Medium | 12.6 | 4.09 |
| 32x2, XP High | 12.8 | 5.86 |
| 32x2, XP Medium | 12.2 | 7.69 |

As described above, we observe as following:

Observation 1: The TP ratio and SNR point for are summarized in Table 1, the following PMI performance of enhanced type II CSI with “paraCombination-r16” achieves significant gain than random PMI. The TP ratios in XP Medium are higher than in XP High cases. The 32Tx case  have similar SNR point and higher TP ratio versus 16 Tx case.

Observation 2: The performance of enhanced type II CSI with “paraCombination-r16=6” has dominant advantage than “paraCombination-r16=2” due to the feedback overhead increase.

Observation 3: In XP High condition, the type II CSI and enhanced type II CSI with “paraCombination-r16=2” have marginal gain versus type I CSI especially in high SNR region. Besides, in XP Medium condition, the various types of codebooks appear obvious differentiation than in XP High condition, the type II CSI achieves 2.6dB at 90% SNR point versus type I CSI, meanwhile, the enhanced type II CSI with “paraCombination-r16=2” achieves 0.8dB gain at 90% SNR point versus type I CSI, the enhanced type II CSI with “paraCombination-r16=6” achieves 5.7dB gain at 90% SNR point versus type I CSI.

- Conclusion

In this contribution, we provide our simulation results for enhanced Type II codebook. The relative ratio at the 90% maximum throughput and SNR point are collected in table 1.

In a word, we draw the conclusions as the following:

Observation 1: The enhanced type II CSI with”paramCombination-r16=6” could bring significant gain and it is recommended as simulation parameter to achieve the type II CSI’s potential advantage.

Observation2: The performance gain of enhanced type II is more obviously in XP Medium correlation channel model than in XP high correlation case. In brief, XP medium model is proposed for enhanced type II CSI reporting test.

Observation 3: Based on our simulation results, 16Tx case is enough to define test set-up.

4. Reference

[1] R4-2005530, “Way forward on PMI reporting requirements for NR MIMO”, Qualcomm, RAN4 94bis-e

[2] R4-2002979, “Test case design for Rel-16 enhanced type II codebook”, Samsung, RAN4 94bis-e

[3] R4-2002978, “Initial simulation results for PMI requirement with Rel-16 enhanced type II codebook”, Samsung,

RAN4 94bis-e

5. Annex

- Table 2: Simulation assumptions for Rel-16 enhanced Type II PMI (FDD)

| Parameter |  | Unit | Test FDD 2Rx |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | 16 ports with (N1,N2) = (4,2) and(O1,O2)=(4,4) 32 ports with (N1,N2) = (4,4) and(O1,O2)=(4,4) |
| Beamforming Model |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 5, (4,-) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (9,-) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | ZP CSI-RS trigger |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 16/32 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 12, (2, 4, 6, 8) for 16 ports Row 16, (2, 4, 6, 8) for 32 ports |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | (5,-) |
|  | CSI-RS interval and offset | slot | 5/1 |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | FFS |
| pmi-FormatIndicator |  |  | Subband |
| Sub-band Size |  | RB | 4 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | 5/1 |
| Aperiodic Report Slot Offset |  |  | 5 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeII-r16 |
|  | paramCombination-r16 |  | 6 |
|  | numberOfPMISubbandsPerCQISubband |  | R = 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,2) for 16 ports (4,4) for 32 ports |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | n1-n2-codebookSubsetRestriction-r16 |  | FFS |
|  | typeII-RI-Restriction-r16 |  | FFS |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 8 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel |  |  | MCS 20 FDD |
| Note 1: For random precoder selection, the precoder shall be updated in each slot (1 ms granularity). Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Taking beam steering approach as specified in B.2.3B.4A of TS 36.101 as staring point with further extension applicable for L beams |  |  |  |