3GPP TSG-RAN WG4 Meeting #95-e                                    R4-2006370

Electronic Meeting, 25th May – 5th June, 2020

Agenda item:			6.11.2.1

Source:			Samsung

Title:	Discussion on L1-SINR Measurement Requirement

Document for:		Approval

# 1	Introduction

The Rel-16 MIMO enhancement work item was approved in RAN#80 with a list of enhancement areas including L1-SINR measurement and reporting. Back to very early discussion of Rel-16 eMIMO, RAN1 has concluded that L1-SINR is confirmed to be supported while L1-RSRQ is excluded to be measured and reported.

During last RAN4 meeting (RAN4#94-Bis-e electronic meeting), the following agreement is achieved for L1-SINR measurement and reporting related RRM requirement, captured in R4-2005335 [8]:

- L1-SINR measurement: - Measurement period for L1-SINR measurement: - For SSB/CSI-RS-based CMR+IMR, the value of P, and whether or not a single P value for both CMR and IMR - Option-1: Define a single P for both CMR and IMR. A L1-SINR measurement occasion is considered as available only when the CMR and the associated IMR are non-overlapped with measurement gap or SMTC window. - Option-2: No requirement when CMR or IMR is fully overlapped with MG. The variable P used for defining L1-SINR measurement period could can be defined as the maximum value between PCMR and PIMR, where - PCMR is the scaling factor for CMR according to the principles of defining variable P for L1-RSRP measurement. - PIMR is the scaling factor for IMR according to the principles of defining variable P for L1-RSRP measurement. - Extend single carrier requirement to CA: - FFS how to extend single carrier requirements to CA case e.g., under the assumption the offset of SMTC occasion cross multiple CCs are the same. - The same approach concluded from Rel-15 TEI discussion to L1-RSRP measurement requirement can be reused. - “Repetition = On” for NZP-IMR: - For L1-SINR measurement with dedicated configured NZP-IMR, the expected UE behaviour and possibility of “repetition=on”: - (a) For L1-SINR measurement, if NZP-IMR is not configured with repetition parameter: - FFS whether or not L1-SINR measurement requirement is applicable. - (b) For L1-SINR measurement, no requirement for the following cases: - SSB based CMR and NZP-IMR configured with “repetition = ON” - NZP CSI-RS based CMR configured with “repetition = OFF” and NZP-IMR configured with “repetition = ON” - NZP CSI-RS based CMR configured with “repetition = ON” and NZP-IMR configured with “repetition = OFF” - For SSB-based CMR+IMR, scaling factor N under different conditions of “repetition” field of IMR: - For SSB-based CMR+IMR L1-SINR measurement, the scaling factor of N=8. - For IMR (NZP CSI-RS) with “repetition = ON”, FFS no requirement shall be applied; - For IMR (NZP CSI-RS) with “repetition” not present, FFS no requirement shall be applied; - For CSI-RS-based CMR+IMR, scaling factor N under different conditions of “repetition” field of IMR: - CSI-RS-based CMR+IMR L1-SINR measurement, - The scaling factor of N is defined as the same as CSI-RS based L1-RSRP measurement, i.e., only considering CMR: - N=1, if CSI-RS resource in a resource set configured with repetition=off is configured as CMR - N= ceil(maxNumberRxBeam / Nres_per_set), if CSI-RS resource in a resource set configured with repetition=on is configured as CMR. - The requirement is applied when CSI-RS based CMR and NZP-IMR are configured with the same repetition pattern. - The requirement is applied only when the number of CSI-RS resource in the resource set is same for both CSI-RS based CMR and NZP-IMR - The requirement is applied for CMR configured with “OFF” only if the QCL of CMR is provided. - For L1-SINR measurement with dedicated configured ZP-IMR: - “Repetition” field is not present for CSI-IM configuration - Measurement Restriction and Scheduling Availability - Measurement Restriction - Measurement restriction requirement is defined under three kinds of signals for measurement, i.e., SSB, CSI-RS and CSI-IM. See draft CR R4-2003539 for details. - For CMR+IMR scenario, longer L1-SINR measurement period is expected when either CMR or IMR is with measurement restriction. - Scheduling restriction - The scheduling restriction due to L1-SINR shall be further studied in RAN4. - Side condition and Others - L1-SINR measurement side condition for Es/Iot for CMR+NZP-IMR - Es/Iot on CMR and NZP-IMR: - (1) Es/Iot >=0dB and <=25dB, and the resultant L1-SINR should be greater than -3 dB. - (2) CMR Es/Iot = NZP-IMR Es/Iot, and CMR Es/Iot and NZP-IMR Es/Iot will be both specified.

Hence, in this discussion paper, we would like to further provide our analysis and view for outstanding issues for L1-SINR measurement to complete Rel-16 RRM core requirement.

# Discussion

#### 2.1 Measurement Period for L1-SINR

In last RAN4 meeting, for SSB/CSI-RS-based CMR+IMR, the value of P, and whether or not a single P value for both CMR and IMR were discussed, where two options remains for further discussion:

- For SSB/CSI-RS-based CMR+IMR, the value of P, and whether or not a single P value for both CMR and IMR - Option-1: Define a single P for both CMR and IMR. A L1-SINR measurement occasion is considered as available only when the CMR and the associated IMR are non-overlapped with measurement gap or SMTC window. - Option-2: No requirement when CMR or IMR is fully overlapped with MG. The variable P used for defining L1-SINR measurement period could can be defined as the maximum value between PCMR and PIMR, where - PCMR is the scaling factor for CMR according to the principles of defining variable P for L1-RSRP measurement. - PIMR is the scaling factor for IMR according to the principles of defining variable P for L1-RSRP measurement.

For the value of P, although the option 1 for a single P can provide a more simple description for sharing factor (especially considering one-to-one mapping between CMR and IMR as agreed by RAN1), the option 2 indeed enjoy the benefit of less restriction on the L1-SINR measurement requirement’s condition. Specifically, option 2 don’t need both CMR and IMR are non-overlapped with measurement gap or SMTC window, which could impose additional scheduling/configuration restriction to gNB. Considering that, we can compromise to option 2 as the method to define sharing factor P as the maximum value between PCMR and PIMR.

Proposal-1: For SSB/CSI-RS-based CMR+IMR L1-SINR measurement, 
  - No requirement when CMR or IMR is fully overlapped with MG;
  - The variable P used for defining L1-SINR measurement period could can be defined as the maximum value between PCMR and PIMR, in which PCMR and PIMR are respectively the scaling factor for CMR and IMR according to the principles of defining variable P for L1-RSRP measurement

#### 2.2 “Repetition = On” for NZP-IMR

As guided in WF[8], for L1-SINR measurement, if NZP-IMR is not configured with repetition parameter, there is still not decided yet whether or not L1-SINR measurement requirement is applicable:

- (a) For L1-SINR measurement, if NZP-IMR is not configured with repetition parameter: - FFS whether or not L1-SINR measurement requirement is applicable.

From our understanding, NZP-IMR should not play the role to decide UE RX beamforming behaviour since NZP-IMR’s RX beamforming filter should always be aligned with the RX beamforming filter used for CMR which is one-to-one mapped with this NZP-IMR. From that perspective, even if NZP-IMR is not configured with repetition parameter, this factor will not impact the factor that L1-SINR measurement requirement should still be applicable.

Observation-1: NZP-IMR’s RX beamforming filter should always be aligned with the RX beamforming filter used for CMR which is one-to-one mapped with this NZP-IMR.

Proposal-2: Whether or not NZP-IMR is not configured with repetition parameter should not impact on L1-SINR measurement requirement’s applicability.

Another remaining issue is whether or not the requirement should apply in the following two cases:

- For SSB-based CMR+IMR, scaling factor N under different conditions of “repetition” field of IMR: - For SSB-based CMR+IMR L1-SINR measurement, the scaling factor of N=8. - For IMR (NZP CSI-RS) with “repetition = ON”, FFS no requirement shall be applied; - For IMR (NZP CSI-RS) with “repetition” not present, FFS no requirement shall be applied;

As mentioned in Observation-1, NZP-IMR’s RX beamforming filter should always be aligned with the RX beamforming filter used for CMR which is one-to-one mapped with this NZP-IMR, and the same principle should be followed as CSI-RS based CMR+IMR, i.e., the requirement is applied when CSI-RS based CMR and NZP-IMR are configured with the same repetition pattern. From that perspective, if “repetition=ON” for IMR in a SSB-based CMR+IMR, UE’s expected behaviour could be not clear enough. Therefore, the following proposal 2 is provided:

Proposal-2: For SSB-based CMR+IMR L1-SINR measurement, the measurement period requirement shall be applied only if IMR (NZP CSI-RS) configured with “repetition = OFF” or repetition filed not present, but no requirement if IMR (NZP CSI-RS) configured with “repetition = ON”.

# Conclusion

In this paper, we provided our views on L1-SINR measurement and reporting, with the following observations and proposals achieved:

Proposal-1: For SSB/CSI-RS-based CMR+IMR L1-SINR measurement, 
  - No requirement when CMR or IMR is fully overlapped with MG;
  - The variable P used for defining L1-SINR measurement period could can be defined as the maximum value between PCMR and PIMR, in which PCMR and PIMR are respectively the scaling factor for CMR and IMR according to the principles of defining variable P for L1-RSRP measurement

Observation-1: NZP-IMR’s RX beamforming filter should always be aligned with the RX beamforming filter used for CMR which is one-to-one mapped with this NZP-IMR.

Proposal-2: Whether or not NZP-IMR is not configured with repetition parameter should not impact on L1-SINR measurement requirement’s applicability.

Proposal-2: For SSB-based CMR+IMR L1-SINR measurement, the measurement period requirement shall be applied only if IMR (NZP CSI-RS) configured with “repetition = OFF” or repetition filed not present, but no requirement if IMR (NZP CSI-RS) configured with “repetition = ON”.

# Reference

[1] RP-192271,	“Revised WID: Enhancements on MIMO for NR”, Samsung.

[2] RP-192270,	“Status Report for WI: Enhancements on MIMO for NR”, Samsung

[3] R4-1912742, “WF on RRM requirement scope for Rel-16 NR eMIMO WI”, Samsung, Huawei.

[4] R4-1911390, “Discussion on L1-SINR measurement and reporting for Rel-16 MIMO enhancement”, Samsung.

[5] R4-1913323, “Further Discussion on L1-SINR Measurement and Reporting”, Samsung.

[6] R4-1915850, “Way forward on NR eMIMO RRM”, Samsung.

[7] R4-2002182, “WF on NR eMIMO RRM requirements”, Samsung.

[8] R4-2005335, “WF on NR eMIMO RRM requirements”, Samsung.