3GPP TSG-RAN WG4 Meeting #95	R4-2006445

# 25th to 29th May 2020

Agenda Item:	12.2

Source: 	MediaTek Inc.

Title: 	NBIOT standalone operation for FCC regulation considerations

Document for:	Approval

- 1. Introduction

For NB1 devices in standalone operation, there is band edge requirement per FCC specification#90.691 and [3][4][5] which has been discussed in RAN4#94bis-e meeting. The contribution provide some discussion on which release to introduce the spec change.

- 2. Discussion

In last RAN4 meeting, it was discussed some UE failure was found under the updated testing conditions against FCC regulation by test labs[6]. A WF[7] was agreed to encourage more analysis data from companies as well as options on which release the new network signalling shall be applied. We further look into FCC regulations. Though some of them[3][5] says “A narrower resolution bandwidth is permitted” but there was no clear guidance how narrow the RBW can be. In general, most test labs adopt 1% EBW (emission bandwidth) that was explicitly described in the regulation. Thus we think for band 2, 5, 25 and 26, these bands shall be included in the new network signalling to narrow their operation frequency range a little bit. Based on analysis report in last meeting [6], we continue propose a spec change in proposal 1. And thanks to offline discussion with companies, the idea is to modify existing NS_04 definition that there’s limited impact on non-backward compatible (NBC) issue for existed UE.

Proposal 1: We propose to modify NS_04 for category NB1 and NB2 device for operating bands 2, 3, 4, 5, 12, 17, 25, 26, 66, 71, 85. The lower limit and upper limit of operating bands are 100KHz narrower from both lower and upper band edge defined in Table 5.5-1 to account for the FCC regulations.

Table 6.2.4F-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |
| --- | --- | --- | --- |
| NS_01 | 6.6.2F.1 | Operating bands defined in 5.5F | N/A |
| NS_02 | 6.6.2F.2.1 | [1, 2, 3, 5, 8, 11, 12, 13, 17, 18, 19, 20, 21, 25, 26, 28, 66, 70 and 85] | [N/A] |
| NS_03 | 6.6.2F.2.2 | [1, 2, 3, 5, 8, 11, 12, 13, 17, 18, 19, 20, 21, 25, 26, 28, 66, 70 and 85] | [N/A] |
| NS_04 | 5.5F | 2, 3, 4, 5, 12, 17, 25, 26, 66, 71, 85 | N/A |

As for which spec release shall the change be applied, here’s our justification. Category NB1 was introduced since Rel-13. Network signalling for category NB1 device was introduced since Rel-14. For those closed spec release, to change network signalling could raise NBC concern and impact on product change. However, without the change, it is very difficult for UE to pass updated test criteria. And from implementation perspective, the spec change is to narrow operating frequency range 100KHz from both side of band edge. It is up to eNB to configure UE uplink configurations. Thus even for some UE does not recognize the new network signalling, UE can still be configured its uplink according to eNB allocating for UL configurations to pass the FCC regulation. So we would propose to introduce spec change start from Rel-14.

Proposal 2: We propose to introduce the new network signalling from Rel-14.

- 3. Conclusion

- From the discussion above, we have following proposals:

Proposal 1: We propose to modify NS_04 for category NB1 and NB2 device for operating bands 2, 3, 4, 5, 12, 17, 25, 26, 66, 71, 85. The lower limit and upper limit of operating bands are 100KHz narrower from both lower and upper band edge defined in Table 5.5-1 to account for the FCC regulations.

Table 6.2.4F-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |
| --- | --- | --- | --- |
| NS_01 | 6.6.2F.1 | Operating bands defined in 5.5F | N/A |
| NS_02 | 6.6.2F.2.1 | [1, 2, 3, 5, 8, 11, 12, 13, 17, 18, 19, 20, 21, 25, 26, 28, 66, 70 and 85] | [N/A] |
| NS_03 | 6.6.2F.2.2 | [1, 2, 3, 5, 8, 11, 12, 13, 17, 18, 19, 20, 21, 25, 26, 28, 66, 70 and 85] | [N/A] |
| NS_04 | 5.5F | 2, 3, 4, 5, 12, 17, 25, 26, 66, 71, 85 | N/A |

Proposal 2: We propose to introduce the new network signalling from Rel-14.

Reference:

- R4-1815344 NBIOT B26 guard band operation, RAN4#89

- R4-1905011 CR to exclude 100kHz for NBIOT for B26 band edge operation due to PLMR emissions requirement., RAN4#90-bis

- FCC #24.238

- FCC #27.53

- FCC #22.917

- R4-2003987 NBIOT guard band operation for FCC regulation considerations, RAN4#94bis-e

- R4-2005219 WF on NB-IoT band edge performance against FCC regulation, RAN4#94bis-e