3GPP TSG-RAN WG4 Meeting #95-e                                                               R4-2006470

Electronic Meeting, 25 May – 5 Jun., 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0666 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on L1 SL-RSRP measurements (section 12.5) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-05-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The re-evaluation procedure was agreed in RAN1 spec. which is missing in RAN4 L1 SL-RSRP measurements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the general description on re-evaluation procedure. - Re-order the description of pre-emption procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Section 12.5 L1 SL-RSRP measurements is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

12.5 L1 SL-RSRP measurements

12.5.1 Introduction

This section contains the measurement requirements related to resource reselection, resource re-evaluation and resource pre-emption of the UE capable of V2X sidelink communication.

12.5.2 SL-RSRP measurements

The UE physical layer shall be capable of performing the L1 SL-RSRP measurements on the carrier operating V2X sidelink communication for determining the subset of resources to be excluded in PSSCH resource selection in sidelink transmission mode 2. After resource (re-)selection procedure, the UE shall re-evaluate the reserved resources by L1 SL-RSRP measurements before transmission of SCI with reservation.

[When the pre-emption mechanism is enabled for the resource pool that UE is monitoring and selecting resource from,] after UE selects from the resource not excluded based on L1 SL-RSRP measurements, the UE shall be capable of triggering reselection of already signalled resource(s) as a resource reservation when the conditions specified in [16] are satisfied.

The L1 SL-RSRP measurement period corresponds to [TBD] and the measurement shall meet the L1 SL-RSRP measurement accuracy requirement in Section [TBD].

[When the pre-emption mechanism is enabled for the resource pool that UE is monitoring and selecting resource from,] after UE selects from the resource not excluded based on L1 SL-RSRP measurement procedure, the UE shall be capable of triggering reselection of already signalled resource(s) as a resource reservation when the conditions specified in [16] are satisfied.

End of Change