3GPP TSG-RAN WG4 Meeting #95-e	R4-2006517

# Electronic Meeting, 25 May – 05 June, 2020

Title:	LS on RRM relaxation in UE power saving

Response to:	-

Release:	Release 16

Work Item:	NR_UE_pow_sav

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact Person:

Name: Xusheng Wei, vivo

E-mail Address:	xusheng.wei@vivo.com

1	Overall description

RAN4 has further discussion on the behaviour of higher priority frequency layer relaxation, and had the following conclusion:

- When Srxlev > SnonIntraSearchP  and Squal > SnonIntraSearchQ:

- No relaxation measurements on higher priority carriers further than Thigher_priority_search is expected, when the criteria of low mobility is not configured or not fulfilled.

  - RAN4’s assumption is that criteria of not in cell edge must be fulfilled in this scenario;

- UE can stop equal/low priority inter-freq/inter-RAT measurements and the UE measures higher priority inter-frequency layers at least every Thigher_priority_search, when criteria of low mobility and not in cell edge are both fulfilled.

- When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ:

- The relaxed requirement for the frequency layer of higher priority uses the same relaxed measurement requirement as those for the frequency layer of equal/lower priority.

- For scenario 3, i.e., when both relaxation criteria are fulfilled simultaneously, the UE measures higher priority inter-frequency layers at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7

RAN4 has also discussed inter-frequency measurements for UE power saving. Currently two sets of parameters SIntraSearchP /SIntraSearchQ and SnonIntraSearchP /SnonIntraSearchQ are introduced to handle IDLE state intra-frequency measurement and inter-frequency measurement, RAN4 identifies that using an independent threshold particularly for inter-frequency measurement could bring more potential on UE power saving.

RAN4 kindly asks RAN2 to take aforementioned RAN4’s conclusions into account.

2	Actions

To RAN2 group.

ACTION: 	RAN4 kindly asks RAN2 to take aforementioned RAN4’s conclusions into account.

3	Dates of next TSG RAN4 meetings

RAN4 #96-e			24.08 – 28.08, 2020, E-meeting

RAN4 #96bis			12.10 – 16.10, Kochi, India