3GPP TSG-RAN WG4 Meeting #95-e		R4-2006529

E-meeting, 25 May – 5 June, 2020

Agenda item:	6.12.4

Source:	Intel Corporation

Title:	Discussion on UE performance requirements for FR2 256QAM

Document for:	Discussion

# Introduction

In the previous meeting WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM was agreed [1].

In this paper we provide our view on UE performance requirements for FR2 256QAM.

# Discussion

## Demodulation requirements

In the previous RAN4 meeting, the following agreements were reached on demodulation requirements:

- Define FR2 PDSCH demodulation requirements for 256QAM - Tx EVM - Option 1: 3% - Option 2: 3.5% - Option 3: 2% - Rx EVM - Option 1: 2% - Option 2: Not Specified - Option 3: Consider agreements from WF R4-1811394 as starting point and check if it is applicable to 256QAM discussion - MIMO configuration - 2Tx 2Rx ULA low - Other options are not precluded - Rank - Option 1: rank 1 - Option 2: rank 1 and 2 - Other options are not precluded - Propagation condition - Option 1: Fading channel - Option 1a: TDLA30-300 - Option 1b: TDLD30-75 - Option 1c: TDL-D for Rank 1, TDL-D for Rank 2 - Option 1d: TDL-A for Rank 1, TDL-D for Rank 2 - Option 1e: TDLD30-35 - Option 2: Static channel - TBD based on simulation results - Channel bandwidth and PRB allocation - Option 1: 100MHz CBW with full PRB allocation - Option 2: 100MHz CBW with partial PRB allocation - Option 3: 50MHz CBW with full PRB allocation - MCS - Option 1: MCS 20 or higher depending on the allocated PRB number - Option 2: MCS 21 - Option 3: MCS 20 - Option 4: MCS 25/26/27 - Other options are not precluded - Carrier frequency - Option 1: band agnostic - Option 2: Further analyze whether it is possible to define band agnostic requirements - DM-RS configuration - Option 1: Type 1 single symbol front loaded, 1 additional DMRS - Option 2: Type 1 single symbol front loaded, 0 additional DMR

DM-RS configuration

Based on TR 38.883 [2], all feasibility study was done under assumptions of 1 additional DMRS. Using of 1 additional DMRS allows to improve channel estimation and noise covariance matrix estimation accuracy. It is rather important for high order modulations which is very sensitive to inaccurate estimation of propagation conditions. Therefore, we propose to use 1 additional DMRS for 256QAM demodulation requirements definition.

Channel bandwidth and PRB allocation

The main limiting factor of FR2 requirements is testable SNR. From TR 38.810 [3], we can observe that testable SNR decreases in case of channel bandwidth increases. Therefore, we suggest to consider the smallest possible FR2 CBW for SCS 120 kHz (i.e. 50 MHz) to have ability for testing of more scenarios with 256QAM.

After previous meeting, we have options with full and partial PRB allocation. Based on our understanding, using of partial allocation does not allow to increase testable SNR, because unallocated resources will be occupied by OCNG signals and effectually all channel bandwidth will contain signals for transmission. Same time, full PRB allocation is baseline scenario for PDSCH requirements. Therefore, we suggest to define FR2 256QAM demodulation requirements for full PRB allocation.

Antenna configuration

Based on our understanding, antenna configuration depends on propagation conditions. If requirements are defined for fading conditions then typical antenna configuration is 2x2. Same time, if requirements are defined for static conditions then typical antenna configuration is 1x2 for scenarios with Rank 1 transmission and 2x2 for scenarios with Rank 2 transmission. 1x2 is usually used for static conditions because it allows to avoid any potential issue with certain precoders which lead to zeroing of signal on one of receive antenna. Therefore, we suggest to reuse all these assumption for FR2 256QAM requirements.

RF Impairments

RF impairments modelling for requirements definition was discussed in the scope of Rel-15 NR WI. Based on WFs [4] and [5], the following agreements were reached

WG4 Meeting #88 [4] - Use the following RF impairments models to define the minimum UE performance requirements for FR1 - TX EVM = 3% for 256QAM - Use the following RF impairments models to define the minimum UE performance requirements for FR2 - TX EVM is modelled as AWGN - No Tx phase noise is modelled - Phase noise is explicitly modelled for Rx. Use phase noise model #2 as defined in TR 38.803 for FR2 demodulation performance tests. - Option 2-1: Rx Phase noise is modelled for requirements definition - Option 2-2: Rx Phase noise is modelled only to find feasible FRC configuration (i.e. achieve maximum throughput and loss in comparison to scenarios without Rx phase noise is less then 1 dB) WG4 Meeting #89 [5] - Issue 5: SNR requirements derivation - Plan for deriving performance requirements (SNR points) - … - … IM results submitted from companies should take PN impact into account.

As for transmit impairments modelling, taking into account that EVM requirements are same for FR1 and FR2, we suggest to reuse FR1 agreement on Tx EVM assumptions (i.e. 3%).

As for receive impairments modelling, we suggest to reuse agreements from Rel-15 discussion and check impact of explicit Rx PN modelling on 256QAM performance to understand whether and under which assumptions requirements can be defined in band agnostic manner. As for phase model we suggest to use model #1, because, based on discussion from feasibility study, model #2 is rather pessimistic for UE side and may not reflect the practical conditions. Also, based on our analysis from [6], we can observe that residual EVM level after common phase error (CPE) compensation for model #2 (UE) is rather high for 256QAM operation.

Band agnostic requirements

In Table 1 PDSCH performance comparison for scenarios without and with explicit phase noise modelling is provided to understand which scenarios can be considered for definition of band agnostic requirements.

Table 1. Impact of PN on PDSCH performance

| Rank configuration | Channel model | Phase noise model | SNR for 70% of max T-put |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MCS 20 | MCS 21 | MCS 22 | MCS 23 |
| Rank 1 | Static | No | 16.1 | 16.6 | 18.0 | 19.3 |
|  |  | Model #1 (CF 29GHz) | 16.4 | 17.1 | 18.4 | 19.6 |
|  |  | Model #1 (CF 39GHz) | 16.5 | 17.3 | 18.6 | 20.2 |
|  | TDL-D | No | 17.9 | 18.6 | 19.8 | 21.0 |
|  |  | Model #1 (CF 29GHz) | 18.3 | 19.1 | 20.6 | 22.0 |
|  |  | Model #1 (CF 39GHz) | 18.6 | 19.5 | 21.2 | 22.8 |
|  | TDL-A | No | 18.9 | 19.8 | 21.2 | 22.6 |
|  |  | Model #1 (CF 29GHz) | 19.3 | 20.2 | 21.9 | 23.5 |
|  |  | Model #1 (CF 39GHz) | 19.7 | 20.7 | 22.5 | 24.4 |
| Rank 2 | Static | No | 19.1 | 19.6 | 20.9 | 22.3 |
|  |  | Model #1 (CF 29GHz) | 19.5 | 20.4 | 21.7 | 23.2 |
|  |  | Model #1 (CF 39GHz) | 20.1 | 21.0 | 22.6 | 24.6 |
|  | TDL-D | No | 23.9 | 24.8 | 26.4 | 27.8 |
|  |  | Model #1 (CF 29GHz) | 25.7 | 27.2 | 30.4 | N/A |
|  |  | Model #1 (CF 39GHz) | 28.1 | 31.3 | N/A | N/A |
|  | TDL-A | No | 29.5 | 30.8 | 33.3 | 36.4 |
|  |  | Model #1 (CF 29GHz) | 36.5 | N/A | N/A | N/A |
|  |  | Model #1 (CF 39GHz) | N/A | N/A | N/A | N/A |

Observations:

- It is rather hard to define scenario with Rank 2 transmission which allows to have band agnostic requirements assuming methodology agreed for Rel-15 WI.

- Band agnostic requirements can be considered for the following scenarios with Rank 1 transmission:

  - Static channel mode: MCS 20-23

  - TDL-D channel mode: MCS 20-21

  - TDL-A channel mode: MCS 20-21

MCS/Rank

Talking into account that FR2 requirements are limited by testable SNR, MCS and Rank should be defined to ensure testability under different test equipment conditions. In Table 2 we provide the maximum SNR values from TR 38.810 [3] for DFF and IFF methods.

Table 2. Max testable SNR in FR2.

| Channel bandwidth | Test method | Max SNR |
| --- | --- | --- |
| 100 MHz | DFF | 19.4dB |
|  | IFF | 19.7dB |
| 200 MHz | DFF | 16.4dB |
|  | IFF | 16.7dB |

From Table 1 we can observe that SNR depends on test method. For analysis we suggest to assume test method with the worst SNR condition (i.e. DFF), because test method is up to TE decision and one of these methods cannot be selected as baseline. For DFF method and CBW 100 MHz, maximum testable SNR is 19.4 dB. Same time, if FR2 256QAM will be defined for CBW 50 MHz then SNR can be increased to 22.4 dB.

Table 3 provides list of parameters used for link level analysis.

Table 3. Link level simulation assumptions.

| Parameter | Value |
| --- | --- |
| CBW | 50 MHz |
| SCS | 120kHz |
| Allocated RBs | Full allocation |
| Propagation | Static (AWGN) TDL-D 30ns delay spread, 35Hz Doppler frequency TDL-A 30ns delay spread, 300Hz Doppler frequency |
| MCS | MCS 20, 21, 22, 23 in TS 38.214 Table 5.1.3.1-2 |
| Antenna configuration | Fading channel: 2x2 for Rank1 and Rank2, Low correlation Static channel: 1x2 for Rank1, 2x2 for Rank2 |
| PDSCH configuration | Type A mapping, Start symbol 1, Duration 13 (for D slots) |
| DMRS configuration | Type 1, Single symbol, 1 additional DMRS |
| PTRS configuration | KPT-RS: 2 (every 2 RBs), LPT-RS: 1 (every 1 symbol) |
| Impairments | TX: EVM = 3% RX: No (Ideal) |

In Table 4 summary of link level simulation results are presented.

Table 4. PDSCH performance

| Rank configuration | Channel model | SNR for 70% of max T-put |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | MCS 20 | MCS 21 | MCS 22 | MCS 23 |
| Rank 1 | Static | 16.1 | 16.6 | 18.0 | 19.3 |
|  | TDL-D | 17.9 | 18.6 | 19.8 | 21.0 |
|  | TDL-A | 18.9 | 19.8 | 21.2 | 22.6 |
| Rank 2 | Static | 19.1 | 19.6 | 20.9 | 22.3 |
|  | TDL-D | 23.9 | 24.8 | 26.4 | 27.8 |
|  | TDL-A | 29.5 | 30.8 | 33.3 | 36.4 |

Based on PDSCH simulation results collection for Rel-15 NR WI, we can observe that difference of ideal and impairment results for FR1 256QAM and FR2 64QAM is about 2.5 – 3 dB. Based on this, we will have the following limitation on SNR for ideal results: 16.4 dB for 100 MHz and 19.4 dB for 50 MHz.

From results in Table 4 we can conclude that

- Scenarios with 100 MHz can be potentially tested only for scenarios with static channel, Rank 1 transmission and MCS 20. However, we suggest not to consider this scenario due to limited margin between SNR bound and simulation results.

- Scenarios with 50 MHz can be potentially tested for the following scenarios:

  - Static channel: Rank 1 + MCS 20-23 and Rank 2 + MCS 20

  - TDL-D channel: Rank 1 + MCS 20-21

  - TDL-A channel: Rank 1 + MCS 20

We suggest to defined requirements for Static channel, Rank 1 and MCS 21, because it allows to achieve sufficient margin relative to the SNR limit.

Proposal 1:	Define FR2 256QAM performance requirements in band agnostic manner for the following assumptions:

- CBW 50 MHz with full allocation

- Propagation conditions: Static

- Antenna configuration: 1x2

- Type 1 single symbol front loaded, 1 additional DMRS

- Rank 1, MCS 21

## SDR requirements

Similar to normal PDSCH requirements, decision on whether to define SDR requirements depends on testable scenarios. Recently, in our paper [7], we analysed the testable SNR for SDR requirements (i.e. testable SNR for Normal and SDR requirements is different due to generation of noise is not needed and more power can be used for generation of useful signal). In Table 5 we provide estimations on testable SNR and MCS (using impairments results from Table 7) for different aggregation factors and different bands for DFF method.

Table 5. Testable SNR and MCS for SDR requirements

| Band |  | Aggregated channel bandwidth, [MHz] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 | 500 | 600 | 700 | 800 | 1000 |
| n257, 258, 261 | TE SNR, [dB] | 32.1 | 29.3 | 26.3 | 23.2 | 22.3 | 21.5 | 20.8 | 20.2 | 18.8 |
|  | Feasible MCS (Rank 1) | MCS27 | MCS27 | MCS26 | MCS23 | MCS22 | MCS22 | MCS21 | MCS21 | N/A |
|  | Feasible MCS (Rank 2) | MCS27 | MCS26 | MCS23 | MCS21 | N/A | N/A | N/A | N/A | N/A |
| n260 | TE SNR, [dB] | 29.5 | 26.7 | 23.7 | 20.6 | 19.7 | 18.9 | 18.2 | 17.6 | 16.2 |
|  | Feasible MCS (Rank 1) | MCS27 | MCS26 | MCS22 | MCS21 | MCS20 | N/A | N/A | N/A | N/A |
|  | Feasible MCS (Rank 2) | MCS26 | MCS23 | MCS21 | N/A | N/A | N/A | N/A | N/A | N/A |

Table 6. PDSCH ideal results

| Rank configuration | MCS 20 | MCS 21 | MCS 22 | MCS 23 | MCS 24 | MCS 25 | MCS 26 | MCS 27 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rank 1 | 16.5 | 17.1 | 18.5 | 19.7 | 20.4 | 21.6 | 22.7 | 24.2 |
| Rank 2 | 19.5 | 20.0 | 21.4 | 22.7 | 23.3 | 24.4 | 25.5 | 27.2 |

Table 7. PDSCH impairments results

| Rank configuration | MCS 20 | MCS 21 | MCS 22 | MCS 23 | MCS 24 | MCS 25 | MCS 26 | MCS 27 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rank 1 | 19.5 | 20.1 | 21.5 | 22.7 | 23.4 | 24.6 | 25.7 | 27.2 |
| Rank 2 | 22.5 | 23.0 | 24.4 | 25.7 | 26.3 | 27.4 | 28.5 | 30.2 |
| Note: Based on simulation results from [R4-1905736], there is about 3 dB difference between average ideal and average impairment results for 64QAM modulation. We expect that SNR difference for 256QAM will be not lower and assume 3 dB difference for these results. |  |  |  |  |  |  |  |  |

From results in Table 5, we can observe that 256QAM MCS and Rank 2 can be tested for channel bandwidth up to 400 MHz for bands n257, n258, n261 and up to 200 MHz for band n260. Also, we can observe that rather high MCSs (i.e. MCS 24-27) can be tested mainly for 50 and 100 MHz aggregated channel bandwidth, which is not the case for SDR requirements. Therefore, we suggest not to define FR2 SDR requirements for 256QAM.

Proposal 2:	Do not define SDR requirements for FR2 256QAM.

# Conclusion

In this paper we provided view on FR2 256QAM performance requirements definition and made the following proposals:

Proposal 1:	Define FR2 256QAM performance requirements in band agnostic manner for the following assumptions:

- CBW 50 MHz with full allocation

- Propagation conditions: Static

- Antenna configuration: 1x2

- Type 1 single symbol front loaded, 1 additional DMRS

- Rank 1, MCS 21

Proposal 2:	Do not define SDR requirements for FR2 256QAM.

# References

- R4-2005531 “WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM”, China Telecom, RAN4 #94-e-Bis.

- TR 38.883 “Study on support of NR downlink 256 Quadrature Amplitude Modulation (QAM) for frequency range 2 (FR2)”

- TR 38.810 “Study on test methods”

- R4-1811394 “Way forward on NR UE PDSCH demodulation requirements and General aspects”, Intel, RAN4 #88, August 2018.

- R4-1816488 “Way forward on NR UE PDSCH demodulation requirements and General aspects”, Intel, RAN4 #89, November 2018.

- R4-1908211 “Discussion on feasibility of DL 256QAM in FR2 scenarios”, Intel, RAN4 #92, August 2019.

- R4-1902881 “NR SDR performance requirements”, Intel, RAN4 #90bis, April 2019.

- R4-1905736 “Summary of simulation results of NR FR2 SDR requirements”, Intel, RAN4 #91, May 2019.