3GPP TSG-RAN WG4 Meeting#95-e	R4-2006628

Online, 25th May – 5th Jun, 2020

Source: 	Qualcomm Incorporated

Title: 	Simulation Results for NR CA PDSCH Demodulation Performance Tests

Agenda item:	6.18.1.1

Document for:	Discussion

# Introduction

In this paper, we present simulation results for NR CA as per simulation assumptions in [1].

# FDD 15kHz

Simulation results are as below.

Table 1: Simulation results for FDD 15kHz

| BW | NRx | SNR in dB at 70% of peak throughput for alignment | SNR in dB at 70% of peak throughput with impairments |
| --- | --- | --- | --- |
| 5 | 2 | 11.16 | 13.16 |
| 10 | 2 | 11.54 | 13.54 |
| 15 | 2 | 11.54 | 13.54 |
| 20 | 2 | 11.81 | 13.81 |
| 25 | 2 | 12.12 | 14.12 |
| 30 | 2 | 11.94 | 13.94 |
| 40 | 2 | 12.19 | 14.19 |
| 50 | 2 | 12.58 | 14.58 |
| 5 | 4 | 6.18 | 8.18 |
| 10 | 4 | 6.39 | 8.39 |
| 15 | 4 | 6.40 | 8.40 |
| 20 | 4 | 6.36 | 8.36 |
| 25 | 4 | 6.55 | 8.55 |
| 30 | 4 | 6.41 | 8.41 |
| 40 | 4 | 6.64 | 8.64 |
| 50 | 4 | 6.94 | 8.94 |

# TDD 15kHz

Simulation results are as below.

Table 2: Simulation results for TDD 15kHz

| BW | NRx | SNR in dB at 70% of peak throughput for alignment | SNR in dB at 70% of peak throughput with impairments |
| --- | --- | --- | --- |
| 5 | 2 | 11.38 | 13.38 |
| 10 | 2 | 11.83 | 13.83 |
| 15 | 2 | 11.73 | 13.73 |
| 20 | 2 | 11.99 | 13.99 |
| 25 | 2 | 12.31 | 14.31 |
| 30 | 2 | 12.07 | 14.07 |
| 40 | 2 | 12.44 | 14.44 |
| 50 | 2 | 12.71 | 14.71 |
| 5 | 4 | 6.38 | 8.38 |
| 10 | 4 | 6.62 | 8.62 |
| 15 | 4 | 6.52 | 8.52 |
| 20 | 4 | 6.42 | 8.42 |
| 25 | 4 | 6.65 | 8.65 |
| 30 | 4 | 6.45 | 8.45 |
| 40 | 4 | 6.75 | 8.75 |
| 50 | 4 | 6.79 | 8.79 |

# TDD 30kHz

Simulation results are as below.

Table 3: simulation results for TDD 30kHz

| BW | NRx | SNR in dB at 70% of peak throughput for alignment | SNR in dB at 70% of peak throughput with impairments |
| --- | --- | --- | --- |
| 5 | 2 | 11.67 | 13.67 |
| 10 | 2 | 11.48 | 13.48 |
| 15 | 2 | 11.57 | 13.57 |
| 20 | 2 | 11.72 | 13.72 |
| 25 | 2 | 11.81 | 13.81 |
| 30 | 2 | 11.72 | 13.72 |
| 40 | 2 | 11.98 | 13.98 |
| 50 | 2 | 12.23 | 14.23 |
| 60 | 2 | 12.06 | 14.06 |
| 80 | 2 | 12.87 | 14.87 |
| 90 | 2 | 12.70 | 14.70 |
| 100 | 2 | 13.21 | 15.21 |
| 5 | 4 | 6.63 | 8.63 |
| 10 | 4 | 6.44 | 8.44 |
| 15 | 4 | 6.45 | 8.45 |
| 20 | 4 | 6.59 | 8.59 |
| 25 | 4 | 6.56 | 8.56 |
| 30 | 4 | 6.50 | 8.50 |
| 40 | 4 | 6.59 | 8.59 |
| 50 | 4 | 6.81 | 8.81 |
| 60 | 4 | 6.72 | 8.72 |
| 80 | 4 | 7.18 | 9.18 |
| 90 | 4 | 7.03 | 9.03 |
| 100 | 4 | 7.46 | 9.46 |

# TDD 120kHz

Simulation results are as below.

Table 4: simulation results for TDD 120kHz

| BW | MCS | Rank | SNR in dB at 70% of peak throughput for alignment | SNR in dB at 70% of peak throughput with impairments |
| --- | --- | --- | --- | --- |
| 50 | 10 | 2 | 8.44 | 10.44 |
| 100 | 10 | 2 | 8.03 | 10.03 |
| 200 | 10 | 2 | 8.02 | 10.02 |
| 400 | 10 | 2 | 8.15 | 10.15 |

# Conclusions

This paper presents the simulation results for NR CA as per assumptions in [1].

# References

[1] R4-2002392, “Simulation assumptions for NR normal CA UE performance requirements”, 3GPP TSG RAN WG4 Meeting #94-e, Online, Feb 24 - Mar 6, 2020.