3GPP TSG-RAN WG4 Meeting # 95-e	 R4-2006701

# Electronic Meeting, 25 May – 5 June, 2020

Source:	LG Electronics

Title:	TP on NR V2X A-MPR for PSSCH/PSCCH

Agenda item:	6.4.3.1

Document for:	Approval

- Introduction

In this contribution, we provide the simulation results for A-MPR for NS_33 and A-MPR for new_NS for the updated ETSI regulations in NR V2X at n47 [1]. Based on the simulation results, we propose A-MPR and TP.

- A-MPR and Simulation results

A-MPR for NS_33

The simulation results are based on simulation assumptions [2]. For fc = 5860MHz, A-MPR should be defined depending on the LCRB and RB start, and for other frequencies, A-MPR could be defined by inner and outer RB allocation for 15kHz and 30kHz SCS as shown in Fig.1~6 and Table 4. Based on simulation results, we propose A-MPR for NR V2X as Table 1 and Table 2.

- Proposal 1: Update A-MPR for NS_33 with Table 1 and Table 2

Table 1 A-MPR for NS_33 at fc = 5860MHz

| Subcarrier spacing (kHz) | Carrier frequency(MHz) | Resources Blocks (LCRB) Note1 | Start Resource Block | A-MPR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | QPSK/16QAM | 64QAM | 256QAM |
| 15 | 5860 | 10 | 0 | ≤ 20 |  |  |
|  |  |  | ≥ 1 and ≤ 3 | ≤ 19 |  |  |
|  |  | 10 and 15 | ≥ 26 and ≤ 38 | ≤ 6 |  |  |
|  |  | ≥ 10 and ≤ 20 | ≥ 12 and ≤ 14 | ≤ 11 |  |  |
|  |  |  | ≥ 15 and ≤ 19 | ≤ 9.5 |  |  |
|  |  |  | ≥ 20 and ≤ 25 | ≤ 7.5 |  |  |
|  |  | ≥ 10 and ≤ 30 | 4 and 5 | ≤ 16 |  |  |
|  |  |  | 6 and 7 | ≤ 14.5 |  |  |
|  |  |  | ≥ 8 and ≤ 11 | ≤ 13.5 |  |  |
|  |  | ≥ 20 and ≤ 30 | ≥ 0 and ≤ 3 | ≤ 19 |  |  |
|  |  | 25 and 30 | ≥ 16 and ≤ 21 | ≤ 9.5 |  |  |
|  |  |  | ≥ 22 and ≤ 27 | ≤ 7.5 |  |  |
|  |  | ≥ 25 and ≤ 40 | ≥ 12 and ≤ 15 | ≤ 12 |  |  |
|  |  | 40 and 45 | 0 and 1 | ≤ 19 |  |  |
|  |  |  | 2 and 3 | ≤ 16 |  |  |
|  |  |  | 4 and 5 | ≤ 14.5 |  |  |
|  |  |  | ≥ 6 and ≤ 11 | ≤ 13.5 |  |  |
|  |  | ≥ 50 | 0 | ≤ 16 |  |  |
| 30 | 5860 | 10 | ≥ 11 and ≤ 14 | ≤ 9 |  |  |
|  |  | 10 and 15 | 0 | ≤ 19 |  |  |
|  |  |  | 1 and 2 | ≤ 17.5 |  |  |
|  |  |  | 3 | ≤ 16 |  |  |
|  |  |  | 4 | ≤ 14.5 |  |  |
|  |  |  | 5 and 6 | ≤ 13.5 |  |  |
|  |  |  | 7 and 8 | ≤ 12 |  |  |
|  |  |  | 9 and 10 | ≤ 10.5 |  |  |
|  |  | ≥ 20 | 0 | ≤ 17.5 |  |  |
|  |  |  | 1 and 2 | ≤ 16 |  |  |
|  |  |  | 3 and 4 | ≤ 14.5 |  |  |
| 60 | 5860 | ≥ 10 | 0 | ≤ 17.5 |  |  |
|  |  | ≥ 10 | 1 | ≤ 16 |  |  |

Table 2 A-MPR for NS_33 at fc = [5870,5880,5890,5900,5910,5920]MHz

| Subcarrier spacing (kHz) | Carrier frequency(MHz) | RB allocations | QPSK | 16QAM | 64QAM | 256QAM |
| --- | --- | --- | --- | --- | --- | --- |
| 15 and 30 | 5870, 5880, 5890, 5900, 5910, 5920 | Inner | ≤ 2.5 |  | ≤ 3 | ≤ 5.5 |
|  |  | Outer | ≤ 3.5 |  | ≤ 4 | ≤ 5.5 |
| 60 | 5870, 5910, 5920 |  | ≤ 5 |  |  | ≤ 5.5 |
|  | 5880, 5890, 5900 |  | ≤ 3.5 |  |  | ≤ 5.5 |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1 [3].

RBStart,Low = max(1, floor(LCRB /3))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low ≤ RBStart ≤ RBStart,High , and

LCRB ≤ NRB

where ceil(x) is the smallest integer greater than or equal to x.

A-MPR for new_NS for the updated ETSI regulations

In last meeting, RAN4 discussed how to study A-MPR with new_NS for the updated ETSI regulations [1]. According to the updated ETSI regulation (EN 302 571), the transmitter unwanted emission limits in the out-of-band domain defined as 250% of the channel bandwidth for the 5GHz ITS frequency band was removed. So, additional SE requirements for new_NS is not required. Based on the simulation results (Figure 6 and Table 5), we propose A-MPR for new_NS as Table 3 to be captured in TR38.886.

However, the A-MPR for new_NS cannot be captured in TS38.101-1 since the updated ETSI regulation has not been officially released. The A-MPR for new_NS will be captured in TS38.101-1 using the value in the TR38.886 once the new ETSI EN 302 571 is officially released.

- Proposal 2: Capture A-MPR for new_NS with Table 3 according to the updated ETSI regulations in the TR38.886.

- Proposal 3: RAN4 will define the A-MPR for new_NS in TS38.101-1 using the values in the TR38.886 once the new ETSI EN 302 571 is officially released.

Table 3 A-MPR for new_NS

| Modulation | A-MPR(dB) |  |
| --- | --- | --- |
|  | Outer RB allocations | Inner RB allocation |
| QPSK | ≤ 4 | ≤ 2 |
| 16QAM | ≤ 4 | ≤ 2 |
| 64QAM | ≤ 4.5 | ≤ 3 |
| 256QAM | ≤ 5.5 |  |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1.

RBStart,Low = max(1, floor(LCRB /2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low ≤ RBStart ≤ RBStart,High , and

LCRB ≤ ceil(NRB/2)

where ceil(x) is the smallest integer greater than or equal to x.

The RB allocation is an Outer RB allocation for all other allocations which are not an Inner RB allocation.

- Conclusion

In this contribution, we provide the updated simulation results for A-MPR for NS_3 in NR V2X at n47 and propose

- Proposal 1: Update A-MPR for NS_33 with Table 1 and Table 2

- Proposal 2: Capture A-MPR for new_NS with Table 3 according to the updated ETSI regulations in the TR38.886

- Proposal 3: RAN4 will define the A-MPR for new_NS in TS38.101-1 using the values in the TR38.886 once the new ETSI EN 302 571 is officially released.

Appendix

------------------- Start of Text Proposal for TR38.886 --------------------

# 8	Transmitter characteristics

## 8.1	NR V2X UE Tx requirements in FR1

------------------      Omit unchanged parts     --------------------

### 8.1.3	UE maximum output power with additional requirements

To comply the EN 302 571 standard emission mask, RAN4 need to derive A-MPR requirements considering with the additional A-SEM and A-SE requirements in EN 302 571 standard as shown in Table 8.1.3-1 and Table 8.1.3-2.

NR V2X UE shall satisfy the additional SEM and additional SE requirements when NS_33 is configured from pre-configured radio parameters or the cell and the indication from upper layers has indicated was informed.

NR V2X UE shall satisfy the additional SEM requirements when NS_xx is configured from pre-configured radio parameters or the cell and the indication from upper layers has indicated was informed.

Note: NS_xx is for new ETSI EN 302 571

Table 8.1.3-1: Additional SEM requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm EIRP)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [-13-12∆fOOBMHz] | 100 kHz |
| 0.5-5 | [-19-169∆fOOBMHz-0.5] | 100 kHz |
| 5-10 | [-27-2∆fOOBMHz-5.0] | 100 kHz |

NOTE 1:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

NOTE 2:	Additional SEM for V2X overrides any other requirements in frequency range 5855-5950MHz.

NOTE 3:	The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex G in TS38.101-1.

Table 8.1.3-2: Additional SE requirements

| n47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65, 68, 72, 73 NR band n77, n78 , n79 | FDL_low | - | FDL_high | -50 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Frequency range | 5925 | - | 5950 | -30 EIRP | 1 | 38, 40, 43 |
|  | Frequency range | 5815 | - | 5855 | -30 EIRP | 1 | 38, 43 |
| NOTE 38: Applicable when NS_33 or NS_34 is configured by the pre-configured radio parameters. NOTE 40: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. NOTE 43: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I. |  |  |  |  |  |  |  |

Also, FCC had regulatory requirements for 40MHz in ITS spectrum as shown in Table 8.1.3-3

Table 8.1.3-3: Additional SEM requirements for 40MHz channel bandwidth (fc =5885 MHz)

| ΔfOOB (MHz) | Emission Limit (dBm) | Measurement Bandwidth |
| --- | --- | --- |
| 0 - 2 | -32 | 100kHz |
| 2-10 | -36 | 100kHz |
| 10-20 | -38 | 100kHz |
| 20-40 | -43 | 100kHz |
| 40 - 100 | -50 | 100kHz |

To comply the regional regulatory requirements, RAN4 specifies the A-MPR requirements according to different network signalling (NS_51) by pre-configured radio parameters or the cell and the indication from upper layers has indicated to NR V2X UE.

The allowed additional Maximum Power Reduction (A-MPR) for the maximum output power due to higher order modulation and transmit bandwidth configuration (resource blocks) will be specified in TS38.101-1 as below.

Table 8.1.3-4: Additional Maximum Power Reduction (A-MPR) for NR V2X

| Network Signalling value | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_33 | Table 8.1.3-1 (A-SEM) Table 8.1.3-2 (A-SE) | n47 | 10 | Table 8.1.3-5 Table 8.1.3-6 |  |
| NS_51 | Table 8.1.3-3 (A-SEM) | n47 | 40 | Table 8.1.3-7 |  |
| NS_xx | Table 8.1.3-1 (A-SEM) | n47 | 10 | Table 8.1.3-8 |  |

Table 8.1.3-5: A-MPR for NS_33 at fc = 5860MHz

| Subcarrier spacing (kHz) | Carrier frequency(MHz) |  | Resources Blocks (LCRB) |  | Start Resource Block |  |  | A-MPR (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | QPSK/16QAM |  | 64QAM |  | 256QAM |
| 15 | 5860 |  | 10 |  | 0 |  |  | ≤ [20] |  |  |  |  |
|  |  |  |  |  | ≥ 1 and ≤ 3 |  |  | ≤ [19] |  |  |  |  |
|  |  |  | 10 and 15 |  | ≥ 26 and ≤ 38 |  |  | ≤ [6] |  |  |  |  |
|  |  |  | ≥ 10 and ≤ 20 |  | ≥ 12 and ≤ 14 |  |  | ≤ [11] |  |  |  |  |
|  |  |  |  |  | ≥ 15 and ≤ 19 |  |  | ≤ [9.5] |  |  |  |  |
|  |  |  |  |  | ≥ 20 and ≤ 25 |  |  | ≤ [7.5] |  |  |  |  |
|  |  |  | ≥ 10 and ≤ 30 |  | 4 and 5 |  |  | ≤ [16] |  |  |  |  |
|  |  |  |  |  | 6 and 7 |  |  | ≤ [14.5] |  |  |  |  |
|  |  |  |  |  | ≥ 8 and ≤ 11 |  |  | ≤ [13.5] |  |  |  |  |
|  |  |  | ≥ 20 and ≤ 30 |  | ≥ 0 and ≤ 3 |  |  | ≤ [19] |  |  |  |  |
|  |  |  | 25 and 30 |  | ≥ 16 and ≤ 21 |  |  | ≤ [9.5] |  |  |  |  |
|  |  |  |  |  | ≥ 22 and ≤ 27 |  |  | ≤ [7.5] |  |  |  |  |
|  |  |  | ≥ 25 and ≤ 40 |  | ≥ 12 and ≤ 15 |  |  | ≤ [12] |  |  |  |  |
|  |  |  | 40 and 45 |  | 0 and 1 |  |  | ≤ [19] |  |  |  |  |
|  |  |  |  |  | 2 and 3 |  |  | ≤ [16] |  |  |  |  |
|  |  |  |  |  | 4 and 5 |  |  | ≤ [14.5] |  |  |  |  |
|  |  |  |  |  | ≥ 6 and ≤ 11 |  |  | ≤ [13.5] |  |  |  |  |
|  |  |  | 50 |  | 0 |  |  | ≤ [16] |  |  |  |  |
| 30 | 5860 |  | 10 |  | ≥ 11 and ≤ 14 |  |  | ≤ [9] |  |  |  |  |
|  |  |  | 10 and 15 |  | 0 |  |  | ≤ [19] |  |  |  |  |
|  |  |  |  |  | 1 and 2 |  |  | ≤ [17.5] |  |  |  |  |
|  |  |  |  |  | 3 |  |  | ≤ [16] |  |  |  |  |
|  |  |  |  |  | 4 |  |  | ≤ [14.5] |  |  |  |  |
|  |  |  |  |  | 5 and 6 |  |  | ≤ [13.5] |  |  |  |  |
|  |  |  |  |  | 7 and 8 |  |  | ≤ [12] |  |  |  |  |
|  |  |  |  |  | 9 and 10 |  |  | ≤ [10.5] |  |  |  |  |
|  |  |  | 20 |  | 0 |  |  | ≤ [17.5] |  |  |  |  |
|  |  |  |  |  | 1 and 2 |  |  | ≤ [16] |  |  |  |  |
|  |  |  |  |  | 3 and 4 |  |  | ≤ [14.5] |  |  |  |  |
| 60 | 5860 |  | 10 |  | 0 |  |  | ≤ [17.5] |  |  |  |  |
|  |  |  | 10 |  | 1 |  |  | ≤ [16] |  |  |  |  |

Table 8.1.3-6: A-MPR for NS_33 at fc = [5870,5880,5890,5900,5910,5920]MHz

| Subcarrier spacing (kHz) | Carrier frequency(MHz) |  |  | RB allocations | QPSK | 16QAM |  | 64QAM |  | 256QAM |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 and 30 | 5870, 5880, 5890, 5900, 5910, 5920 |  |  | Inner | ≤ [2.5] |  |  | ≤ [3] |  | ≤ [5.5] |  |
|  |  |  |  | Outer | ≤ [3.5] |  |  | ≤ [4] |  | ≤ [5.5] |  |
| 60 | 5870, 5910, 5920 |  |  |  | ≤ [5] |  |  |  |  | ≤ [5.5] |  |
|  | 5880, 5890, 5900 |  |  |  | ≤ [3.5] |  |  |  |  | ≤ [5.5] |  |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1 [3].

RBStart,Low = max(1, floor(LCRB /3))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low ≤ RBStart ≤ RBStart,High , and

LCRB ≤ NRB

where ceil(x) is the smallest integer greater than or equal to x.

Table 8.1.3-7: A-MPR for NS_51

| Carrier frequency(MHz) | Modulation | A-MPR(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocation |
| 5885 | QPSK | ≤ (9.5 + ΔNote1) | [≤ 8.0] | [≤ 3.5] |
|  | 16QAM |  | [≤ 8.0] | [≤ 3.5] |
|  | 64QAM |  | [≤ 8.5] | [≤ 4.5] |
|  | 256QAM |  | [≤ 8.5] | [≤ 6.0] |
| Note1: Δ is 0, 3, and 5 for 60kHz, 30kHz, and 15kHz SCS, respectively. |  |  |  |  |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1 [3].

RBStart,Low = max(1, floor(NRB /3.5))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low ≤ RBStart ≤ RBStart,High , and

LCRB ≤ ceil(NRB/3.5)

where ceil(x) is the smallest integer greater than or equal to x.

An Edge RB allocation is the one for which the RB(s) is (are) allocated at the lowermost or uppermost edge of the channel with LCRB ≤ floor(NRB*0.2) RBs.

The RB allocation is an Outer RB allocation for all other allocations which are not an Inner RB allocation or Edge RB allocation.

Table 8.1.3-8: A-MPR for NS_xx

| Modulation | A-MPR(dB) |  |
| --- | --- | --- |
|  | Outer RB allocations | Inner RB allocation |
| QPSK | ≤ [4] | ≤ [2] |
| 16QAM | ≤ [4] | ≤ [2] |
| 64QAM | ≤ [4.5] | ≤ [3] |
| 256QAM | ≤ [5.5] |  |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1 [3].

RBStart,Low = max(1, floor(LCRB /2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low ≤ RBStart ≤ RBStart,High , and

LCRB ≤ ceil(NRB/2)

where ceil(x) is the smallest integer greater than or equal to x.

The RB allocation is an Outer RB allocation for all other allocations which are not an Inner RB allocation.

------------------ End of Text Proposal for TR38.886 --------------------

Reference

[1] R4-2005223, “WF on MPR/A-MPR requirements for PSSCH/PSCCH transmission,” LG Electronics

[2] R4-2002783, “A-MPR simulation assumptions and initial results for NR V2X UE at n47,” LG Electronics

[3] TS 38.101-1.

Appendix

Simulation results for A-MPR (NS_33)

Figure 1 A-MPR for NR V2X with QPSK and 15kHz SCS

Figure 2 A-MPR for NR V2X with 64QAM and 15kHz SCS

Figure 3 A-MPR for NR V2X with 256QAM and 15kHz SCS

Figure 4 A-MPR for NR V2X with QPSK and 30kHz SCS

Figure 5 4 A-MPR for NR V2X with 64QAM and 30kHz SCS

Figure 6 A-MPR for NR V2X with 256QAM and 30kHz SCS

Table 4 A-MPR for NR V2X with 60kHz SCS

| Modulation order | Resource Blocks (NRB) | Start Resource Block | Carrier Frequency (MHz) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5860 | 5870 | 5880 | 5890 | 5900 | 5910 | 5920 |
| QPSK | 10 | 0 | 17.1 | 4.4 | 2.7 | 2.7 | 2.7 | 4.0 | 4.0 |
|  | 10 | 1 | 15.6 | 4.0 | 2.6 | 2.6 | 2.6 | 4.4 | 4.4 |
| 16QAM | 10 | 0 | 17.0 | 4.7 | 2.7 | 2.6 | 2.7 | 4.0 | 4.0 |
|  | 10 | 1 | 15.6 | 4.0 | 2.6 | 2.6 | 2.7 | 4.4 | 4.5 |
| 64QAM | 10 | 0 | 17.2 | 4.4 | 2.7 | 2.7 | 2.7 | 4.0 | 3.8 |
|  | 10 | 1 | 15.4 | 4.1 | 2.6 | 2.6 | 2.6 | 4.4 | 4.4 |
| 256QAM | 10 | 0 | 17.1 | 4.8 | 4.7 | 4.7 | 4.7 | 4.5 | 4.6 |
|  | 10 | 1 | 15.6 | 4.5 | 4.6 | 4.7 | 4.6 | 4.6 | 4.6 |

Simulation results for A-MPR (new_NS)

Figure 7 A-MPR for NR V2X by new_NS

A-MPR values for 10MHz BW and 60kHz SCS by new_NS are summarized in following table.

Table 5 A-MPR for NR V2X with 60kHz SCS

| Resource Blocks (NRB) | Start Resource Block | A-MPR |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | QPSK | 16QAM | 64QAM | 256QAM |
| 10 | 1 | 2.8 | 2.8 | 3 | 4.7 |
| 10 | 2 | 2.8 | 2.8 | 3 | 4.7 |