3GPP TSG-RAN WG4 Meeting # 95-e 	R4-2006706

# Electronic Meeting, 25 May – 5 June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 686 | Rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR of interruption for switching between NR SL and LTE SL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-05-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduce interruption requirement for switching between NR SL and LTE SL by taking RF session’s conclusion into account |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce interruption requirement for switching between NR SL and LTE SL |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Miss corresponding RRM interruption requirement |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< START OF CHANGE #1 >

12.7.3	V2X Sidelink Communication Dropping due to switching between NR V2X Sidelink and E-UTRA V2X Sidelink

This sub-clause contains the requirements related to the interruptions on the V2X sidelink communication due to switching between NR V2X sidelink and E-UTRA V2X sidelink. It is applicable for UE capable of both NR V2X sidelink and E-UTRA V2X sidelink in time division manner.

The UE is allowed an interruption of up to one slot when switching from E-UTRA V2X sidelink to NR V2X sidelink in NR slot.

When switching from NR V2X sidelink to E-UTRA V2X sidelink in NR slot

- the UE is allowed an interruption of up to the duration shown in table 12.7.3.-1 if the NR slot includes PSCCH and PSSCH except for other V2X sidelink signals.

- Otherwise, the UE is allowed an interruption of up to one slot.

Table 12.7.3-1: Interruption length when swiching from NR V2X sidelink to E-UTRA V2X sidelink in NR slot including PSCCH and PSSCH except for other V2X sidelink signals

|  | NR Slot length (ms) | ld (symbols) | Interruption length (number of slots) |
| --- | --- | --- | --- |
| 0 | 1 | ≤ 11 | N/A |
|  |  | > 11 | 1 |
| 1 | 0.5 | ≤ 9 | N/A |
|  |  | > 9 | 1 |
| 2 | 0.25 | 6~13 | 1 |
| NOTE 1: ld is duration of scheduled resources for transmission of PSSCH and the associated PSCCH [6] |  |  |  |

The UE is allowed an interruption of up to one subframe when switching from E-UTRA V2X sidelink to NR V2X sidelink in E-UTRA subframe.

< END OF CHANGE #1 >