3GPP TSG-RAN WG4 Meeting #95-e		                                     R4-2006894

Electronic Meeting, 25 May – 05 June 2020

# Source:	Thales, Hughes Network Systems Ltd, Intelsat

# Title:	Possible bands for NR based satellite networks

# Type:	discussion

# Document for:	Information

# Agenda Item:	10

Release	Rel-17

# Introduction

The objective of this document is to provide for information a list of all frequency bands that are allocated to Satellite networks in the different ITU regions here below depicted:

Figure 1: ITU regions and the dividing lines between them.  Region 1 (Yellow), Region 2 (Blue), Region 3 (Purple)

Any of these frequency bands could be considered for the operation of Satellite network using NR radio interface (see RAN2 WI: NR_NTN_solutions). The use of some frequency bands may require coexistence studies if not already concluded in ITU-R.

# Possible frequency bands for NR based Satellite networks

## L band spectrum for Satellite services

The following bands are referred as L band and identified by the ITU Radio Regulation for use by GEO based Mobile Satellite Services (MSS) on a worldwide basis.

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space to earth) | 1518 – 1559 MHz | 1518 – 1559 MHz (Note 1) | 1518 – 1559 MHz |
| Uplink (earth to space) | 1626.5 – 1660.5 MHz & 1668 – 1675 MHz | 1626.5 – 1660.5 MHz & 1668 – 1675 MHz (Note 2) | 1626.5 – 1660.5 MHz & 1668 – 1675 MHz |

Note 1: In the table above, the frequency band 1518 - 1525 MHz is not allocated to MSS domestically in the US.

Note 2: In the table above, the frequency band 1668-1675 MHz is not allocated to MSS domestically in the US. The band 1670 – 1675 MHz is allocated for fixed and mobile services in US.

Note: In the table above, the identified spectrum bands can also be used by Complementary Ground Component (CGC) or Ancillary Terrestrial Component (ATC) of the network

The following bands are referred as L band and identified by the ITU Radio Regulation for use by Non-GEO based Mobile Satellite Services on a worldwide basis.

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space to earth) | 1613.8 – 1626.5 MHz (Note 2) | 1613.8 – 1626.5 MHz (Note 3) | 1613.8 – 1626.5 MHz (Note 2) |
| Uplink (earth to space) | 1610.0 – 1626.5 MHz | 1610.0 – 1626.5 MHz | 1610.0 – 1626.5 MHz |

Note 3: In the table above, the 1613.8-1626.5 MHz segment is allocated for downlink operations (space-to-Earth) on a secondary basis.

## 2.2	  S band spectrum for Satellite services

The following bands are referred as S band and identified for use by ITU Radio Regulation via GEO or Non GEO based Mobile Satellite Services to provide International Mobile Telecommunications (IMT) on a worldwide basis.

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space to earth) | 2170 - 2200 MHz & 2483.5 - 2500 MHz | 2160 - 2200 MHz & 2483.5 - 2500 MHz | 2170 - 2200 MHz & 2483.5 - 2500 MHz |
| Uplink (earth to space) | 1980 - 2010 MHz | 1980 - 2025 MHz | 1980 - 2010 MHz |

Note: In the table above, the identified spectrum bands can also be used by Complementary Ground Component (CGC) or Ancillary Terrestrial Component (ATC) of the network.

## 2.3		C band spectrum for Satellite services

The following bands are referred as C band and are identified by the ITU Radio Regulation for use by GEO based satellite services (Fixed and/or Mobile). The sub-bands 3700-4200 MHz and 5925-6425 MHz may be used by Non GEO systems, subject to protection of GEO systems.

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space to earth) | 3400 - 4200 MHz and 4500 - 4800 MHz | 3400 - 4200 MHz and 4500 - 4800 MHz | 3400 - 4200 MHz and 4500 - 4800 MHz |
| Uplink (earth to space) | 5725 - 7025 MHz | 5850 - 7025 MHz | 5850 - 7025 MHz |

Note: the use of the bands 4500-4800 MHz and 6725-7025 MHz by GEO systems is subject to the provisions of ITU Radio Regulation Appendix 30B.

## 2.4		Ku band spectrum for Satellite services

The following bands are referred as Ku bands and are identified by the ITU Radio Regulations for use by GEO and Non GEO based satellite services (Fixed and/or Mobile), subject to protection of GEO systems.

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space to earth) | 10.7 - 12.75 GHz | 10.7 - 12.7 GHz | 10.7 - 12.75 GHz |
| Uplink (earth to space) | 12.75 - 13.25 GHz & 13.75 - 14.5 GHz | 12.75 - 13.25 GHz & 13.75 - 14.5 GHz | 12.75 - 13.25 GHz & 13.75 - 14.5 GHz |

Note1: the use of the bands 12.75-13.25 GHz, 10.7-10.95 GHz and 11.2-11.45 GHz by GEO systems is subject to the provisions of ITU Radio Regulation Appendix 30B.

Note2: the use of the bands 11.7-12.5 GHz in Region 1, 12.2-12.7 GHz in Region 2 and 11.7-12.2 GHz in Region 3 by GEO systems is subject to the provisions of ITU Radio Regulation Appendix 30.

## 2.5   Ka band spectrum for Satellite services

The following bands are referred as Ka band and identified by ITU Radio Regulation for use by GEO based Satellite services (Fixed and/or Mobile) on a worldwide basis.

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space to earth) | 17.3 – 20.2 GHz | 17.7 – 20.2 GHz | 17.7 – 20.2 GHz |
| Uplink (earth to space) | 27.5 – 30.0 GHz | 27.0 – 30.0 GHz | 27.0 – 30.0 GHz |

The following bands are referred as Ka band and identified for use by Non GEO based Satellite services (Fixed and/or Mobile) on a worldwide basis, subject to protection of GEO systems.

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space to earth) | 17.7 – 20.2 GHz | 17.7 – 20.2 GHz | 17.7 – 20.2 GHz |
| Uplink (earth to space) | 27.5 – 29.1 GHz & 29.5 – 30.0 GHz | 27.0 – 29.1 GHz & 29.5 – 30.0 GHz | 27.0 – 29.1 GHz & 29.5 – 30.0 GHz |

## 2.6		Q/V band spectrum for Satellite services

The following bands are referred as Q and V bands and are identified by the ITU Radio Regulation for use by GEO and Non-GEO based satellite services (Fixed and/or Mobile) subject to protection of GEO systems.

|  | Region 1 | Region 2 | Region 3 |
| --- | --- | --- | --- |
| Downlink (space to earth) | 37.5 – 42.5 GHz, 47.5 - 47.9 GHz, 48.2 - 48.54 GHz & 49.44 - 50.2 GHz | 37.5 – 42.5 GHz | 37.5 – 42.5 GHz |
| Uplink (earth to space) | 42.5 – 43.5 GHz, 47.2 – 50.2 GHz & 50.4 – 51.4 GHz | 42.5 – 43.5 GHz, 47.2 – 50.2 GHz & 50.4 – 51.4 GHz, | 42.5 – 43.5 GHz, 47.2 – 50.2 GHz & 50.4 – 51.4 GHz |

Note: The note 5.516В of the ITU RR identifies the following segments for high-density fixed-satellite service applications:

| Band | Direction | Region |
| --- | --- | --- |
| 39.5-40 GHz | (space-to-Earth) | Region 1 |
| 40-40.5 GHz | (space-to-Earth) | All Regions |
| 40.5-42 GHz | (space-to-Earth) | Region 2 |
| 47.5-47.9 GHz | (space-to-Earth) | Region 1 |
| 48.2-48.54 GHz | (space-to-Earth) | Region 1 |
| 49.44-50.2 GHz | (space-to-Earth) | Region 1 |
| 48.2-50.2 GHz | (Earth-to-space) | Region 2 |

END