3GPP TSG-RAN WG4 Meeting # 95-e                	R4-2007098

Electronic Meeting, 25 May – 5 June, 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on CSI-RS measurement bandwidth

Agenda item:	6.16.1.1

Document for:	Discussion

Introduction

During RAN#94bis-e meeting, it was agreed to define single requirement for CSI-RS based RRM measurement at least for the CSI-RS configuration {D=3 with PRBs ≥ 48} as below [1]. It is still open whether to define additional CSI-RS configuration. In this contribution, we provide our view on the aspect.

  - Define requirements for CSI-RS configuration with {D=3 with PRBs ≥ 48}. FFS whether to define requirements for {D=1 with PRBs ≥ 96}.

  - Not define additional CSI-RS based measurement requirement

Discussion

The functionality of CSI-RS based RRM measurements has been concluded in RAN1/2. According to [2], for one neighbor cell to be measured, a UE shall measure the CSI-RS resources within the configured bandwidth indicated by csi-rs-MeasurementBW and density, where the measurement bandwidth ranges from 24PRBs to 264PRBs, and the density is set to 1 or 3. In the reply LS to RAN2 [3], RAN1 further confirms that all the CSI-RS configurations are applicable for inter-cell mobility as cited below.

Question#1: What is the CSI-RS pattern and its variability in time when the CSI-RS is transmitted for the purpose of connected mode inter-cell mobility?

Response#1: In Rel-15, CSI-RS transmitted for the purpose of connected mode inter-cell mobility can be configured with a periodicity of 4, 5, 10, 20, 40 ms in time, located on one indicated symbol within the corresponding slots, spanning 24, 48, 96, 192, or 264 contiguous RBs in frequency, occupying 1 or 3 RE(s) per RB, while the exact position within an RB is also configurable.

As seen from the simulation results in Fig.1, the accuracy performance with density = 3 is better than that with density=1 because more REs are measured achieving better averaging. However, a larger density means more resources are required for CSI-RS transmission, which brings signaling overhead and degrades the system performance. Such difference shall be reflected when defining the requirement to help the network configuration.

With CSI-RS configuration {D=3 with PRBs ≥ 48}, the network is expecting a qualified measurement performance when CSI-RS bandwidth to be measured is equal or higher than 48PRBs. However, if density = 1 is configured, the performance under CSI-RS resource with 48PRB is significantly degraded. The UE shall at least measure 96PRB in order to achieve comparable performance with {D=3 with PRBs ≥ 48}. It is proposed to define additional CSI-RS configuration {D=1 with PRBs ≥ 96} for the CSI-RS based measurement requirement.

Proposal1: It is proposed to define additional CSI-RS configuration {D=1 with PRBs ≥ 96} for the CSI-RS based measurement requirement.

Density = 1						Density = 3

Figure 1 CSI-RS based measurement performance under 4GHz 15kHz SCS

Conclusion

This contribution discussed whether to define additional CSI-RS configuration for the CSI-RS based RRM measurement requirement. The proposal is summarized as below:

Proposal1: It is proposed to define additional CSI-RS configuration {D=1 with PRBs ≥ 96} for the CSI-RS based measurement requirement.

References

- R4-2005355, WF on CSI-RS configuration and intra/inter-frequency measurements definition for CSI-RS based L3 measurement

- 3GPP TS38.331, NR Radio Resource Control

- R1-1907810, Reply LS on CSI-RS configuration transfer over Xn, Huawei