3GPP TSG-RAN WG4 Meeting #95-e										  	     		    R4-2007122

Electronic meeting, 25th May – 5th June, 2020

Source: 	Nokia, Nokia Shanghai Bell

Title: 					TP to TR 38.809: Emission requirements

Agenda item:			6.5.2.2.1

Document for:			Approval

# 1	Introduction

A text proposal to TR 38.174 is provided based on earlier agreements summarized below.

From RAN4#94bis-e chairman minutes

- In FR2 wide area IAB-MT shall re-use BS relative ACLR requirements, local area IAB-MT shall meet 24 dBc ACLR.

From [1]

- OOB boundary in FR2 is 1.5 GHz

From [2]

- For wide area IAB-MT, the multi-band, multi-carrier and CA requirements follow BS approach

The changes are done on top of the TR version agreed in R4-2005458.

This TP is intended to be further updated, if needed, based on feedback and agreements in RAN4#95e.

# 2	References

[1]		R4-2005494, TP to TS 38.174: Addition of OTA IAB-Node out-of-band receiver blocking requirement in subclause 10.6, Ericsson

[2]		R4-2005482, WF on IAB system parameters, Huawei

<Start of text proposal>

## 9.7	OTA unwanted emissions

Detailed structure of the subclause is TBD.

### 9.7.1	Out-of-band boundary

### In FR2, IAB-MT out-of-band boundary is set to 1.5 GHz.

### 9.7.2	Relative ACLR in FR2

#### 9.7.2.1 Wide area IAB-MT relative ACLR

### Based on the results of the co-existence study and taking into account the characteristic of the deployment scenarios applicable for different IAB-MT classes, it was agreed that wide area IAB-MT can re-use BS type 2-O requirements for relative ACLR. This results in specifying 28 dBc ACLR for 24.25 – 33.4 GHz and 26 dBc ACLR for 37 – 52.6 GHz.

### For wide area IAB-MT, the multi-band, multi-carrier and CA requirements follow BS approach. Therefore, also ACLR and CACLR requirements for non-contiguous spectrum are re-used from TS 38.104.

#### 9.7.2.1 Local area IAB-MT relative ACLR

### Based on the results of the co-existence study and taking into account the characteristic of the deployment scenarios applicable for different IAB-MT classes, it was agreed that local area IAB-MT needs to meet 24 dBc ACLR.

## 9.8	OTA transmitter intermodulation

## <End of text proposal>