3GPP TSG-RAN4 Meeting #95-e	R4-2007739

# Electronic Meeting, 25 May – 5 June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0795 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on CSI-RS based measurement requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CSIRS_L3meas-Core |  |  |  |  |  | Date: |  |  | 2020-05-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CSI-RS based measurement requirements needs to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify CSI-RS based measurement requirements -CSI-RS intra-frequency measurement without gap -CSI-RS intra-frequency measurement with gap |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CSI-RS measurement requirements are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 9.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

9.8	CSI-RS L3 measurements

9.8.1	Introduction

TBA

9.8.2	Intra-frequency measurements

9.8.2.1	Introduction

TBA

9.8.2.2	Requirements applicability

TBA

9.8.2.3	Number of cells and number of CSI-RS resources

TBA

9.8.2.4	Measurement Reporting Requirements

TBA

9.8.2.5	Intra-frequency measurements without measurement gaps

If a UE is configured with the higher layer parameters CSI-RS-Resource-Mobility and associatedSSB, the UE may base the timing of the CSI-RS resource on the timing of the cell given by the cellId of the CSI-RS resource configuration. Additionally, for a given CSI-RS resource, if the associated SS/PBCH block is configured but not detected by the UE, the UE is not required to monitor the corresponding CSI-RS resource.

If associatedSSB is unkown, the CSI-RS based measurement shall include cell identification time of associatedSSB and CSI-RS based measurement period without gap.

- Cell identification time of associatedSSB is the intra-frequency period for cell identification Tidentify_intra_with_index in Section 9.2.5.1 or in section 9.2.6.2 or Tidentify_inter_with_index in section 9.3.4.

- The measurement period for intrafrequency measurements without gaps is as shown in table 9.8.2.5-1, Table 9.8.2.5-2.

If associatedSSB is known, the measurement period for intrafrequency measurements without gaps is as shown in table 9.8.2.5-1, 9.8.2.5-2.

Editeral note: the requirements for the case that the multiple CSI-RS resources from different cells are transmitted in the same OFDM symbols in one MO, and the CSI-RS resources are QCL-ed with different associated SSB are FFS.

If SCG DRX is in use, intrafrequency measurement period requirements specified in table 9.8.2.5-1, Table 9.8.2.5-2, shall depend on the SCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply.

Table 9.8.2.5-1: Measurement period for intrafrequency CSI-RS based measurements without gaps(Frequency FR1)

| DRX cycle | T SCI-RS_measurement_period_intra |
| --- | --- |
| No DRX | max(200ms, 5 x CMTC period) x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, 5 x max(CMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | 5 x DRX cycle x CSSFintra |
| NOTE 1: The requirements apply assuming CSI-RS configuration with {D=3 with PRBs ≥ 48}. |  |

Table 9.8.2.5-2: Measurement period for intrafrequency CSI-RS based measurements without gaps(Frequency FR2)

| DRX cycle | T CSI-RS_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_w/o_gaps x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(Mmeas_period_w/o_gaps x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: The requirements apply assuming CSI-RS configuration with {D=3 with PRBs ≥ 48}. |  |

Mmeas_period_w/o_gaps : For a UE supporting power class 1, Mmeas_period_w/o_gaps =40. For a UE supporting FR2 power class 2, Mmeas_period_w/o_gaps =24. For a UE supporting power class 3, Mmeas_period_w/o_gaps =24. For a UE supporting power class 4, Mmeas_period_w/o_gaps =24.

9.8.2.6	Intra-frequency measurements with measurement gaps

If a UE is configured with the higher layer parameters CSI-RS-Resource-Mobility and associatedSSB, the UE may base the timing of the CSI-RS resource on the timing of the cell given by the cellId of the CSI-RS resource configuration. Additionally, for a given CSI-RS resource, if the associated SS/PBCH block is configured but not detected by the UE, the UE is not required to monitor the corresponding CSI-RS resource.

If associatedSSB is not indentified by the UE, the CSI-RS based measurement shall include cell identification time of associatedSSB and CSI-RS based measurement period with gap.

- Cell identification time of associatedSSB is the intra-frequency period for cell identification Tidentify_intra_with_index in Section 9.2.5.1 or in section 9.2.6.2 or Tidentify_inter_with_index in section 9.3.4.

- The measurement period for intrafrequency measurements with gaps is as shown in table 9.8.2.6-1, Table 9.8.2.6-2.

If associatedSSB is already indentified by the UE, the measurement period for intrafrequency measurements with gaps is as shown in table 9.8.2.6-1, 9.8.2.6-2.

Editeral note: the requirements for the case that the multiple CSI-RS resources from different cells are transmitted in the same OFDM symbols in one MO, and the CSI-RS resources are QCL-ed with different associated SSB are FFS.

If SCG DRX is in use, intrafrequency measurement period requirements specified in table 9.8.2.6-1, Table 9.8.2.6-2, shall depend on the SCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply.

Table 9.8.2.6-1: Measurement period for intrafrequency CSI-RS based measurements with gaps(Frequency FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200ms, 5 x max(MGRP, CMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, 5 x max(MGRP, CMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | 5 x max(MGRP, DRX cycle) x CSSFintra |

Table 9.8.2.6-2: Measurement period for intrafrequency CSI-RS based measurements with gaps(Frequency FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, Mmeas_period with_gaps x max(MGRP, CMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, Mmeas_period with_gapsx max(MGRP, SMTC period, DRX cycle) x CSSFintra |
| DRX cycle>320ms | Mmeas_period with_gaps x max(MGRP, DRX cycle) x CSSFintra |

Mmeas_period_ with_gaps: For a UE supporting power class 1, Mmeas_period_ with_gaps =40. For a UE supporting power class 2, Mmeas_period_ with_gaps =24. For a UE supporting power class 3, Mmeas_period_ with_gaps =24. For a UE supporting power class 4, Mmeas_period with_gaps =24.

<End of Change 1>