3GPP TSG-RAN WG4 Meeting #95-e	R4-2008133

# Electronic Meeting, 25 May – 6 June, 2020

Source: 	Skyworks Solutions, Inc.

Title: 	[NR_n65_BW] Back-off Measurements

Agenda Item:	8.20.1	UE RF (38.101-1)	[NR_n65_BW]

Document for:	Discussion

# Introduction

This contribution presents power amplifier (PA) back-off measurements for n65 operation at 50MHz channel bandwidth.

# Discussion

Measurements are performed using the agreed assumptions from WF [1], as follows:

- Band 34 protection: carrier frequency = 1980MHz and 1975MHz, 0dB filter rejection; and

- Two sets of PA bias are used: 1 set for high power mode for low to medium back-off, 1 set for mid/low power mode for medium to high back-off.

## Power Amplifier Calibration

- PA calibration point is 20 MHz, 15 kHz, QPSK, DFT-S-OFDM, 100 RB at lower channel edge with 1 dB MPR;

- Post PA losses: 4dB;

- Power Class 3 (PC3) operation;

- Local Oscillator (LO) leakage: -28dBc;

- IQ Image rejection: -28dB;

- Counter-IM3 (C-IM3): -60dBc; and

- Counter-IM5 (C-IM5): -70dBc.

## Measurement Results

### B34 Protection Measurement Results

Table 1 below summarizes measurement results to meet band 34 protection requirements for operation at 1980MHz and 1975MHz.

Table 1: Measured PC3 back-off levels for band 34 protection at 50MHz CBW, carrier = 1975MHz and 1980MHz

| RB length | RB start | PC3 back-off 1975 MHz [dB] | PC3 back-off 1980 MHz [dB] |
| --- | --- | --- | --- |
| 1 | 0 | 4.6 | 1.6 |
| 1 | 6 | 4.6 | 1.6 |
| 1 | 18 | 4.5 | 4.4 |
| 1 | 30 | 4.6 | 4.4 |
| 1 | 42 | 13.8 | 4.7 |
| 1 | 54 | 13.9 | 15.0 |
| 1 | 66 | 14.0 | 14.9 |
| 1 | 78 | 15.1 | 15.1 |
| 1 | 201 | 17.0 | 17.1 |
| 1 | 235 | 16.9 | 16.5 |
| 1 | 252 | 16.7 | 16.6 |
| 1 | 170 | 1.6 | 1.6 |
| 1 | 205 | 16.5 | 16.6 |
| 270 | 0 | 17.9 | 18.0 |
| 200 | 0 | 16.1 | 16.2 |
| 150 | 0 | 12.3 | 13.1 |
| 100 | 0 | 11.6 | 12.1 |
| 200 | 70 | 17.5 | 18.1 |
| 150 | 120 | 18.2 | 20.0 |
| 100 | 170 | 18.4 | 19.6 |
| 50 | 220 | 13.8 | 20.3 |

The intention here is to provide reference measurements points to calibrate simulation results.

The highest observed raw back-off level is approximately 20dB and 18dB for PC3 operation at 1980MHz and 1975MHz, respectively.

Observation 1:

Band 34 protection highest PA back-off for PC3 operation in n65 50MHz operation is approximately 20dB and 18dB for operation at 1980 and 1975MHz, respectively.

# Conclusions

In this contribution, we present our measurements for QPSK waveforms for n65 50MHz operation.

Observation 1:

Band 34 protection highest PA back-off for PC3 operation in n65 50MHz operation is approximately 20dB and 18dB for operation at 1980 and 1975MHz, respectively.

# References

[1] R4-2002856 “WF on UE RF requirements for adding channel BW to band n65”, Ericsson, Skyworks, 3GPP TSG-RAN WG4 Meeting #94-e, electronic meeting, February 24-March 06 2020.