3GPP TSG-RAN WG4 Meeting #95-e	R4-2008284

# Electronic Meeting, 25th May - 5th Jun, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | - | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR addition on FRC and propagation conditions definition for LTE-based 5G terrestrial broadcast |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_terr_bcast-Perf |  |  |  |  |  | Date: |  |  | 2020-05-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduce FRC and propagation conditions for performance requirements for LTE-based 5G terrestrial broadcast as per agreements in RAN4#94e-bis |  |  |  |  |  |  |  |  |
| Summary of change: |  | For FRC, add Table A.3.8.1-9 for PMCH 0.37kHz SCS and Table A.3.8.1-10 for PMCH 2.5kHz SCS, update Table A.3.7-1 for PBCH repetition, update Table A.3.1.1-1N and Table A.3.1.1-1O for overview. For propagation conditions for PMCH 0.37kHz SCS, add section B.2.6.3 and Table B.2.6.3-1. For propagation conditions for PMCH 2.5kHz SCS, add section B.2.6.4 and Table B.2.6.4-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FRC and propagation conditions for performance requirements for LTE-based 5G terrestrial broadcast will remain undefined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.1.1, A.3.7, A.3.8.1, B.2.6.3, B.2.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of the change 1>

A.3.1.1	Overview of DL reference measurement channels

In Table A.3.1.1-1 to A.3.1.1-1V are listed the DL reference measurement channels specified in annexes A.3.2 to A.3.15 of this release of TS 36.101. This table is informative and serves only to a better overview. The reference for the concrete reference measurement channels and corresponding implementation’s parameters as to be used for requirements are annexes A.3.2 to A.3.15 as appropriate.

<Unchanged parts are skipped>

Table A.3.1.1-1N: Overview of DL reference measurement channels (PBCH Performance)

| Duplex | Table | Name | BW | Mod | TCR | RB | RB Offset | UE Categ | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD / TDD | Table A.3.7-1 | R.21 | 1.4 | QPSK | 40/ 1920 |  |  |  |  |
| FDD / TDD | Table A.3.7-1 | R.22 | 1.4 | QPSK | 40/ 1920 |  |  |  |  |
| FDD / TDD | Table A.3.7-1 | R.23 | 1.4 | QPSK | 40/ 1920 |  |  |  |  |
| FDD / TDD | Table A.3.7-1 | R.23-1 | 1.4 | QPSK | 40/ 4416 |  |  |  |  |

<Unchanged parts are skipped>

Table A.3.1.1-1O: Overview of DL reference measurement channels (PMCH Performance)

| Duplex | Table | Name | BW | Mod | TCR | RB | RB Offset | UE Categ | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD |  |  |  |  |  |  |  |  |  |
| FDD | Table A.3.8.1-1 | R.40 FDD | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| FDD | Table A.3.8.1-1 | R.37 FDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.8.1-2 | R.38 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 1 |  |
| FDD | Table A.3.8.1-3 | R.39-1 FDD | 5 | 64QAM | 2/3 | 25 |  | ≥ 1 |  |
| FDD | Table A.3.8.1-3 | R.39 FDD | 10 | 64QAM | 2/3 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-4 | R.81-1 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-4 | R.81-2 FDD | 10 | 64QAM | 2/3 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-5 | R.82-1 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-6 | R.83-1 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-6 | R.83-2 FDD | 10 | 64QAM | 2/3 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-7 | R.84-1 FDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-8 | R.85-1 FDD | 3 | QPSK | 1/3 | 15 |  | ≥ 1 |  |
| FDD | Table A.3.8.1-8 | R.85-2 FDD | 5 | 16QAM | 1/2 | 25 |  | ≥ 1 |  |
| FDD | Table A.3.8.1-8 | R.85-3 FDD | 10 | 64QAM | 2/3 | 50 |  | ≥ 2 |  |
| FDD | Table A.3.8.1-9 | R.106-1 FDD | 10 | 64QAM | 0.48 | 50 |  | [TBD] |  |
| FDD | Table A.3.8.1-9 | R.106-2 FDD | 10 | 64QAM | 0.52 | 50 |  | [TBD] |  |
| FDD | Table A.3.8.1-10 | R.107 FDD | 10 | 16QAM | 0.46 | 50 |  | ≥ 2 |  |
| TDD |  |  |  |  |  |  |  |  |  |
| TDD | Table A.3.8.2-1 | R.40 TDD | 1.4 | QPSK | 1/3 | 6 |  | ≥ 1 |  |
| TDD | Table A.3.8.2-1 | R.37 TDD | 10 | QPSK | 1/3 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.8.2-2 | R.38 TDD | 10 | 16QAM | 1/2 | 50 |  | ≥ 1 |  |
| TDD | Table A.3.8.2-3 | R.39-1 TDD | 5 | 64QAM | 2/3 | 25 |  | ≥ 1 |  |
| TDD | Table A.3.8.2-3 | R.39 TDD | 10 | 64QAM | 2/3 | 50 |  | ≥ 2 |  |

<End of the change 1>

<Start of the change 2>

A.3.7 Reference measurement channels for PBCH performance requirements

Table A.3.7-1: Reference Channel FDD/TDD

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.21 | R.22 | R.23 | R.23-1 |
| Number of transmitter antennas |  | 1 | 2 | 4 | 1 |
| Channel bandwidth | MHz | 1.4 | 1.4 | 1.4 | 1.4 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| Target coding rate |  | 40/1920 | 40/1920 | 40/1920 | 40/4416 |
| Payload (without CRC) | Bits | 24 | 24 | 24 | 24 |

<End of the change 2>

<Start of the change 3>

A.3.8.1	FDD

<Unchanged parts are skipped>

Table A.3.8.1-8: Fixed Reference Channel for subcarrier spacing 15kHz with with MBMS dedicated cell

| Parameter | PMCH |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Unit | Value |  |  |  |  |  |
| Reference channel |  |  | R.85-1 FDD | R.85-2 FDD | R.85-3 FDD |  |  |
| Channel bandwidth | MHz |  | 3 | 5 | 10 |  |  |
| Allocated resource blocks |  |  | 15 | 25 | 50 |  |  |
| Allocated subframes per Radio Frame(Note1) |  |  | 10 | 10 | 10 |  |  |
| Modulation |  |  | QPSK | 16QAM | 64QAM |  |  |
| Target Coding Rate |  |  | 1/3 | 1/2 | 2/3 |  |  |
| Information Bit Payload (Note 2) |  |  |  |  |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits |  | 1064 | 4968 | 19848 |  |  |
| Number of Code Blocks per Sub-Frame (Note 3) |  |  | 1 | 1 | 4 |  |  |
| Binary Channel Bits Per Subframe |  |  |  |  |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits |  | 3060 | 10200 | 30600 |  |  |
| MBMS UE Category |  |  | ≥ 1 | ≥ 1 | ≥ 2 |  |  |
| Note 1: For FDD mode, all 10 subframes are available for MBMS, in line with TS 36.331 [7]. Note 2: 2 OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211 [4]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |  |  |  |  |

Table A.3.8.1-9: Fixed Reference Channel for subcarrier spacing 0.37 kHz with 5G terrestrial broadcast MBMS dedicated cell

| Parameter | PMCH |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Unit | Value |  |  |  |  |  |
| Reference channel |  |  | R.106-1 FDD | R.106-2 FDD |  |  |  |
| Channel bandwidth | MHz |  | 10 | 10 |  |  |  |
| Allocated resource blocks |  |  | 50 | 50 |  |  |  |
| Allocated slots per 40ms(Note1) |  |  | 13 | 13 |  |  |  |
| Modulation |  |  | 64QAM | 64QAM |  |  |  |
| Target Coding Rate |  |  | 0.48 | 0.52 |  |  |  |
| Information Bit Payload (Note 2) |  |  |  |  |  |  |  |
| For each slot | Bits |  | 63776 | 63776 |  |  |  |
| Number of Code Blocks per Sub-Frame (Note 3) |  |  | 11 | 11 |  |  |  |
| Binary Channel Bits Per Subframe |  |  |  |  |  |  |  |
| For each slot | Bits |  | 133650 | 121500 |  |  |  |
| MBMS UE Category |  |  | [TBD] | [TBD] |  |  |  |
| Note 1: First subframe of every 40ms is allocated for non-MBMS transmission. Note 2: Zero OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211 [4]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |  |  |  |  |

Table A.3.8.1-10: Fixed Reference Channel for subcarrier spacing 2.5 kHz with 5G terrestrial broadcast MBMS dedicated cell

| Parameter | PMCH |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Unit | Value |  |  |  |  |  |
| Reference channel |  |  | R.107 FDD |  |  |  |  |
| Channel bandwidth | MHz |  | 10 |  |  |  |  |
| Allocated resource blocks |  |  | 50 |  |  |  |  |
| Allocated subframes per 40ms(Note1) |  |  | 39 |  |  |  |  |
| Modulation |  |  | 16QAM |  |  |  |  |
| Target Coding Rate |  |  | 0.46 |  |  |  |  |
| Information Bit Payload (Note 2) |  |  |  |  |  |  |  |
| For each subframes | Bits |  | 9912 |  |  |  |  |
| Number of Code Blocks per Sub-Frame (Note 3) |  |  | 2 |  |  |  |  |
| Binary Channel Bits Per Subframe |  |  |  |  |  |  |  |
| For each subframes | Bits |  | 21600 |  |  |  |  |
| MBMS UE Category |  |  | ≥ 2 |  |  |  |  |
| Note 1: First subframe of every 40ms is allocated for non-MBMS transmission. Note 2: Zero OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211 [4]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |  |  |  |  |

<End of the change 3>

<Start of the change 4>

B.2.6.3	Subcarrier spacing 0.37kHz

Table B.2.6.3-1 shows propagation conditions that are used for the MBSFN performance requirements in multi-path fading environment in an extended delay spread environment for subcarrier spacing as 0.37kHz.

Table B.2.6.3-1: Propagation Conditions for Multi-Path Fading Environments for MBSFN Performance Requirements in an extended delay spread environment with subcarrier spacing 0.37kHz

| Extended Delay Spread |  |
| --- | --- |
| Zero Doppler frequency |  |
| Relative Delay [μs] | Relative Mean Power [dB] |
| 0 | -11 |
| 130 | -10 |
| 220 | -4.5 |
| 240 | -3.5 |
| 400 | 0 |
| 520 | -13 |
| 650 | -20 |
| 800 | -25 |

B.2.6.4	Subcarrier spacing 2.5kHz

Table B.2.6.4-1 shows propagation conditions that are used for the MBSFN performance requirements in multi-path fading environment in an extended delay spread environment for subcarrier spacing as 2.5kHz.

Table B.2.6.4-1: Propagation Conditions for Multi-Path Fading Environments for MBSFN Performance Requirements in an extended delay spread environment with subcarrier spacing 2.5kHz

| Extended Delay Spread |  |
| --- | --- |
| Maximum Doppler frequency [162Hz] |  |
| Relative Delay [ns] | Relative Mean Power [dB] |
| -310 | -3.6 |
| -280 | -1.5 |
| -160 | -1.4 |
| 0 | 0 |
| 60 | -0.6 |
| 780 | -7.0 |
| 49650 | -10 |
| 49680 | -11.5 |
| 49800 | -11.4 |
| 49960 | -13.6 |
| 50020 | -10.6 |
| 50740 | -17.0 |
| 109650 | -20 |
| 109680 | -21.5 |
| 109800 | -21.4 |
| 109960 | -23.6 |
| 110020 | -20.6 |
| 110740 | -27.0 |

<End of the change 4>