3GPP TSG-RAN WG4 Meeting #95-e	R4-2009132

# Electronic Meeting, 25 May – 05 June, 2020

Title:	LS on RRM relaxation in UE power saving

Response to:	R2-2005855

Release:	Release 16

Work Item:	NR_UE_pow_sav-Core

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact Person:

Name: 	Xusheng Wei

E-mail Address:	xusheng.wei@vivo.com

1 Overall description

RAN4 thanks RAN2 for the LS R2-2005855 [1]. RAN4 has further discussions on UE power saving and the following conclusions are achieved:

The measurement relaxation method for higher priority or equal/lower priority applies to inter-RAT measurement with higher priority or equal/lower priority.

RAN4 concludes that it is up to RAN2’s decision on whether to introduce carrier specific threshold for inter-frequency measurement relaxation.

Regarding the impact of EMR on UE power saving, RAN4’s conclusion is that measurements on EMR carriers should not be relaxed if T331 is running.

RAN4 concludes that when network configures both low mobility and not-at-cell-edge criteria, if network indicates option b, the relaxation method corresponding to scenario #3 will be used when both relaxation criteria have been fulfilled

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and only criteria of low mobility is configured, if the low mobility criteria is fulfilled and highPriorityMeasRelax is configured and set to True:

UE enters the scenario #3 RRM measurement relaxation (1 hour) for higher priority inter-freq/inter-RAT measurement; while UE is not required to do any lower and equal priority inter-freq/inter-RAT measurement

RAN4 kindly asks RAN2 to take RAN4’s conclusions into account.

2 Actions

To RAN2 group.

ACTION: RAN4 kindly asks RAN2 to take RAN4’s conclusions into account.

3 Dates of next TSG RAN4 meetings

TSG-RAN WG4 Meeting #96-e           August 17 – 28, 2020    			E-meeting

TSG-RAN WG4 Meeting #97-e           Nov.02 –14, 2020    			E-meeting

4 References

- [1] R2-2005855 LS, RAN2 #110-e, June, 2020.