3GPP TSG-RAN WG4 Meeting # 94-e-Bis 												R4-200XXXX

Electronic Meeting, 20 – 30 Apr., 2020

Agenda item:			12.3

Source:	Ericsson

Title:	Email discussion summary for [94e Bis][232]LTE_RRM_Maintenance

Document for:	Information

# Introduction

Maintenance of LTE specifications, focussed on at least addressing TBDs, FFSs and square brackets in release 15 and release 16 core specifications for ITU specification submission.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: Conclude on any open technical issues and identify in principle agreeable CRs which may be agreed or revised in 2nd round

- 2nd round: Review draft revised CRs to get an agreeable set of maintenance CRs at the end of second round which address at least the needs of the ITU specification submission

# Topic #1: euCA requirements finalization

Main technical topic overview. Closure of outstanding R15 euCA technical issues

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2007146 | Nokia, Nokia Shanghai Bell | Proposal 1: UE shall support the idle mode CA measurements of at least 2 inter-frequency carriers of which zero or one inter-frequency carrier may be a non-overlapping carrier and at most 1 inter-frequency carrier can be overlapping. Proposal 2: Clarify in 36.133 that the fields s-NonIntraSearch in SystemInformationBlockType3 do not affect measurements of inter-frequency CA candidate cells. |
| R4-2007147.zip | Nokia, Nokia Shanghai Bell | Cat F Rel 15 CR related to P1 in R4-2007146 |
| R4-2007148 | Nokia, Nokia Shanghai Bell | Cat A Rel 15 CR related to P1 in R4-2007146 |
| R4-2007149.zip | Nokia, Nokia Shanghai Bell | Cat F Rel 15 CR related to P2 in R4-2007146 |
| R4-2007150 | Nokia, Nokia Shanghai Bell | Cat A Rel 15 CR related to P12 in R4-2007146 |

## Open issues summary

- Number of carriers to measure

- Snon-intrasearch

### Sub-topic 1-1 : number of carriers to measure

Sub-topic description: euCA requirements finalization

Open issues and candidate options before e-meeting: (from current 36.133):  A UE which supports ca-IdleModeMeasurements shall support the idle mode CA measurements of at least [1, 2 or 3] inter-frequency carriers of which zero or one inter-frequency carrier may be a non-overlapping carrier.

Issue 1-1: Required number of carriers to measure

- Proposals

  - Option 1: : UE shall support the idle mode CA measurements of at least 2 inter-frequency carriers of which zero or one inter-frequency carrier may be a non-overlapping carrier and at most 1 inter-frequency carrier can be overlapping.

  - Option 2: Other?

- Recommended WF

  - Check if proposal 1/option 1 (from R4-2007146, Nokia) is agreeable and if so, discussion can take place on corresponding CR wording.

### Sub-topic 1-2 : s-NonIntraSearch

Sub-topic description: s-NonIntraSearch clarifications

Open issues and candidate options before e-meeting: 36.133 section 4.9.2.2 refers to requirements in section 4.2.2.4, in which the s-NonIntraSearch may be applied.

Issue 1-2: TBA

- Proposals

  - Option 1: clarify in the RAN4 specification 36.133 section 4.9.2.2, that the s-NonIntraSearch thresholds do not apply to carriers configured for early measurement reporting

  - Option 2: Other?

- Recommended WF

  - Check if proposal 2/option 1 (from R4-2007146, Nokia) is agreeable and if so, discussion can take place on corresponding CR wording.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2007147 | Company A |
|  | Company B |
| R4-2007149 | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #2: Rel-13 maintance

Release 13 essential corrections

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2007424.zip | Rohde & Schwarz | Cat F Rel 13 CR In SchedulingRequest-Config-DEFAULT tables of tests A.7.3.50/51/54/55/58/58 the CR-ConfigIndex changed from 0 (5 ms periodicity) to 5 (20 ms periodicity). |
| R4-2007425 | Rohde & Schwarz | Cat A Rel-14 CR shadow for 7424 |
| R4-2007426 | Rohde & Schwarz | Cat A Rel-15 CR shadow for 7424 |
| R4-2007427 | Rohde & Schwarz | Cat A Rel-16 CR shadow for 7424 |
| R4-2007724.zip | Huawei, Hisilicon | Cat F Rel 13 CR : Add the clarification that “provided that the cell is at least YdB better ranked, where ‘Y’ is specified in Table 4.6.2.6-3” |
| R4-2007725 | Huawei, Hisilicon | Cat A Rel-14 CR shadow for 7724 |
| R4-2007726 | Huawei, Hisilicon | Cat A Rel-15 CR shadow for 7724 |
| R4-2007727 | Huawei, Hisilicon | Cat A Rel-16 CR shadow for 7724 |
| R4-2006082 | Anritsu | Cat F Rel13 CR : Delete patrameter of sr-Configindex in Table A.7.3.55.1-4. |
| R4-2006083 | Anritsu | Cat A rel 14 CR shadow for 6082 |
| R4-2006084 | Anritsu | Cat A rel 15 CR shadow for 6082 |
| R4-2006085 | Anritsu | Cat A rel 16 CR shadow for 6082 |
| R4-2006086 | Anritsu | Cat F rel 13 CRChange a reference to the RMC in the associated test as follows: A.8.1.26 PDSCH parameters in Table A.8.1.26.1-2 R.11 HD-FDD -> R.25 HD-FDD |
| R4-2006087 | Anritsu | Cat A rel 14 CR shadow for 6086 |
| R4-2006088 | Anritsu | Cat A rel 14 CR shadow for 6086 |
| R4-2006089 | Anritsu | Cat A rel 14 CR shadow for 6086 |

## Open issues summary

Discussion of updates in CRs

### Sub-topic 2-1

Sub-topic description: Rel-13 essential corrections

Open issues and candidate options before e-meeting:

Issue 2-1: Please provide comments on CRs directly

- Proposals

  - Option 1: TBA

  - Option 2: TBA

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Please provide comments on CRs directly |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2007424 | Company A |
|  | Company B |
| R4-2007724 | Company A |
|  | Company B |
| R4-2006082 |  |
| R4-2006086 |  |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #3: Rel-14 maintance

Main technical topic overview. Rel-14 essential corrections

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2007876 | Huawei, HiSilicon | Cat F Rel-14 CR : Change the inter-frequency reselection margin for eMTC to 8dB for normal and enhanced coverage for RSRP based, and 5.5dB for RSRQ based. A redundant note is also removed. |
| R4-2007877 | Huawei, HiSilicon | Cat A Rel-15 CR shadow for 7876 |
| R4-2007878 | Huawei, HiSilicon | Cat A Rel-16 CR shadow for 7876 |

## Open issues summary

Discussion of updates in CRs

### Sub-topic 2-1

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-1: Please provide comments on CR directly

- Proposals

  - Option 1: TBA

  - Option 2: TBA

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Rapporteur | Please provide comments on CRs directly |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2007876 | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #4: Rel-15 maintance

Main technical topic overview. Discussion of release 15 clean up, excluding euCA which is discussed under topic 1.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006966 | Ericsson | CAT F Rel 15 CR : All tentative values in square brackets are finalised except for values which are expected to be addressed by other release 15 CRs in RAN4#95 All TBD in specifications are addressed |
| R4-2006968 | Ericsson | Cat F Rel 15 CR / Clause A.8.7A renamed from TBD to E-UTRAN FDD – UTRAN TDD Measurements. Spelling corrections. |
| R4-2006969 | Ericsson | Cat A Rel 16 CR to 6968 |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-1: Please provide comments on CR directly

- Proposals

  - Option 1: TBA

  - Option 2: TBA

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Rapporteur | Please provide comments on CRs directly |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2006966 | Company A |
|  | Company B |
| R4-2006968 | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #5: Rel-16 maintance

Main technical topic overview. Release 16 clean up.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006967 | Ericsson | CAT F Rel 16 CR : All tentative values in square brackets are finalised except for values which are expected to be addressed by other release 15 CRs in RAN4#95 All TBD in specifications are addressed |
| R4-2007118 | Nokia, Nokia Shanghai Bell | Cat F Rel 16 CR : Subclause references are corrected in subclauses 5.2.2.4.1 , 5.2.2.5.1 and 5.3.2.2.1 Typo corrected in subclause 5.5.2.3.2. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-1: Please provide comments on CR directly

- Proposals

  - Option 1: TBA

  - Option 2: TBA

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Rapporteur | Please provide comments on CRs directly |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2006967 | Company A |
|  | Company B |
| R4-2007118 | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |