3GPP TSG-RAN WG4 Meeting #95-e 	DRAFT    R4-200xxxx

Online, 25 May - 5 Jun 2020

Source: 	Huawei

Title: 	WF on IAB-MT class descriptions

Agenda Item:	6.5.1.2

Document for:	Approval

- Background

In the last meeting a WF on IAB system parameters [R4-2005482] was approved along with the following options for IAB class description

Option 1: use target deployment scenario and below parameters for IAB-MT class definition and description:

-	Option 1a: Minimum distance/ Typical distance

-	Option 1b: Minimum distance/Typical distance combined with other parameters including Planed/Unplanned and Backhaul link condition

Option 2: use target deployment scenario for IAB class definition and description

-	Wide area IAB-MT target for Macro and Micro cell deployment

-	Local area IAB-MT target for Micro and Pico Cell deployment

Note it was agreed during the RAN4’#94e-bis that this issue would be deprioritized until the RF requirements for the BS classes were finalised.

During RAN4#95e we had 2 contributions on the issue:

R4-2007399 – which proposed descriptions using minimum distance (option 1a)

R4-2007903 – which proposed descriptions based on the deployment (option 2)

The summary for the 1st round discussion captured the following:

Opinion is still split on the best way forward, although there seems a slight preference for a non-quantative solution based on option 2 (4 companies: Nokia, CATT, Huawei, Samsung). There are 3 companies supporting a quantative, distance based definition approach with 2 favoring min distance (Qualcomm, ZTE) and 1 for typical distance (Ericsson). There is no consensus on what the distance should be

The intention of the WF is to try to further clarify and narrow the options

- Way Forward

The IAB-MT class definition options are:

Option 1: use target deployment scenario and below parameters for IAB-MT class definition and description:

-	 option 1a: Minimum distance

-	 option 1b: Typical distance

Option 2: use target deployment scenario for IAB class definition and description from the following options:

- 	option 2a: (minimal information)

Wide Area IAB-MT nodes are characterised by requirements derived from Macro Cell and Micro Cell scenarios.

Local Area IAB-MT nodes are characterised by requirements derived from Pico Cell and Micro Cell scenarios.

- 	option 2b: (additional link based information)

Wide Area IAB-MT nodes are characterised by requirements derived from Macro Cell and Micro Cell scenarios where the path loss is higher but stable.

Local Area IAB-MT nodes are characterised by requirements derived from Pico Cell and Micro Cell scenarios where the path loss is lower but more variable

Companies are encouraged to provide further views on the IAB-MT description, particularly what distances should be used for option 1 if this is agreed as the agreeable approach.

- References

[1]	R4-2005482	WF on IAB system parameters	Huawei

[2] R4-2007399	Discussion on IAB MT class	ZTE

[3] R4-2007903	IAB-MT Class descriptions	Huawei