3GPP TSG-RAN WG4 Meeting # 95-e 															R4-2008769

Electronic Meeting, 25 May – 5 June, 2020

Agenda Item:	6.5.1.1

Source: 	Ericsson

Title:	TP to TS 38.174: system parameter

Document for:	Approval

- Introduction

Section 2 proivde some reasoning on the text proposal and Section 3 provides text proposal on chapter 5 system parameter for the IAB TS 38.174 (Integrated access and backhaul radio transmission and reception).

# Disucssion

The text proposal is based on the WF [1]. We noticed that for the generic description text, it is not within the requirement itself and need some discussion. For some of the generic description, it is not likely to make a reference as not only BS/UE need to be replaced by IAB-DU/IAB-MT,  but there is a need to modify further on the text to selectively to add IAB-DU/IAB-MT, the example for such case is the chapter 5.3. For some other cases, the text itself is very short and almost the same for UE spec and BS spec, rather than do reference to a versioned spec, it will be more convenient to just provide the whole text to give better readability.  Below lists some of the thinking for each of the subchapter text proposal:

5.1 General: in this chapter, the text of UE spec and BS spec is basicall the same, considering the good readability, it is suggested to copy the text directly into the IAB TS.

5.3.1 channel bandwidth:in general section text, the IAB-MT or IAB-DU replacement need to be carefully examined and further modification is needed thus the referening is not possible.

5.3.2 and 5.3.3: requirement can be referenced.

5.3.4: IAB-DU need some modification on the text while IAB-MT can be referenced.

5.3.5: specific text for the supported band need to be additional stated when refereeing.

5.4.: specific text for the supported band need to be additional stated when refereeing.

Additionally there are comments from companies from first round discussion which is listed below and TP is modified accordingly for suggestions below. Some reply upon the giving comments but still not address all comments and we need continue to discuss.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2004172 | Qualcomm: The text proposal looks generally good to us. Two suggestions: 1) IAB-MT and IAB-DU should be spelled always in the same way, with the dash in the middle. 2) Many sections (e.g. Sec. 5.4.2.3) seem to contain a spell out of FR1 IAB operating bands. We think it would be better to directly refer to the IAB operating bands table in Sec. 5.2. It would avoid additional work in the future. |
|  | Nokia: It seems the change marks are not provided on top of the latest draft of TS 38.174 so it is unclear which content is proposed. The wording “For IAB-DU … specified as the same as BS in TS…” could be improved to “The BS requirements in TS… …apply for IAB-DU.” Similar wording improvement should be done also for IAB-MT references. It seems like some sections could be missing, e.g. now IAB-DU channel bandwidth for CA is included but IAB-MT channel bandwidth for CA is not included. Versioned references for system parameters may be problematic as possible updates to UE/BS specifications would not be automatically reflected for IAB. This TP cannot be approved as it is now. |
|  | Samsung: for channel raster and sync raster simplification can be considered to refer to TS38.104 |
|  | Huawei: 38.104 5.3.2 is referenced but this clause contains NB-IoT requirements – how are these excluded. The references point to text with UE channel bandwidth and BS channel bandwidth etc. This is not clearly explained what terms to use instead, this is consistent throughout the referencing. The reference to the UE spec 38.101-1 sub clause 5.3.3 the referenced text contains the phrase “In the case that multiple numerologies are multiplexed in the same symbol due to BS transmission of SSB” how is the “BS” supposed to be interpreted here? If this type of referencing is to be used then these issues need to be resolved. There are a number of specific references to non-versioned specs. It was agreed to handle the IAB-MT channel BW’s like the BS (i.e. by declaration) but the UE spec is referenced. IN BS spec the table shows BW configuration, in UE its maximum BW configuration. Does this imply a requirement? Maybe not but can we discuss how the agreement to use BE declared BW’s is demonstrated in the specs. |
|  | CATT: Couple of comments as followings, - I have a clarification question why 38.101 version number is explicitly captured in the TP in reference part but other specs are not. Although there was a bullet said “Specific references must be made to versioned documents”, I think we should discuss this aspect further. My understanding is that the latest spec should be referred, i.e. no version number in the reference specs. Otherwise any future correction CRs for these specs should be updated in TS38.714. - Which version is based on for this TP, such as clause 5.3.1 and 5.3.4, it seems it’s copied originally from 38.104? There’re many wording like “ as the same as BS” or “as the same as UE”, my suggestion is removing them. There should be no misunderstanding if we don’t have these wording. |

During the 1st round discussion[2], there are more comments regarding this TP and lists below:

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Sub topic 2-1: We think there is still a need to re-discuss the usage of specific references. For example, we expect that band n259 will be added to BS and UE specifications in this meeting and having specific references to older versions of the specifications will therefore exclude n259. From this perspective, using non-specific references would be practical for spec maintenance. We see a risk that the requirements for access link will differentiate depending on whether the link originates from IAB-DU and regular BS. |
| CATT | We agree with Nokia the spec reference should be re-discussed. If we see other RAN1/RAN2 spec reference, there’s no specific version. We think that should be the correct approach. There will be maintenance CRs for both BS and UE spec in future, if we refer the current version, we don’t know the plan on how to maintain IAB spec. |
| Qualcomm | Sub-topic 2-1: we also agree with Nokia and CATT |
| ZTE | Sub-topic 2-1: agree with Nokia, CATT and QC on its spec reference. Frankly speaking, from ETSI drafting rule perspective, if we want to refer to one specific table or section, then we need to add the corresponding spec version, i think that’s reason why Ericsson add that by intention.. |
| Ericsson | Sub topic 2-1: no versioned reference is ok on the condition that the future change of referenced requirement would be automatically apply to IAB. For FR2 band, it is agreed that all FR2 band will apply to IAB so it may be good just to reference to the 38.104 FR2 band table. If with great carefulness, we follow the previous WF on TS drafting, we would be ok to use no versioned reference. For now, we would like to keep the versioned reference until we see more TP on other RF requirement. |
| Huawei | The rules on specific referencing are quite clear, if you refer to the whole document than it can be non-specific, if you refer to a specific sub-clause or table it must be specific. If you wish to avoid specific referencing then that has to be done when citing the reference. The example given by Nokia about a new band is exactly the problem with specific referencing and why we favor copying the requirements in most cases. In this case the IAB spec specifically lists the IAB bands so a CR is needed to add a new one, it would be necessary to update the specific reference version at this time. The argument works the other way, if the BS spec is modified with a requirement you do not want implemented in IAB and you use general referencing then it gets automatically included if you want it or not. Unfortunately there is no zero maintenance way to handle referencing. For the operating bands it’s probably ok to use a general reference e.g. “The IAB uses the operating bands specified in 38.104” – this is a general reference As opposed to “The IAB uses the operating bands specified in 38.104 sub-clause 5.2” – this is a specific reference But this is harder with eth RF requirements where we have to pick and choose a bit more and hence use the sub-clauses to point to the applicable requirement. |
| Samsung | It is not against to add more FR2 band. But the legacy agreement on FR2 band is in WF R4-1912890 as: - For new FR2 bands introduced in future, RAN4 will discuss the introduction of new FR2 bands into IAB specifications after RF requirements for FR2 bands are completed. If it is updated agreement to support for any new FR2 band introduced in future for IAB, it would be better to capture it somewhere explicitly. |

And also

| CR/TP number | Comments collection |
| --- | --- |
| R4-2007577 | Nokia, Nokia Shanghai Bell: See also more general comments above in section 2.3.1. Regarding the specific language used in referencing, it could be considered to better take into account that IAB-MT and IAB-DU terms do not exist in 38.104 or UE RF requirement specifications. Therefore, a wording that could be considered is “UE requirements in TS 38.101-2… …apply for IAB-MT”. Examples of this can be found in R4-2007121 for comparison, and naturally it is important that in the end all TPs use aligned language. Additionally, it seems there are some change marks which are not based on the latest version of the specification. |
|  | CATT: there’s a clause of CA, should CA be included at the current stage? |
|  | Huawei: agree with Nokia comment about the phrasing of the reference. |

# Text Proposal

--------------------------------------------------Start of TP------------------------------------------------------

## 2	Reference

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.104: “NR; Base Station (BS) radio transmission and reception”, [V16.2.0]

[3]	3GPP TS 38.101-1: “NR User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”, [v 16.2.0]

[4]	3GPP TS 38.101-2: “NR User Equipment (UE) radio transmission and reception: Part 2: Range 2 Standalone”, [V16.2.0]

[5]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios "

[6]	3GPP TS 38.133: “NR: Requirements for support of radio resource management”

[7]	3GPP TS 38.211: "NR; Physical channels and modulation".

[8]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

## 5.1	General

The channel arrangements presented in this clause are based on the operating bands and IAB-DU or IAB-MT channel bandwidths defined in the present release of specifications.

NOTE:	Other operating bands and IAB-DU or IAB-MT channel bandwidths may be considered in future releases.

Requirements throughout the RF specifications are in many cases defined separately for different frequency ranges (FR). The frequency ranges in which NR can operate according to the present version of the specification are identified as described in table 5.1-1.

Table 5.1-1: Definition of frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1 | 410 MHz – 7125 MHz |
| FR2 | 24250 MHz – 52600 MHz |

## 5.2	Operating bands

NR IAB is designed to operate in the operating bands in FR1 defined in table 5.2-1 and operating bands in FR2 defined in 38.104[2].

Table 5.2-1 NR IAB operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex Mode |
| --- | --- | --- | --- |
| n41 | 2469 MHz – 2690 MHz | 2469 MHz – 2690 MHz | TDD |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |

## 5.3	Channel bandwidth

### 5.3.1	General

The IAB-DU channel bandwidth supports a single NR RF carrier in the uplink or downlink at the IAB node. Different UE or IAB-MT channel bandwidths may be supported within the same spectrum for transmitting to and receiving from UEs or IAB-MT connected to the IAB-DU. The placement of the UE or IAB-MT channel bandwidth is flexible but can only be completely within the IAB-DU channel bandwidth. The IAB-DU shall be able to transmit to and/or receive from one or more UE or IAB-MT Bandwidth parts that are smaller than or equal to the number of carrier resource blocks on the RF carrier, in any part of the carrier resource blocks.

The IAB-MT channel bandwidth supports a single NR RF carrier in the uplink or downlink at the IAB-MT. From a BS or IAB-DU perspective, different IAB-MT channel bandwidths may be supported within the same spectrum for transmitting to and receiving from UEs or IAB-MT connected to the IAB-DU. Transmission of multiple carriers to the same IAB-MT (CA) or multiple carriers to different UEs or IAB-MT within the  IAB-DU channel bandwidth can be supported.

From a IAB-MT perspective, the IAB-MT is configured with one or more BWP / carriers, each with its own IAB-MTchannel bandwidth. The IAB-MT does not need to be aware of the BS or IAB-DU channel bandwidth or how the BS or IAB-DU allocates bandwidth to different UEs or IAB-MT.

The placement of the IAB-MT channel bandwidth for each IAB-MT carrier is flexible but can only be completely within the BS or IAB-DU channel bandwidth.

The relationship between the IAB-DU or IAB-MT channel bandwidth, the guardband and the transmission bandwidth configuration is shown in Figure 5.3.1-1.

.

Figure 5.3.1-1: Definition of channel bandwidth and transmission bandwidth configuration for one NR channel

### 5.3.2	Transmission bandwidth configuration

For IAB-DU, the transmission bandwidth configuration is the same as specified for BS in TS 38.104 [2], subclause 5.3.2.

### For IAB-MT, the transmission bandwidth configuration is the same as specified for UE in TS 38.101-1[3] for FR1 in subclause 5.3.2 and in TS 38.101-2 [4] for FR2 in subclause 5.3.2.

### 5.3.3	Minimum guardband and transmission bandwidth configuration

For IAB-DU, the minimum guardband and transmission bandwidth configuration is the same as specified for BS in TS38.104[2], subclause 5.3.3.

For IAB-MT, the minimum guardband and transmission bandwidth configuration is the same as specified for UE in TS38.101-1[3] for FR1 and in TS 38.101-2 [4] for FR2 in subclause 5.3.3.

### 5.3.4	RB alignment

For each IAB-DU channel bandwidth and each numerology, IAB-DU transmission bandwidth configuration must fulfil the minimum guardband requirement specified in clause 5.3.3.

For IAB-DU, for each numerology, its common resource blocks are specified in clause 4.4.4.3 in [7], and the starting point of its transmission bandwidth configuration on the common resource block grid for a given channel bandwidth is indicated by an offset to “Reference point A” in the unit of the numerology.

For IAB-DU, for each numerology, all UE and IAB-MT transmission bandwidth configurations indicated to UEs or IAB-MT served by the IAB-DU by higher layer parameter carrierBandwidth defined in TS 38.331 [8] shall fall within the IAB-DU transmission bandwidth configuration.

For IAB-MT, the RB alignment is the same as specified for UE in TS38.101-1[3] for FR1 in subclause 5.3.4 and in TS 38.101-2 [4] for FR2 in subclause 5.3.3.

### 5.3.5	IAB-DU and IAB-MT channel bandwidth per operating band

For IAB-DU, the channel bandwidth for NR bands for FR1 in Table 5.2.1 and for NR bands for FR2 defined in TS38.104[2] is the same as specified for BS in TS38.104[2], subclause 5.3.5.

For IAB-MT, the channel spacing for NR bands for FR1 in Table 5.2-1 is the same as specified for UE in TS38.101-1[3] in subclause 5.3.5 and for NR bands for FR2 defined in TS38.104[2] is the same as specified for UE in TS38.101-2[4] in subclause 5.3.5.

## 5.3A	IAB-DU channel bandwidth for CA

The IAB-DU channel bandwidth for CA is the same as specified for BS in TS38.104[2], subclause 5.3A.

## 5.4	Channel arrangement

### 5.4.1	Channel spacing

For IAB-DU, the channel spacing is the same as specified for BS in TS38.104[2], subclause 5.4.1.

For IAB-MT, the channel spacing is the same as specified for UE in TS38.101-1[3] for FR1 in subclause 5.4.1 and in TS38.101-2[4] for FR2 in subclause 5.4.1.

### 5.4.2	Channel raster

#### 5.4.2.1	NR-ARFCN and channel raster

For IAB-DU, the NR-ARFCN and channel raster is the same as specified for BS in TS38.104[2], subclause 5.4.2.1.

For IAB-MT, the NR-ARFCN and channel raster is the same as specified for UE in TS38.101-1[3] for FR1 in subclause 5.4.2.1 and in TS38.101-2[4] for FR2 in subclause 5.4.2.1.

#### 5.4.2.2	Channel raster to resource element mapping

For IAB-DU, the Channel raster to resource element mapping is the same as specified for BS in TS38.104[2], subclause 5.4.2.2.

For IAB-MT, the Channel raster to resource element mapping is the same as specified for UE in TS38.101-1[3] for FR1 in subclause 5.4.2.2 and in TS38.101-2[4] for FR2 in subclause 5.4.2.2.

#### 5.4.2.3	Channel raster entries for each operating band

For IAB-DU, the channel raster entries for NR bands for FR1 in Table 5.2-1 and NR bands for FR2 defined in TS38.104[2] are the same as specified for BS in TS38.104[2], subclause 5.4.2.3.

For IAB-MT, the channel raster entries for NR bands for FR1 in Table 5.2-1 are the same as specified for UE in TS38.101-1[3] in subclause 5.4.2.3 and for NR bands for FR2 defined in TS38.104[2] are the same as specified for UE in TS38.101-2[4] in subclause 5.4.2.3.

### 5.4.3	Synchronization raster

#### 5.4.3.1	Synchronization raster and numbering

For IAB-DU, the synchronization raster and numbering are the same as specified for BS in TS38.104[2], subclause 5.4.3.1.

For IAB-MT, the synchronization raster and numbering are the same as specified for UE in TS38.101-1[3] for FR1 in subclause 5.4.3.1 and in TS38.101-2[4] for FR2 in subclause 5.4.3.1.

#### 5.4.3.2	Synchronization raster to synchronization block resource element mapping

For IAB-DU, the synchronization raster to synchronization block resource element mapping is the same as specified for BS in TS38.104[2], subclause 5.4.3.2.

#### For IAB-MT, the synchronization raster to synchronization block resource element mapping is the same as specified for UE in TS38.101-1[3] for FR1 in subclause 5.4.3.2 and in TS38.101-2[4] for FR2 in subclause 5.4.3.2.

#### 5.4.3.3	Synchronization raster entries for each operating band

For IAB-DU, the synchronization raster entries for NR bands for FR1 in Table 5.2-1 and for NR bands for FR2 defined in TS38.104[2] are the same as specified for BS in TS38.104[2], subclause 5.4.3.3.

For IAB-MT, the synchronization raster entries entries for NR bands for FR1 in Table 5.2-1 are the same as specified for UE in TS38.101-1[3] in subclause 5.4.3.3 and for NR bands for FR2 defined in TS38.104[2] are the same as specified for UE in TS38.101-2[4] in subclause 5.4.3.

--------------------------------------------------End of TP------------------------------------------------------

- References

- R4-2002484, WF on TS referencing rules, Nokia

- R4-2008695, Email discussion summary for [95e][306] NR_IAB_General