3GPP TSG-RAN WG4 Meeting # 95-e 	draft R4-2008782

Electronic Meeting, 25 May – 5 June, 2020

Source: 	ZTE

Title:	TP to TR 38.809 on IAB TX IMD

Agenda Item:	6.5.2.1.3

Document for:	Approval

Introduction

The IAB TX IMD requirement has been discussed in [1]. There is one TP to TR is endorsed in RAN4#94-e meeting[2],It is proposed to approve the TP to TR as following.

This is the revision of R4-2007544.

References

R4-2007543 on IAB TX IMD, ZTE

R4-2002497 TP to TR 38.xxx: OTA transmitter intermodulation, Ericsson

--------------Start of text proposal-------------

  - 7.7	Transmitter intermodulation

For conducted transmitter intermodulation, it is agreed to reuse the base station framework for both IAB-DU and IAB-MT.

--------------Start of next text proposal-------------

  - 9.8	OTA transmitter intermodulation

    - 9.8.1 IAB DU intermodulation

For FR1 the IAB node could be co-located if the IAB TDD pattern for transmission and receiving is the same. IAB DU intermodulation requirement shall be reused from BS spec.

Therefore IAB-DU type 1-O will reuse the framework of OTA TX intermodulation requirement defined in TS 38.104 section 9.8 for NR BS type 1-O.

There is no OTA TX intermodulation requirement for IAB-DU type 2-O as there is no TX IMD requirement for BS type 2-O.

    - 9.8.2 IAB MT intermodulation

The co-location scenario of IAB is listed below as:

-co-located with another IAB-MT in same band

This scenario shall be further considered as if the IAB-DU and IAB-MT of one IAB-Node don’t transmit simultaneously, there will exist the scenario that during UL timeslot one is transmitting in the backhaul link and the other is receiving in the access link, and correspondingly during DL timeslot one can be transmitting in the access link and another is receiving in the backhaul link. In this case, the excessive interference will prevent the two IAB nodes to be co-located. Hence co-location scenario can only be considered for the following case:

-co-located as IAB-DU and IAB-MT of one IAB-Node transmit simultaneously in the same band with same TDD pattern as 	the other co-located BS and/or IAB-Node.

- In case of two co-located IAB-Nodes, all IAB-MTs and IAB-Dus need to follow same TDD pattern.

In this case, the co-location scenario is similar to a base station co-located with another base station in same band, hence the intermodulation requirement shall be reused from BS spec.

-co-located with another IAB-DU/BS in another band

With the above co-location scenarios, it is proposed to use the framework of base stationTX intermodulation requirement for IAB MT with certain co-location scenarios. Considering the large coupling loss, there is no TX IMD requirement for IAB-MT type 2-O.

--------------End of text proposal-------------