3GPP TSG-RAN WG4 Meeting #95-e	R4-200xxxx

Electronic Meeting, 25 May – 5 June, 2020

Agenda item:			9.1.1

Source:	Moderator (CAICT)

Title:	Email discussion summary for [95e][326] FS_NR_MIMO_OTA_test

Document for:	Information

Introduction

Based on the agreed WF [1], FR2 MIMO OTA remaining open issues on PSP validation and QoQZ shall be finalized this meeting:

Remaining Open issues：

  - FR2 PSP Validation procedures

    - CE vendors align on the PSP validation procedure before next RAN4 e-meeting, e.g.,

      - consider if Roll/AZ positioner could be used for SP method [R4-2005560]

      - consider different virtual array configurations and NF compensation methods for the PAS estimation step for KS method [R4-2004565]

    - If misalignment still shows next meeting, then better accuracy vs. measurement time will be the KPI to select one procedure to finalize this topic.

  - FR2 QoQZ procedure

    - QoQZ procedure based on the agreed 6 probes location shall be finalized next RAN4 e-meeting

In addition, the maintenance of Rel-15 NR UE Testability TR38.810 (i.e. FR2 test methods for UE RF, RRM and Demod) is also discussed in this email thread.

List of candidate target of discussion for 1st round and 2nd round

- 1st round: make decision on PSP and QoQZ validation procedure.

- 2nd round: finalize the content of TPs based on the decision of 1st round.

Topic #1: FR2 test methods

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006308 | CAICT | TP to TR 38.827 v1.3.0 on RMC correction |
| R4-2006431 | Samsung | Proposal 1: feasible SNR range can be split into two parts: single probe contribution (SNRsingle-probe) and multi-probe contribution (ΔSNRmulti-probe), i.e., SNR3D-MPAC = SNRsingle-probe + ΔSNRmulti-probe Observation 1: the multi-probe SNR contribution (ΔSNRmulti-probe) is affected by two aspects, probe weights at TE side and antenna directivity pattern at UE side. Observation 2: the multi-probe SNR contribution (ΔSNRmulti-probe) is expected in the range of [0, 3.5]dB. Proposal 2: it is encouraged for TE vendors to input an aligned estimated value to ΔSNRmulti-probe among [0, 3.5]dB based on probe weights for CDL-A InO and CDL-C UMi respectively |
| R4-2006740 | Keysight Technologies | Observation 1: Suitable omnidirectional loop and dipole reference antennas with insignificant azimuth gain variations currently do not exist. Observation 2: The current FR1 QoQZ procedure cannot be leveraged for the NR FR2 3D MPAC QoQZ validation procedure. Observation 3: For NR MIMO OTA, only the single-directional EIRP/EIS metrics need to be assessed in the QoQZ procedure Observation 3: The existing NR FR2 QoQZ validation approach is suitable for NR FR2 3D MPAC systems Proposal 1: Leverage the NR FR2 UE RF QoQZ procedure for NR FR2 3D MPAC MIMO OTA systems using just a single measurement probe Proposal 2: Include the draft text below as FR2 QoQZ Validation procedure in [6]. |
| R4-2006741 | Keysight Technologies | Observation 1: The two-step process using the MUSIC algorithm shows PSP of >96% when the OTA PAS is considered the reference. Observation 2: The PSP results using the spatial correlation-based approach are not as good as with the MUSIC based approach. Proposal 1: Adopt the PSP validation approach based on a virtual antenna array configuration with two vertical sectors and one horizontal sector and the MUSIC algorithm. |
| R4-2006742 late contribution | Keysight Technologies | TP to 38.827 to add PSP validation procedure |
| R4-2006743 | Keysight Technologies | Observation 1: The current FR2 3D MPAC system definition has ambiguities in terms of UE positioning and the NR MIMO probes Observation 2: The spherical coordinate system lends itself to a common ambiguity which could be avoided by limiting rotation angles to certain ranges. Observation 3: Whether the UE is rotated so that the test direction w.r.t. to the UE is aligned with the channel model coordinate system z axis or whether the UE is rotated so that the z axis of the UE’s coordinate system is aligned with the fixed test point w.r.t. the channel model coordinate system will yield additional ambiguities. Observation 4: Option 1 introduces the least amount of positioner blockage while Options 2 and 3 can obstruct the path between UE and NR MIMO probes. Proposal 1: Select a single positioner, probe layout configuration, and rotation assumption to avoid ambiguities Proposal 2: Select Option 4 of the FR2 NR MIMO system configuration as the baseline for 3D MPAC systems. Proposal 3: Perform the UE rotations based on the assumption that the test direction w.r.t. to the UE is aligned with the channel model coordinate z axis Proposal 4: Limit the turntable rotation to a range of 0 to 180o only. |
| R4-2007085 | OPPO | Proposal: to realize the placement of FR2 uplink communication path in MPAC, two approaches could be considered: - Add an independent antenna near the 6 downlink probes - Pick up the uplink signals from one or several of the 6 probes Consequently, in order to guarantee the FR2 3D-MPAC system working effectively, the following aspects is FFS. - The dynamic range of uplink signal - The isolation between downlink and uplink |
| R4-2007592 | Spirent Communications | Observation 1. PSP validation most likely need some phase taper correction technique. Observation 2. If the number of measurement time is limited, number of virtual elements will also become limiter mandating to use some super-resolution technique to estimate DoA. Proposal 1. Apply either of the PSP validations proposed in [1],[2], using super-resolution technique to estimate DoA. |
| R4-20007594 | Spirent Communications | TP on Verification of Channel Model implementation in TR38.827, PSP |
| R4-2007658 | ROHDE & SCHWARZ | Observation 1: Current probe layout centered around 90º elevation and close to 0º azimuth will affect the DUT performance. Observation 2: Simulations to evaluate the PSP performance implement a rotation of the DUT so the original absolute impinging angles of each channel model are respected. Proposal: Channel model rotations shall be specified in TR 38.827 and FR2 test points shall be rotated to compensate them on a per channel model basis. |
| R4-2008273 Reserved | Keysight Technologies | TP to TR38.827 to avoid ambiguities for FR2 MIMO OTA Testing |

Open issues summary

Sub-topic 1-1 FR2 PSP validation procedure

Issue 1-1-1: FR2 6 probes location

- Proposals

    - Proposal 1. Apply either of the PSP validations proposed in [1],[2], using super-resolution technique to estimate DoA.

    - Proposal 2: Adopt the PSP validation approach based on a virtual antenna array configuration with two vertical sectors and one horizontal sector and the MUSIC algorithm

Related TPs: R4-20007594 and R4-2006742

- Recommended WF

  - Make decision on PSP validation procedure and text proposal

Sub-topic 1-2 FR2 QoQZ procedure

Issue 1-2-1: PSP Validation procedures

- Proposals

  - Proposal 1: Leverage the NR FR2 UE RF QoQZ procedure for NR FR2 3D MPAC MIMO OTA systems using just a single measurement probe.

  - Proposal 2: Include the draft text below as FR2 QoQZ Validation procedure in [6].

- Recommended WF

  - Make decision on QoQZ procedure and corresponding text proposal.

Sub-topic 1-3 other FR2 system issues

Issue 1-3-1: 3D-MPAC SNR analysis

- Proposals

  - Proposal 1: feasible SNR range can be split into two parts: single probe contribution (SNRsingle-probe) and multi-probe contribution (ΔSNRmulti-probe), i.e., SNR3D-MPAC = SNRsingle-probe + ΔSNRmulti-probe.

  - Proposal 2: it is encouraged for TE vendors to input an aligned estimated value to ΔSNRmulti-probe among [0, 3.5]dB based on probe weights for CDL-A InO and CDL-C UMi respectively.

- Recommended WF

  - TBA

Issue 1-3-2: UE orientations in 3D-MPAC system

- Proposals

  - Proposal 1: Select a single positioner, probe layout configuration, and rotation assumption to avoid ambiguities

  - Proposal 2: Select Option 4 of the FR2 NR MIMO system configuration as the baseline for 3D MPAC systems.

  - Proposal 3: Perform the UE rotations based on the assumption that the test direction w.r.t. to the UE is aligned with the channel model coordinate z axis

  - Proposal 4: Limit the turntable rotation to a range of 0 to 180o only.

  - Proposal 5: Channel model rotations shall be specified in TR 38.827 and FR2 test points shall be rotated to compensate them on a per channel model basis.

- Recommended WF

  - TBA

Issue 1-3-3: FR2 Uplink path

  - Proposal: to realize the placement of FR2 uplink communication path in MPAC, two approaches could be considered:

    - Add an independent antenna near the 6 downlink probes

    - Pick up the uplink signals from one or several of the 6 probes

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: Sub topic 1-3: - Issue 1-3-1: - Issue 1-3-2: - Issue 1-3-3: |

CRs/TPs comments collection

NR MIMO OTA is a close-to-finalize SI, suggest to focus on finalizing the text proposals for TR.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2006308 | Company A: |
| R4-2006740 |  |
| R4-2006742 |  |
| R4-20007594 |  |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |

Discussion on 2nd round (if applicable)

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Topic #2: Others (including Rel-15 TR38.810 maintenance)

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006310 | CAICT | The latest FR1 MIMO OTA measurement results based on the agreed test system in TR38.827 is presented. Some observations on UE performance are also discussed. This selected approach is able to differentiate 4x4 MIMO OTA performance and more work for higher MCS is expected in the future. |
| R4-2007084 | OPPO | Proposal 1: UE movement with certain speed should be a typical dynamic testing scenario to evaluate the period of new beam re-obtaining and refining as one test case of dynamic testing. Technically, with the help of coordinate system conversion, the scenario of UE movement with certain speed could be re-structured in MPAC with UE rotation on the turn table, accompanied with downlink power change to simulate the BS-UE distance variation. Furthermore, the performance of new beam re-obtaining and refining should be evaluated under different UE movement speed. Obviously, these test scenarios can not be covered by current FR2 MIMO OTA 3D-MPAC static testing. Observation 1: UE movement based scenario can not be covered by current FR2 MIMO OTA 3D-MPAC static testing. Proposal 2: UE movement based scenario should be re-structured in MPAC with rotation and downlink power highly aligned to simulate movement speed. The scenario is recommended to be studied and implemented in a separate new SI. |
| R4-2007285 | Qualcomm Incorporated | Proposal 1: For MIMO OTA sensitivity requirements, RAN4 to agree the introduction of MMO-OTA T-put measurement during at least BEAM_SELECT_WAIT_TIME based on agreed test conditions, e.g. Probe-layout, Channel model, etc. and to discuss performance metric to assess UE performances in WI phase. And during BEAM_SELECT_WAIT_TIME, DL transmission power is constant and its level is FFS. |
| R4-2007563 | Qualcomm Incorporated | TP to TR38.827 on FR2 test procedure Proposal: Include the following text proposal on FR2 test procedure in TR38.827. |
| R4-2008014 (TR38.810 CR) | Keysight Technologies | Beam correspondence – SRS configuration corrections in section 5.2.1.3.7 Clarifed the ‘usage’ as ‘beamManagement’ for the up to 8 SRS resources. Added the creation of an additional SRS resource set of type ‘semi-persistent’ and ‘usage’ set to ‘codebook’. Clarified how the spatial relationship is set during semi-persistent SRS activation. |

Open issues summary

Sub-topic 2-1 FR1 MIMO OTA measurement results

- Discussion

  - The latest FR1 MIMO OTA measurement results based on the agreed test system in TR38.827 is presented.

- Recommended WF

  - TBA

Sub-topic 2-2 FR2 dynamic testing

- Proposals

  - Proposal 1: UE movement with certain speed should be a typical dynamic testing scenario to evaluate the period of new beam re-obtaining and refining as one test case of dynamic testing.

  - Proposal 2: UE movement based scenario should be re-structured in MPAC with rotation and downlink power highly aligned to simulate movement speed. The scenario is recommended to be studied and implemented in a separate new SI.

  - Proposal 3: For MIMO OTA sensitivity requirements, RAN4 to agree the introduction of MMO-OTA T-put measurement during at least BEAM_SELECT_WAIT_TIME based on agreed test conditions, e.g. Probe-layout, Channel model, etc. and to discuss performance metric to assess UE performances in WI phase. And during BEAM_SELECT_WAIT_TIME, DL transmission power is constant and its level is FFS.

Related TP: R4-2007563

- Recommended WF

  - TBA

Sub-topic 2-3 Rel-15 TR38.810 maintanence in AI 4.13

- CR R4-2008014

  - Beam correspondence – SRS configuration corrections in section 5.2.1.3.7.

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 2-1: Sub topic 2-2: Sub topic 2-3: |

CRs/TPs comments collection

NR MIMO OTA is a close-to-finalize SI, suggest to focus on finalizing the text proposals for TR.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2007563 |  |
| R4-2008014 |  |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |

Discussion on 2nd round (if applicable)

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Refrenece

- R4-2005556, WF on FR2 MIMO OTA, CAICT, Keysight, Spirent, 3GPP TSG-RAN WG4 Meeting #94-e-bis, April 2020