3GPP RAN WG4 Meeting #95-e		R4-2008289

Electronic meeting, 25th May – 5th June 2020	(revision of R4-2006338)

Agenda item:	8.18.2

Source:	Apple Inc, Comcast, Nokia, Nokia Shanghai Bell

Title:	LTE/NR spectrum sharing in band 48/n48 frequency range (sync pattern)

WI/SI:	NR_n48_LTE_48_coex-Core

Release:	Rel-16

Document for:	Decision

# 1	Introduction

Dynamic spectrum sharing is an important feature that allows for sharing existing spectrum between the LTE and NR carriers, thus enabling smoother transition from LTE and faster adoption of NR. Amongst a number of prerequisites for running LTE and NR within the same frequency band, either FDD or TDD, an operator has to ensure that the NR synchronization blocks do not overlap with LTE reference signals. As an example, the following WI was agreed to enable dynamic spectrum sharing in band 41/n41 frequency range [1].

After the RAN#86 meeting, a new WI was agreed [2] aiming to analyse and introduce, if needed, changes to support dynamic spectrum sharing in band 48/n48 frequency range. During the RAN4#94bis-e meeting, a number of companies expressed their view on the required modifications [3]-[9], but no consensus was reached.

From the agreed WI perspective, the following issues have to be addressed: channel raster, UL shift, synchronisation pattern. In this discussion paper we elaborate further on the sync pattern and required changes to avoid collision between the LTE CRS and NR SSB.

.

# 2	Sync pattern for spectrum sharing in band 48/n48

## 2.1	Background

As discussed during the previous RAN4 meetings, there can be overlaps between the LTE CRS and NR SSB transmissions. As can be seen from the Figure 2.1-1 below, existing band n48 synchronization pattern C still can work for 1 and 2 port LTE transmission because the corresponding NR SSB can be transmitted in LTE OFDM symbols #1-2 and #8-9. However, if LTE uses 4 antenna port transmission, then all the sync pattern C NR SSB transmission opportunities overlap with the LTE CRS symbols. One solution to this problem is to introduce sync pattern B for band n48. Referring to the same Figure 2.1-1, the sync pattern B NR SSB can be transmitted in LTE OFDM symbols #2-3 even if the 4-port LTE transmission is used.

Observation 1a:	NR sync pattern C can work with 1-2 port LTE deployments, but 4-port LTE CRS transmission will always collide with NR SSB.

Figure 2.1-1: LTE CRS and NR SSB transmissions for 1ms sub-frame.

It also worth noting that 4-port LTE transmission is a widely deployed feature, which is also commonly supported by different UE vendors. Figure 2.1-2 below shows MIMO activation statistics gathered from three major US carriers for the LTE 4x4 MIMO capable devices. The statistics is based on total number of 2x2 and 4x4 MIMO activations for the period of one month, which was collected from 4x4 MIMO capable UEs and divided by the number of UEs, i.e. it can be construed as average number of LTE MIMO activations per UE. As can be seen from the picture, in all the major networks LTE 4x4 MIMO is being aggressively activated (and reasons for only 2x2 MIMO activation are multi-fold, e.g. coverage, site support, etc). It is also worth noting that since every operator has a different number of UEs, deployments, and MIMO activation strategies, the figure naturally shows different average number of MIMO activations per a UE. Nevertheless, the relative ratio between 2x2 and 4x4 MIMO is very close across all the operators showing importance of the 4-port LTE transmission modes.

Observation 1b:	4-port LTE transmission modes are widely supported by UEs and used in commercial deployments.

Figure 2.1-2: Average number of LTE MIMO activations.

Furthermore, since band n48 spectrum is managed dynamically by the SAS entity, an operator cannot be sure that he will have a large channel bandwidth; and in certain cases, the allocated spectrum can be rather small. In this case 4-port LTE transmission becomes quite a crucial feature allowing for better spectrum utilisation of scarce radio resources.

Observation 1c:	Since the band n48 spectrum is managed by the SAS entity and allocated channels can be rather small, the 4-port LTE transmission feature becomes even more important allowing to achieve better spectrum utilisation.

## 2.2	Potential solutions

One of the potential solutions to avoid LTE CRS and NR SSB overlaps is to leverage LTE MBSFN sub-frames so that LTE CRS can be muted in those sub-frames where NR SSB is expected. However, the problem is that LTE MBSFN configuration allows to mute only specific sub-frames, which are not the ones where NR SSB can be sent. Figure 2.2-1 below presents TDD configurations #1 and #2 considered by the CBRS alliance for band 48/n48. The LTE MBSFN can be enabled for sub-frames #3-4 and #7-8; but the NR SSB can be transmitted only in the first symbols of the half-sub-frame.

Figure 2.2-1: LTE MBSFN and NR SSB transmissions for 10ms frame.

Observation 2a:	NR sync pattern B can work with 4-port LTE deployments.

Observation 2b:	Since candidate LTE MBSFN sub-frames do not overlap with OFDM symbols where NR SSB is transmitted, LTE MBSFN cannot be considered as a viable solution to avoid overlaps (unless some further changes are introduced impacting other WGs).

Based on the presented considerations, the most flexible solution would be to consider sync pattern B for band n48 so that 4-port LTE deployments can also be used. However, as discussed earlier in RAN WG4, there are several concerns with that approach which mostly relate to the fact that band n48 overlaps with other bands, e.g. band n77 and band n78, as can be seen from Figure 2.2-2. It should be noted that there is also C-band, for implementation of which band n77 was chosen. However, since the actual C-band does not overlap with band n48, there should be no concern with it.

Observation 3a:	C-band does not overlap with band n48, even if it is implemented by band n77.

Observation 3b:	The main concern is band n77/n78 overlapping with band n48.

Firstly, even though band n77 and n78 overlap with band n48, this happens only for 150MHz in 3550-3700MHz frequency range. So, potential performance impact will be limited and will not concern the whole band n77/n78. Furthermore, band n48 and n77/78 are deployed in different geographical areas. It means that a UE knows from its SIM subscription and geographical location which patterns it should be probe while scanning for a particular range. As an example, a UE with EU subscription located in EU will not probe for pattern B while scanning 3550-3700MHz range. Similarly, a UE with US subscription in US will probe for both patterns B and C while scanning the 3550-3700MHz frequency range. All these optimisations are already used by UEs, which instead of trying blindly different RATs and frequencies, use the subscription information to decide which RAT should be probed first and for which frequencies. In the worst-case scenario, a UE might unnecessarily probe for both patterns B and C, which will happen only in the overlapping range of 3550-3700MHz, outside of which only pattern C can be used. Nevertheless, our understanding is that it will not cause any serious problem. Having different sync patterns in the same frequency range might increase initial search time and cell selection process during the cold start, but once a UE camps on the operator serving cell, it will follow cell re-selection process and frequencies indicated by a particular operator. In the worst-case scenario, even if a UE fails to camp on e.g. band n48 or n77 because it does not find the synchronization signal, it will search for other bands and RATs.

Another concern expressed during previous meetings was increased power consumption. However, overall power consumption while searching for SSB is rather marginal when compared to the UE power consumption when it exchanges data with the network in the CONNECTED state. It becomes an issue only when a UE constantly and continuously searches for available networks potentially probing not only different NR bands, but also different RATs. In this case however adding sync pattern B to band n48 will have only marginal effect.

Figure 2.2-2: 3GPP NR bands n48, n77, n78, and the C-band.

Proposal:	Adopt synchronisation pattern B for band n48 definition.

# 3	Conclusions

In this discussion paper we have presented our further views on the problem of the synchronisation raster for the dynamic spectrum sharing between LTE and NR on band n48. As presented in the paper, 4-port LTE transmission is a widely deployed feature common supported by UEs, which becomes even more important for band n48 where the SAS entity might allocate rather small channels. And since all the NR SSB instances of the synchronisation pattern C collide with the 4-port LTE CRS instances, addition of the synchronisation pattern B can solve the problem.

Proposal:	Adopt synchronisation pattern B for band n48 definition.

# 4	References

RP-182883, "New WI proposal: LTE/NR spectrum sharing in Band 41/n41", KDDI Corporation

RP-192427, "New WID: LTE/NR spectrum sharing in band 48/n48 frequency range", Apple Inc.

R4-2003175, "DSS in LTE/NR band 48/n48", Samsung

R4-2003212, "On LTE/NR dynamic spectrum sharing for band 48/n48", Intel Corporation

R4-2003336, "LTE/NR spectrum sharing in band 48/n48", Ericsson

R4-2005035, "Study of DSS in band 48/n48 channel raster", CableLabs

R4-2004396, "Views on dynamic spectrum sharing between LTE band 48 and NR band n48", Google Inc.

R4-2004507, "Views on band 48/n48 spectrum sharing", Nokia, Nokia Shanghai Bell

R4-2004686, "LTE/NR spectrum sharing in band 48/n48 frequency range", Apple Inc.

Annex A (excerpt from TS 38.101-1)

Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n2 | 1850 MHz – 1910 MHz | 1930 MHz – 1990 MHz | FDD |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n5 | 824 MHz – 849 MHz | 869 MHz – 894 MHz | FDD |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n8 | 880 MHz – 915 MHz | 925 MHz – 960 MHz | FDD |
| n12 | 699 MHz – 716 MHz | 729 MHz – 746 MHz | FDD |
| n14 | 788 MHz – 798 MHz | 758 MHz – 768 MHz | FDD |
| n18 | 815 MHz – 830 MHz | 860 MHz – 875 MHz | FDD |
| n20 | 832 MHz – 862 MHz | 791 MHz – 821 MHz | FDD |
| n25 | 1850 MHz – 1915 MHz | 1930 MHz – 1995 MHz | FDD |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n303 | 2305 Mhz – 2315 MHz | 2350 MHz – 2360 MHz | FDD |
| n34 | 2010 MHz – 2025 MHz | 2010 MHz – 2025 MHz | TDD |
| n38 | 2570 MHz – 2620 MHz | 2570 MHz – 2620 MHz | TDD |
| n39 | 1880 MHz – 1920 MHz | 1880 MHz – 1920 MHz | TDD |
| n40 | 2300 MHz – 2400 MHz | 2300 MHz – 2400 MHz | TDD |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |
| n48 | 3550 MHz – 3700 MHz | 3550 MHz – 3700 MHz | TDD |
| n50 | 1432 MHz – 1517 MHz | 1432 MHz – 1517 MHz | TDD1 |
| n51 | 1427 MHz – 1432 MHz | 1427 MHz – 1432 MHz | TDD |
| n65 | 1920 MHz – 2010 MHz | 2110 MHz – 2200 MHz | FDD4 |
| n66 | 1710 MHz – 1780 MHz | 2110 MHz – 2200 MHz | FDD |
| n70 | 1695 MHz – 1710 MHz | 1995 MHz – 2020 MHz | FDD |
| n71 | 663 MHz – 698 MHz | 617 MHz – 652 MHz | FDD |
| n74 | 1427 MHz – 1470 MHz | 1475 MHz – 1518 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL |
| n76 | N/A | 1427 MHz – 1432 MHz | SDL |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |
| n80 | 1710 MHz – 1785 MHz | N/A | SUL |
| n81 | 880 MHz – 915 MHz | N/A | SUL |
| n82 | 832 MHz – 862 MHz | N/A | SUL |
| n83 | 703 MHz – 748 MHz | N/A | SUL |
| n84 | 1920 MHz – 1980 MHz | N/A | SUL |
| n86 | 1710 MHz – 1780 MHz | N/A | SUL |
| n90 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD5 |
| NOTE 1: UE that complies with the NR Band n50 minimum requirements in this specification shall also comply with the NR Band n51 minimum requirements. NOTE 2: UE that complies with the NR Band n75 minimum requirements in this specification shall also comply with the NR Band n76 minimum requirements. NOTE 3: Uplink transmission is not allowed at this band for UE with external vehicle-mounted antennas. NOTE 4: A UE that complies with the NR Band n65 minimum requirements in this specification shall also comply with the NR Band n1 minimum requirements. NOTE 5: Unless otherwise stated, the applicability of requirements for Band [n90] is in accordance with that for Band n41; a UE supporting Band [n90] shall meet the requirements for Band n41. |  |  |  |