3GPP TSG-RAN WG4 Meeting # 95-e 	                                                           R4-200xxxx

Electronic Meeting, 25 May – 5 June, 2020

Agenda item:			5.11.3

Source:	Moderator (Huawei)

Title:	Email discussion summary for [95e][230] NB_IOTenh3_RRM

Document for:	Information

Introduction

The remaining issues for Rel-16 NB-IoT RRM include:

- Filtering and combination of samples between anchor carrier and non-anchor carriers

- Update RRM requirements for new introduced UE specific DRX cycles (320ms and 640ms)

- Requirements for RSRP change based TA validation for PUR.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round:

- Discuss the remaining issues above

- Discuss and collect comments for the submitted CRs/draft CRs.

- 2nd round:

- Discuss the remaining issues based on the 1st round discussion

- Discuss the revised CRs/draft CRs if any

Topic #1: RRM core requirements

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006166 | Qualcomm Incorporated | Observation 1. In the proposal of WF, it is not clear whether SINR to be used for qualifying a measurement for combining and filtering is on the anchor carrier or non-anchor carrier. Observation 2. SINR ≥ -6 dB is the typical side condition in LTE operation with lower MCL and leads to accurate NRSRP measurement results. Observation 3. Typical UE implementation makes one measurement per DRX cycle. Observation 4. UE behavior in case condition on difference of anchor and non-anchor carrier measurement is exceeded is not clearly defined. Observation 5. The value of M cannot ever be smaller than 12 dB since the NRSRP estimation accuracy in normal coverage is +/- 6 dB. Proposal 1. RAN4 to not pursue imposing a condition on when UE can combine/filter measurements of anchor and non-anchor carriers. |
| R4-2006167 | Qualcomm Incorporated | Proposal 1. Use Nserv=2 for normal coverage and Nserv=4 for enhanced coverage for shorter DRX cycles. Proposal 2. Keep the same requirements for WUS for shorter DRX cycles as in DRX cycles ≤ 5.12s. Proposal 3. For intra-frequency and inter-frequency neighbor cell measurement when not configured with eDRX: - 40 DRX cycles for 320/640ms DRX to detect the cell, 1 DRX cycle to measure, and 5 DRX cycles to evaluate in normal coverage - 415 DRX cycles for 320/640 DRX to detect the cell, 1 DRX cycle to measure, and 10 DRX cycles to evaluate in enhanced coverage when -15 ≤ Q2 < -6 - 45 DRX cycles for 320/640 DRX to detect the cell, 1 DRX cycle to measure, and 10 DRX cycles to evaluate in enhanced coverage when Q2 ≥ -6. Proposal 4. For eDRX cycles, the same existing requirements should apply for Tdetect with 1 DRX cycle for measurement and 2 DRX cycles for evaluation for both intra-frequency and inter-frequency neighbour cells. Proposal 5. Use the same requirements as in DRX cycle of 1.28s for shorter DRX cycles of 320/640ms when eDRX is and is not configured. |
| R4-2007113 | Nokia, Nokia Shanghai Bell | Proposal 1: For UE in normal coverage, NRSRP measurements on non-anchor carrier can be filtered or combined with NRSRP measurement on anchor carrier after translating the non-anchor carrier measurement with parameter nrs-PowerOffsetNonAnchor, provided that the comparison of NRSRP samples between anchor and non-anchor carrier, taken during the same measurement period and accounting for the signal power offset, yields a difference within a margin M. - The margin M and the periodicity of NRSRP comparison P are signaled from network to UE. |
| R4-2007690 | Huawei, HiSilicon | Proposal 1: There is no prerequisite or condition for UE in enhanced coverage when filtering the measurement between anchor and non-anchor carriers. Proposal 2: The threshold for comparison between anchor carrier and non-anchor carrier in normal coverage is 18 dB, which is a predefined value in TS 36.133. Proposal 3: The measurement results for comparison shall be from anchor and non-anchor carrier within the same DRX cycles, which means in same DRX cycles, UE shall perform measurement on both anchor carrier and non-anchor carrier within in the same DRX cycles. Proposal 4: UE shall perform measurement on anchor carrier at least every M DRX cycles, where M is FFS. Observation 1: The decision (i.e. disabling the NRSRP measurement on non-anchor carrier) shall not be made only based on one single-shot comparison between samples from anchor and non-anchor carriers. Proposal 5: Only after K consecutive times that the measurement on non-anchor carrier is regarded as unavailable that UE shall only perform measurement on anchor carrier for a certain period of time. |
| R4-2007691 | Huawei, HiSilicon | Proposal 1: When eDRX is not configured: • Normal coverage: Nserv = 4 for DRX = 320ms and 640ms • Enhanced coverage: Nserv = 8 for DRX = 320ms and 640ms When eDRX is configured: • Normal coverage: Nserv = 2 for DRX = 320ms and 640ms • Enhanced coverage: Nserv = 4 for DRX = 320ms and 640ms Proposal 2: When eDRX is not configured, the relaxation factor shall be updated for 320ms and 640ms DRX cycles as: • Min(n , 16) for 640ms; • Min(n , 32) for 320ms If eDRX is configured, for normal coverage, the relaxation factor shall be: DRX cycle length [s]; Value. 2.56 ≤ PTW length [s] < 5.12; 5.12 ≤ PTW length [s] < 7.68; 7.68 ≤ PTW length [s] < 12.8; 12.8 ≤ PTW length [s] < 23.04; 23.04 ≤ PTW length [s]. 0.32; Min(n , 2); Min(n , 4); Min(n , 8); Min(n , 16); Min(n , 32). 0.64; 1; Min(n , 2); Min(n , 4); Min(n , 8); Min(n , 16). For enhanced coverage, the relaxation factor shall be: DRX cycle length [s]; Value. 2.56 ≤ PTW length [s] < 5.12; 5.12≤ PTW length [s] < 7.68; 7.68 ≤ PTW length [s] < 12.8; 12.8 ≤ PTW length [s] < 23.04; 23.04 ≤ PTW length [s] < 43.52; 43.52 ≤ PTW length [s]. 0.32; 1; Min(n , 2); Min(n , 4); Min(n , 8); Min(n , 16); Min(n , 32). 0.64; N/A; 1; Min(n , 2); Min(n , 4); Min(n , 8); Min(n , 16). Proposal 3: The Tdetect for measurement of neighbor cell s without eDRX, the number of DRX is calculated based on the 2 instances (NPSS or NSSS) per DRX cycle for 320 ms and 5 instances (NPSS or NSSS) per DRX cycle 640 ms respectively. Proposal 4: For measurement period (Tmeasure) without eDRX, 2 DRX cycles for 640ms and 4 DRX cycles for 320ms are defined. Proposal 5: For evaluation period without eDRX, it is suggested to define Tevaluate as 4*Tmeasure for NC and 8*Tmeasure for EC. Proposal 6: For measurement of neighbor cell with eDRX, the existing requirement for Tdectect shall apply; Tmeasure is 1 DRX for 320ms and 640ms DRX; Tevalaute is 2 DRX cycles for NC and same requirements as that without eDRX for EC. Proposal 7: The existing requirements (DRX cycles length ≤ 5.12s) apply for new introduced DRX cycles (320ms and 640ms) |

Open issues summary

Sub-topic 1-1 PUR

There is one CR for this topic (R4-2007688). Please provide comments in 1.3.2	CRs/TPs comments collection

Sub-topic 1-2 Filtering and combination of NRSRP measurement between anchor carrier and non-anchor carrier

Issue 1-2-1: Condition on filtering and combination of NRSRP measurement between anchor carrier and non-anchor carrier in enhanced coverage.

- Proposals

  - Option 1: There is no prerequisite or condition for UE in enhanced coverage when filtering the measurement between anchor and non-anchor carriers. (R4-2007690, Huawei)

- Recommended WF

  - Agree on option 1.

Issue 1-2-2: Whether to introduce conditions on filtering and combination of NRSRP measurement between anchor carrier and non-anchor carrier in normal coverage.

- Proposals

  - Option 1: No. RAN4 to not pursue imposing a condition on when UE can combine/filter measurements of anchor and non-anchor carriers. (R4-2006166, Qualcomm)

  - Option 2: Yes. (R4-2007113, Nokia; R4-2007690, Huawei)

- Recommended WF

  - Need more discussion.

Issue 1-2-3: The SINR for qualifying a measurement for combining and filtering is on the anchor carrier or non-anchor carrier.

- Proposals

  - Option 1: SINR ≥ -6 dB on the non-anchor carrier. (R4-2006166, Qualcomm)

  - Option 2: SINR ≥ -6 dB on the anchor carrier. (R4-2006166, Qualcomm)

- Recommended WF

  - Need more discussion.

Issue 1-2-4: The value of the margin M dB and the method for definition.

- Proposals

  - Option 1: 8 dB, signalled from Network to UE. (R4-2007113, Nokia)

  - Option 2: 18 dB, predefined in TS 36.133 (R4-2007690, Huawei)

  - Option 3: Larger than12 dB (R4-2006166, Qualcomm)

- Recommended WF

  - Need more discussion.

Issue 1-2-5: The periodicity of NRSRP comparison.

- Proposals

  - Option 1: Signalled from Network to UE. (R4-2007113, Nokia)

  - Option 2: UE shall perform measurement on anchor carrier at least every M DRX cycles, where M is FFS. (R4-2007690, Huawei)

  - Option 3: It is left to UE implementation (Qualcomm)

- Recommended WF

  - Need more discussion.

Issue 1-2-6: UE behaviour on comparison of NRSRP measurements of anchor and non-anchor carriers

- Proposals

  - Option 1: Use at least 3 samples from each carrier for the comparison. After the positive comparison, the UE is allowed to stay on the non-anchor carrier until the next comparison takes place. (R4-2007113, Nokia)

  - Option 2: If the difference is larger than the threshold, the measurement on non-anchor carrier is regarded as unavailable, and in the next DRX cycle, UE shall also perform measurement on anchor and non-anchor carrier for more reliable comparison. Only after K consecutive times that the measurement on non-anchor carrier is regarded as unavailable that UE shall only perform measurement on anchor carrier for a certain period of time. (R4-2007690, Huawei)

- Recommended WF

  - Need more discussion.

Sub-topic 1-3 Updating RRM requirements for UE specific DRX cycles (320ms and 640ms)

Issue 1-3-1: Nserv for measurement of serving cell

- Proposals

  - Option 1: Nserv = 2 for normal coverage and Nserv = 4 for enhanced coverage. (R4-2006167, Qualcomm)

  - Option 2: (R4-2007691, Huawei)

When eDRX is not configured:

•	Normal coverage: Nserv = 4 for DRX = 320ms and 640ms

•	Enhanced coverage: Nserv = 8 for DRX = 320ms and 640ms

When eDRX is configured:

•	Normal coverage: Nserv = 2 for DRX = 320ms and 640ms

•	Enhanced coverage: Nserv = 4 for DRX = 320ms and 640ms.

- Recommended WF

  - Need more discussion when eDRX is not configured.

Issue 1-3-2: WUS reception (clause 4.6.2.9)

- Proposals

  - Option 1: Same requirements for WUS for shorter DRX cycles as in DRX cycles ≤ 5.12s. (R4-2006167, Qualcomm; R4-2007691, Huawei)

- Recommended WF

  - Agree on proposal 1.

Issue 1-3-3: Intra-frequency and inter-frequency neighbour cell measurement without eDRX:

- Proposals

  - Option 1: (R4-2006167, Qualcomm)

- 40 DRX cycles for 320/640ms DRX to detect the cell, 1 DRX cycle to measure, and 5 DRX cycles to evaluate in normal coverage

- 415 DRX cycles for 320/640 DRX to detect the cell, 1 DRX cycle to measure, and 10 DRX cycles to evaluate in enhanced coverage when -15 ≤ Q2 < -6

- 45 DRX cycles for 320/640 DRX to detect the cell, 1 DRX cycle to measure, and 10 DRX cycles to evaluate in enhanced coverage when Q2 ≥ -6.

  - Option 2: (R4-2007691, Huawei)

- The Tdetect for measurement of neighbor cell s without eDRX, the number of DRX is calculated based on the 2 instances (NPSS or NSSS) per DRX cycle for 320 ms and 5 instances (NPSS or NSSS) per DRX cycle 640 ms respectively.

- For measurement period (Tmeasure) without eDRX, 2 DRX cycles for 640ms and 4 DRX cycles for 320ms are defined.

- For evaluation period without eDRX, it is suggested to define Tevaluate as 4*Tmeasure for NC and 8*Tmeasure for EC.

- Recommended WF

  - Need more discussion.

Issue 1-3-4: Intra-frequency and inter-frequency neighbour cell measurement with eDRX:

- Proposals

  - Option 1: For eDRX cycles, the same existing requirements should apply for Tdetect with 1 DRX cycle for measurement and 2 DRX cycles for evaluation for both intra-frequency and inter-frequency neighbour cells. (R4-2006167, Qualcomm)

  - Option 2: For measurement of neighbor cell with eDRX, the existing requirement for Tdectect shall apply; Tmeasure is 1 DRX for 320ms and 640ms DRX; Tevalaute is 2 DRX cycles for NC and same requirements as that without eDRX for EC. (R4-2007691, Huawei)

- Recommended WF

  - Need more discussion on number of DRX cycles for evaluation for EC.

Issue 1-3-5: Measurement of Serving with WUS :

- Proposals

  - Option 1: Use the same requirements as in DRX cycle of 1.28s for shorter DRX cycles of 320/640ms when eDRX is and is not configured. (R4-2006167, Qualcomm)

  - Option 2: (R4-2007691, Huawei)

- When eDRX is not configured, the relaxation factor shall be updated for 320ms and 640ms DRX cycles as:

•	Min(n , 16) for 640ms;

•	Min(n , 32) for 320ms

- If eDRX is configured, for normal coverage, the relaxation factor shall be:

| DRX cycle length [s] | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2.56 ≤ PTW length [s] < 5.12 | 5.12 ≤ PTW length [s] < 7.68 | 7.68 ≤ PTW length [s] < 12.8 | 12.8 ≤ PTW length [s] < 23.04 | 23.04 ≤ PTW length [s] |
| 0.32 | Min(n , 2) | Min(n , 4) | Min(n , 8) | Min(n , 16) | Min(n , 32) |
| 0.64 | 1 | Min(n , 2) | Min(n , 4) | Min(n , 8) | Min(n , 16) |

- For enhanced coverage, the relaxation factor shall be:

| DRX cycle length [s] | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2.56 ≤ PTW length [s] < 5.12 | 5.12≤ PTW length [s] < 7.68 | 7.68 ≤ PTW length [s] < 12.8 | 12.8 ≤ PTW length [s] < 23.04 | 23.04 ≤ PTW length [s] < 43.52 | 43.52 ≤ PTW length [s] |
| 0.32 | 1 | Min(n , 2) | Min(n , 4) | Min(n , 8) | Min(n , 16) | Min(n , 32) |
| 0.64 | N/A | 1 | Min(n , 2) | Min(n , 4) | Min(n , 8) | Min(n , 16) |

- Recommended WF

  - Need more discussion.

Sub-topic 1-4 NTA_offset setting for NR coexistence with NB-IoT

There is one CR for this topic (R4-2007114). Please provide comments in 1.3.2	CRs/TPs comments collection

Companies views’ collection for 1st round

Open issues

Sub-topic 1-2 Filtering and combination of NRSRP measurement between anchor carrier and non-anchor carrier

| Company | Comments |
| --- | --- |
| Huawei | Issue 1-2-1: Support the recommended WF. As we have discussed the issue for several meetings. To make progress, companies agreed to adopt option 2b as the baseline. So there is no prerequisite or condition for UE in enhanced coverage when filtering the measurement between anchor and non-anchor carriers. Issue 1-2-2: Option 2. As mentioned in our comments to Issue 1-2-1, companies had compromised to the option 2b in the last meeting, thus we shall consider the condition in normal coverage. Issue 1-2-3: Option 2. Issue 1-2-4: Since there is no relative accuracy for NB-IoT. A relaxed margin shall be considered. We can compromise to 12 dB. Issue 1-2-5: We prefer a predefined value in the spec. It is also the way other margin are defined. For the value of M, we suggest the following values: DRX Cycle Length(s); M (DRX cycle). 0.32; 160. 0.64; 80. 1.28; 40. 2.56; 20. 5.12; 10. 10.24; 10. Issue 1-2-6: Support option 2. UE shall measure NRSRP on anchor and non-anchor within the same DRX every M DRX as suggested in issues 1-2-5. If the difference is larger than the margin, UE shall make the same comparison in the next DRX, and only after K times negative comparison, UE shall only perform NRSRP measurement on anchor carrier for certain time period. We also give our thoughts on the value of K here: DRX Cycle Length(s); K. 0.32; 8. 0.64; 8. 1.28; 4. 2.56; 4. 5.12; 2. 10.24; 2. |
| Qualcomm | Issue 1-2-1: Agree on WF (this was agreed in last meeting). Issue 1-2-2: We support option 1. We have yet to see a practical algorithm that realizes the benefits of RRM measurement in non-anchor carrier (power saving) while still enforcing restrictions on combining. The suggested algorithms from both Nokia and Huawei imposes new UE behaviors which UE does not need to do in some practical implementations. Our concerns are further discussed in topics below. Issue 1-2-3: We believe option 1 makes more sense. Since the discussion is about qualifying non-anchor carrier measurements for combining or not combining, it makes sense to look into non-anchor carrier SINR. But regardless, SINR > -6 dB is a high enough SINR that should not create any issue in combining or filtering. This is one of the many concerns that we have with the proposed algorithm. Issue 1-2-4: We cannot agree to option 1 as this violates the allowed estimation inaccuracy. This is another concern that we have with the proposed algorithm. How does a threshold of more than 12 dB (after power offset conversion) make any measurement better or more reliable? Issue 1-2-5: we can neither support option 1 or option 2. Option 1 implies that NW knows the condition of UE (whether it is in relaxed neighbor cell measurement or not, whether it needs to do resync often or not) and our concern is that by setting an unsuitable value, the power saving gains will be lost. Option 2 does not make sense to us either. What if UE does not need to do resync for a long time? Why should this better UE implementation be forced to drop back to anchor carrier? The goal of this feature is to save power by not bouncing back and forth between anchor and non-anchor and option 2 is doing the exact opposite. Issue 1-2-6: We cannot support either option 1 or option 2. Option 1 suggests 3 samples from each anchor and non-anchor carrier for averaging before comparison which is never going to happen very rarely because UE does not fallback so often to anchor carrier for sync (UE may fallback to anchor carrier, let’s say, once every minute.). Also, a typical UE implementation should not be required to do measurement on anchor and non-anchor in the same DRX cycle. We can’t support option 2 either. For this option, we don’t understand what “unavailable” measurement even means. If measurement on non-anchor carrier is bad, UE comes out of relaxed serving cell monitoring and perform measurement on the anchor carrier only anyway. RAN4 made the following agreements a few meetings ago: - If the following conditions are met: - The relaxed cell monitoring criteria defined in TS 36.304 clause 5.2.4.12, and - Transmit power difference of NRS between anchor carrier and non-anchor paging carrier is signalled to the UE via the existing parameter nrs-PowerOffsetNonAnchor, and - UE is not configured with any positioning measurements, - Then UE: - Stops monitoring neighbor cells - May stop performing NRSRP measurements on anchor carrier - May continue to perform NRSRP measurements only on the non-anchor carrier - May continue to evaluate relaxed monitoring conditions using non-anchor carrier NRSRP measurements and nrs-PowerOffsetNonAnchor for translation to their equivalent NRSRP in anchor carrier - Exiting this state will be based on not fulfilling the relaxed conditions in TS 36.304 clause 5.2.4.12. |
| Huawei: | Further comments: To Qualcomm: For issues 1-2-3: If the SINR condition is for non-anchor carrier, does it mean that for normal coverage, there is a new condition for non-anchor carrier NRSRP measurement? |
|  | Moderator: Considering that the issue about filtering among carriers has lasted for several meetings, and this is the last meeting we have, we need to make some progress. Observed from inputs from companies, the validation methods will anyway introduce extra UE behavior which goes against the purpose of the feature to save power. I summarize some inputs and agreements from the previous meetings, and try to get the whole picture and some potential solutions: - One condition for non-anchor carrier NRSRP measurement is the relaxed monitoring criteria is met, which means propagation channels in such scenarios are typically slowly varying. - UE may identify the need to perform re-synchronization on the anchor carrier or fallback due to other purposes. - UE could choose to performance NRSRP measurement on non-anchor carrier or not when the conditions are met. Based on the above observations, companies are encouraged to consider the following solutions: - In normal coverage, the SINR on non-anchor carrier shall meet the condition (>-6dB) - Don’t specify the fixed comparison period since UE could switch back to anchor carrier for sync or other purpose and the channel condition is slowly varying. - If the samples for filtering is from different carriers, UE shall validate the following condition: ¦ NRSRP (anchor) – (NRSRP (non-anchor) + Power_offset_anchor) ¦ < 12dB If the above inequation is not satisfied, UE shall abandon the first sample. Note: Based on the condition in 1nd bullet (SINR>-6dB on non-anchor carrier), it will happen in the case where UE switches to anchor carrier from non-anchor carrier and finds the NRSRP in anchor carrier is extreme low, thus the relaxation criteria is not fulfilled and UE shall re-evaluate the conditions for non-anchor carrier measurement. - There is no performance test for NRSRP measurement on non-anchor carrier, which could not be tested since UE could choose not to perform measurement on non-anchor carrier even the conditions are met. |
| Ericsson | Issue 1-2-1: We agree with the recommended WF. Issue 1-2-2: We support option 2. Issue 1-2-3: We are fine with option 1. Issue 1-2-4: We support option 1, but we are also fine to have this margin pre-defined in RAN4 spec. Issue 1-2-5: We are fine with option 2.We don’t have strong view about these two options. Issue 1-2-6: We support option 1. |
| Nokia | Issue 1-2-1: We support option 1. Issue 1-2-2: We support option 2. Issue 1-2-3: We support option 1. Issue 1-2-4: We support option 1. We think 12 or 18 dB are quite high values. Considering that two power measurements are compared (not on single shot basis), relative accuracy is relevant. But as these three proposed numbers (8 dB to 18 dB) differ in the range of 10 dB, a signaling solution for the margin M is preferred. Issue 1-2-5: We support option 1. Issue 1-2-6: We support option 1. |

Sub-topic 1-3 Updating RRM requirements for UE specific DRX cycles (320ms and 640ms)

| Company | Comments |
| --- | --- |
| Huawei | Issue 1-3-1: We support option 2. For shorter DRX cycles, we shall define a larger number of Nserv. The value in option 2 is taken eMTC as reference. Issue 1-3-2: We support the recommended WF. Issue 1-3-3: We support option 2. We suggest to keep the same principle when define requirements for different DRX cycles. For shorter DRX cycles, larger number of DRX cycles shall be defined. UE is not needed to perform measurement too frequent with short DRX cycles, while the delay is same or less. Based on option 2, the updated value is listed here: For NC: DRX cycle length [s]; Tdetect,NB_Intra_NC [s] (number of DRX cycles); Tmeasure,NB_Intra_NB_NC [s] (number of DRX cycles); Tevaluate,NB_intra_NB_NC [s] (number of DRX cycles). 0.32; 32 (100); 1.28 (4); 5.12 (16). 0.64; 26 (40); 1.28 (2); 5.12 (8). For EC -15≤ Q2 < -6: DRX cycle length [s]; Tdetect,NB_Intra_ EC [s] (number of DRX cycles); Tmeasure,NB_Intra_ EC [s] (number of DRX cycles); Tevaluate,NB_intra_ EC [s] (number of DRX cycles). 0.32; 338 (1054); 1.28 (4); 10.24 (32). 0.64; 266 (415); 1.28 (2); 10.24 (16). For EC Q2-6 DRX cycle length [s]; Tdetect,NB_Intra_ EC [s] (number of DRX cycles); Tmeasure,NB_Intra_ EC [s] (number of DRX cycles); Tevaluate,NB_intra_ EC [s] (number of DRX cycles). 0.32; 37 (113); 1.28 (4); 10.24 (32). 0.64; 29 (45); 1.28 (2); 10.24 (16). Issue 1-3-4:The difference between option 1 and option 2 is the number of DRX cycles for evaluation in EC with eDRX. It could be observed that the number of DRX for evaluation for 1.28s is 10, so for 0.32s and 0.64 it should be more than 10. The suggested number of value is shown here: Tmeasure,NB_Inter_EC [s] (number of DRX cycles); Tevaluate,NB_interEC [s] (number of DRX cycles). 0.32 (1); 10.24 (32). 0.64 (1); 10.24 (16). Issue 1-3-5: After checking RAN2’s spec. We think option 1 is reasonable. |
| Qualcomm | Issue 1-3-1: we support option 1. We don’t see why eMTC requirements should be duplicated here. Existing requirements for Nserv is not dependent on eDRX or DRX cycles so why should these new cycles be different? Issue 1-3-2: Agree on WF Issue 1-3-3: We support option 1. There is no need to support increased number of samples in shorter DRX cycles. Actually, because the issues corresponding to time/freq drift is significantly less of an issue in shorter DRX cycles, UE can even use less number of samples to come up with a measurement. So we think option 1 is reasonable. We have a question for proponent of option 2: what is the point or benefit of a shorter DRX cycle if UE takes the same time to make a measurement? Our view is that shorter DRX cycles should enable mobility scenarios and increasing the number of samples (as in option 2) does not allow this. Issue 1-3-4: Support option 1. Similar comments as in issue 1-3-3. Issue 1-3-5: Support option 1 as the RAN2 spec does not allow larger N. |
| Huawei | Further comments: Issue 1-3-1: For shorter DRX cycles, UE could trigger the neighbor cell measurement mistakenly due to instantaneous bad channel conditions. There is no issue related to power saving since the measure is done at least each DRX cycles. If companies has strong views on it, we can compromise to using same value for eDRX and DRX. Issue 1-3-3: The intention of option 2 is keep the same principle when defining requirement for different DRX length. It could be observed that for the existing DRX lengths, more DRX cycles is defined for the consideration of power saving. We don’t understand why for the new DRX cycles, the number of DRX cycles is same regardless of the DRX length. Also as discussed in the last meeting, we shall follow the same principle as requirement are defined for other DRX lengths, which is summarized in R4-164550. It could be observed that - Cell detection will be calculated based on number of needed NPSS and NSSS - For DRX it will be scaled. - Number of instances (NPSS or NSSS) to acquire per DRX cycle: - 1.28s: 5 - 2.56s: 10 - 5.12s: 10 - 10.24s: 20. For option 1, it means the number of instances per DRX cycles for 0.32 and 0.64 is assumed to be same as that of 1.28s. We think it is unreasonable. Option 2 means the number of instance is assumed to be : - 0.32s: 2 - 0.64s: 5 - 1.28s: 5 - 2.56s: 10 - 5.12s: 10 - 10.24s: 20 Issue 1-3-4: The difference between option 1 and option 2 is the number of DRX cycles when eDRX is configured in EC. It could be observed that in the existing requirement, there are more DRX cycles need for shorter DRX length (10 DRXs for DRX =1.28), however according to option 1, it means less DRXs for shorter DRX. (i.e. 10 DRXs for 1.28; 2 DRX for 0.32s), which is even less than the case when DRX = 1.28s. In general, the power consumption and performance shall be both considered, and it is also the way the existing requirements are defined. Otherwise, requirements for different DRX cycles are following different principles. |
| Ericsson | Issue 1-3-1: We support option 1, and share Qualcomm’s view that it shall not depend on the DRX cycle. Issue 1-3-2: We are fine with option 1. Issue 1-3-3, issue 1-3-4 and issue 1-3-5: We have similar view as Qualcomm and support option 1. |
| Huawei | Further comments: Issue 1-3-3 and Issue 1-3-4. We understand that the latency could be reduced by enabling the short DRX cycles (320ms and 640ms). However, option 1 is too strict requirements for NB-IoT, and it is even stricter than the requirements for LTE and it will greatly increase the power consumption. We think the power consumption should also be considered for NB-IoT. We can compromise to the following values, where the absolute delay is much shorter than 1.28s. We think it could be a good balance between performance and power consumption. Intra-frequency and inter-frequency neighbour cell measurement without eDRX NC DRX cycle length [s]; Tdetect,NB_Intar/inter_ NC [s] (number of DRX cycles); Tmeasure,NB_Intra/Inter_ NC [s] (number of DRX cycles); Tevaluate,NB_Intra/Inter_ NC [s] (number of DRX cycles). 0.32; 26 (80); 1.28 (4); 5.12 (16). 0.64; 26 (40); 1.28 (2); 5.12 (8). 1.28; 51 (40); 1.28 (1); 6.5 (5). 2.56; 51 (20); 2.56 (1); 7.68 (3). 5.12; 102 (20); 5.12 (1); 10.24 (2). 10.24; 102 (10); 10.24 (1); 20.48 (2). EC SCH Ês/Iot of neighboring cell: Q2; DRX cycle length [s]; Tdetect,NB_Intra/Inter_ EC [s] (number of DRX cycles); Tmeasure,NB_Intra/Inter_ EC [s] (number of DRX cycles); Tevaluate,NB_Intra、Inter_ EC [s] (number of DRX cycles). -15≤ Q2 < -6; 0.32; 256 (800); 1.28 (4); 10.24 (32). 0.64; 266 (415); 1.28 (2); 10.24 (16). 1.28; 532 (415); 1.28 (1); 12.8 (10). 2.56; 532 (208); 2.56 (1); 15.36 (6). 5.12; 1063 (208); 5.12 (1); 20.48 (4). 10.24; 1063 (104); 10.24 (1); 30.72 (3). Q2-6; 0.32; 26 (80); 1.28 (4); 10.24 (32). 0.64; 29 (45); 1.28 (2); 10.24 (16). 1.28; 58 (45); 1.28 (1); 12.8 (10). 2.56; 59 (23); 2.56 (1); 15.36 (6). 5.12; 113 (22); 5.12 (1); 20.48 (4). 10.24; 113 (11); 10.24 (1); 30.72 (3). Intra-frequency and inter-frequency neighbour cell measurement with eDRX: Take intra-f as example eDRX_IDLE cycle length [s]; DRX cycle length [s]; PTW length [s] (number of 2.56s periods); Tdetect,NB_Intra_EC [s] (number of DRX cycles); Tmeasure,NB_Intra_EC [s] (number of DRX cycles); Tevaluate,NB_intraEC [s] (number of DRX cycles). 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76; 0.32; ≥12.8 (5); (406); 0.32 (1); 10.24 (32). 0.64; ≥12.8 (5); 0.64 (1); 10.24 (16). 1.28; ≥15.36 (6); 1.28 (1); 12.8 (10). 2.56; ≥17.92 (7); 2.56 (1); 15.36 (6). 5.12; ≥23.04 (9); 5.12 (1); 20.48 (4). 10.24; ≥33.28 (13); 10.24 (1); 30.72 (3). NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section X of TS 24.008 [34].. |

CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2007688 | Company A |
|  | Company B |
|  | Qualcomm: RSRP should be changed to NRSRP. Also, the numbers 4 and 5 dB quoted for normal and enhanced coverage are eMTC numbers; not NB-IoT. Ericsson: PUR needs to be aligned with the corresponding changes for eMTC. |
| R4-2007114 | Company A |
|  | Company B |
| R4-2007685 |  |
| R4-2007686 | The first change needs a reference and should have 4T instead of fourT The second change is not needed as it is not a requirement, it’s a suggestion and can be left to UE implementation. Ericsson: We think some restructuring of the changes is needed to make the requirements more clear, we have shown it in R4-2007976. For example, UE needs to evaluate the cell using a different S-criteria when NRSRP is measured on the non-anchor carrier compared to anchor carrier, this needs to be made clearer. Another reason for restricting is that, currently the three criteria listed are for the measurements, and with current wording one may interpret that it also applies to the serving cell evaluation which is not the intention. Thus this CR needs to be revised to include the changes in R4-2007976, we can revise the CR and send it out. |
| R4-2007687 | Same comment as for R4-2007686. |
| R4-2007689 | Need to be revised based on conclusion of sub topic 1-3. Ericsson: Revision needed based on ongoing discussions. |
| R4-2007976 | This CR can be merged into R4-2007686 and R4-2007687. |
| R4-2007976 |  |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1-2 | Issue 1-2-1: Condition on filtering and combination of NRSRP measurement between anchor carrier and non-anchor carrier in enhanced coverage. Tentative agreements: There is no prerequisite or condition for UE in enhanced coverage when filtering the measurement between anchor and non-anchor carriers. Recommendations for 2nd round: Not needed. Issue 1-2-2: Whether to introduce conditions on filtering and combination of NRSRP measurement between anchor carrier and non-anchor carrier in normal coverage. Status: 1 company supports option 1, and 3 companies support option 2. Based on the agreements from last meeting, the condition on filtering and combination of NRSRP measurement between carriers shall be considered in normal coverage. Moderate suggest the following tentative agreements and keep discussing the mechanism of comparison. Tentative agreements: NRSRP measurements on non-anchor carrier can be filtered or combined with NRSRP measurement on anchor carrier after translating the non-anchor carrier measurement with parameter nrs-PowerOffsetNonAnchor, provided that - SINR ≥ -6 dB on the non-anchor carrier - the comparison of NRSRP samples between anchor and non-anchor carrier accounting for the signal power offset, yields a difference within a margin M dB Recommendations for 2nd round: Keep discussing the value of M, the periodicity for comparison and samples for comparison in the 2nd round. Issue 1-2-3: The SINR for qualifying a measurement for combining and filtering is on the anchor carrier or non-anchor carrier. Tentative agreements: SINR ≥ -6 dB on the non-anchor carrier Recommendations for 2nd round: Not needed. Issue 1-2-4: The value of the margin M dB and the method for definition. Status:2 companies support option 1, and 2 companies support option 3, and 1 companies support it should be signaled by NW. Considering that the M dB margin is for comparison, though there is not relative accuracy defined for NB-IoT, it is reasonable to take absolute accuracy as reference. Thus, moderator suggest it should be defined in the spec, and provide a compromised value of 10 dB for 2nd round discussion Tentative agreements: The margin M dB is predefined in 36.133. Candidate options: Option 1: 8 dB Option 2: 12 dB Option 3: 10 dB (Moderator’s suggestion) Recommendations for 2nd round: Discuss the value from the above options Issue 1-2-5: The periodicity of NRSRP comparison. Status: 1 company supports option 1, 2 companies support option 2, and 1 company support option 3. Companies should consider that enabling NRSRP measurement on non-anchor is to save power, and the reliability should also be guaranteed, and this is why UE has to fulfill the conditions (i..e relaxation monitoring criteria) before performing measurement on non-anchor carrier. So we should consider both the powering and reliability instead of only one of them, otherwise there is no benefits to support the feature (no power saving or unreliable measurement). Based on inputs, moderator suggest following 2 options for 2nd round discussion with some suggested value of periodicity. Tentative agreements: No Candidate options: Option 1: It is left to UE implementation Option 2: The periodicity is defined in TS 36.133 with the value of (Moderator’s suggestion) 2a: 5 mins 2b: 1 hour 2c: 24 hour Recommendations for 2nd round: Discuss the options above in the 2nd round Issue 1-2-6: UE behaviour on comparison of NRSRP measurements of anchor and non-anchor carriers Status: 2 companies support option 1 and 1 company can compromise to option 1, 1 company support neither option 1 or 2. Tentative agreements: No Candidate options: Option 1: Use at least 3 samples from each carrier for the comparison. After the positive comparison, the UE is allowed to stay on the non-anchor carrier until the next comparison takes place Option 2: If measurement on non-anchor carrier is bad, UE comes out of relaxed serving cell monitoring and perform measurement on the anchor carrier only. (Comments from 1st round) Recommendations for 2nd round: Discuss the above options in the 2nd round. Could Qualcomm clarify the meaning of “measurement on non-anchor carrier is bad” and is it related to the M dB Margin? |

|  | Status summary |
| --- | --- |
| Sub-topic#1-4 | Issue 1-3-1: Nserv for measurement of serving cell Status: 1 company supports option 2, and 2 companies support option 1. Tentative agreements: Nserv = 2 for normal coverage and Nserv = 4 for enhanced coverage Recommendations for 2nd round: Not needed. Issue 1-3-2: WUS reception (clause 4.6.2.9) Status: 3 company support option 1. Tentative agreements: Same requirements for WUS for shorter DRX cycles as in DRX cycles ≤ 5.12s Recommendations for 2nd round: Not needed. Issue 1-3-3: Intra-frequency and inter-frequency neighbour cell measurement without eDRX: Status: 2 company support option 1, and 1 company supports option 2. Companies should notice that Option 1 means the requirement for 320ms and 640ms DRX is even stricter than that for LTE(i.e. when DRX = 0.32s, for LTE: 4 DRX for measurement, 16 DRX for evaluation; Option 1: 1 DRX for measurement, 5 DRX for evaluation). Tentative agreements: No Candidate options: Option 1: (original) - 40 DRX cycles for 320/640ms DRX to detect the cell, 1 DRX cycle to measure, and 5 DRX cycles to evaluate in normal coverage - 415 DRX cycles for 320/640 DRX to detect the cell, 1 DRX cycle to measure, and 10 DRX cycles to evaluate in enhanced coverage when -15 ≤ Q2 < -6 - 45 DRX cycles for 320/640 DRX to detect the cell, 1 DRX cycle to measure, and 10 DRX cycles to evaluate in enhanced coverage when Q2 ≥ -6. Option 2: (Proposed by Huawei during the 1st round discussion) For normal coverage: DRX cycle length [s]; Tdetect,NB_Intar/inter_ NC [s] (number of DRX cycles); Tmeasure,NB_Intra/Inter_ NC [s] (number of DRX cycles); Tevaluate,NB_Intra/Inter_ NC [s] (number of DRX cycles). 0.32; 26 (80); 1.28 (4); 5.12 (16). 0.64; 26 (40); 1.28 (2); 5.12 (8). 1.28; 51 (40); 1.28 (1); 6.5 (5). 2.56; 51 (20); 2.56 (1); 7.68 (3). 5.12; 102 (20); 5.12 (1); 10.24 (2). 10.24; 102 (10); 10.24 (1); 20.48 (2). For enhanced coverage SCH Ês/Iot of neighboring cell: Q2; DRX cycle length [s]; Tdetect,NB_Intra/Inter_ EC [s] (number of DRX cycles); Tmeasure,NB_Intra/Inter_ EC [s] (number of DRX cycles); Tevaluate,NB_Intra/Inter_ EC [s] (number of DRX cycles). -15≤ Q2 < -6; 0.32; 256 (800); 1.28 (4); 10.24 (32). 0.64; 266 (415); 1.28 (2); 10.24 (16). 1.28; 532 (415); 1.28 (1); 12.8 (10). 2.56; 532 (208); 2.56 (1); 15.36 (6). 5.12; 1063 (208); 5.12 (1); 20.48 (4). 10.24; 1063 (104); 10.24 (1); 30.72 (3). Q2-6; 0.32; 26 (80); 1.28 (4); 10.24 (32). 0.64; 29 (45); 1.28 (2); 10.24 (16). 1.28; 58 (45); 1.28 (1); 12.8 (10). 2.56; 59 (23); 2.56 (1); 15.36 (6). 5.12; 113 (22); 5.12 (1); 20.48 (4). 10.24; 113 (11); 10.24 (1); 30.72 (3). Recommendations for 2nd round: Discuss the above options in the 2nd round. Issue 1-3-4: Intra-frequency and inter-frequency neighbour cell measurement with eDRX: Status: 2 company support option 1, and 1 company supports option 2. The difference between Option 1 and option 2 is the number of DRX cycle for evaluation in enhanced coverage, so the consistent parts are agreeable. Tentative agreements: In normal coverage, the same existing requirements should apply for Tdetect with 1 DRX cycle for measurement and 2 DRX cycles for evaluation for both intra-frequency and inter-frequency neighbour cells. In enhance coverage, the same existing requirements should apply for Tdetect with 1 DRX cycle for measurement. Candidate options: For evaluation in enhanced coverage, option 1 and option 2 are shown in the following tables: Option 1: eDRX_IDLE cycle length [s]; DRX cycle length [s]; PTW length [s] (number of 2.56s periods); Tdetect,NB_Intra_EC [s] (number of DRX cycles); Tmeasure,NB_Intra_EC [s] (number of DRX cycles); Tevaluate,NB_intraEC [s] (number of DRX cycles). 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76; 0.32; ≥12.8 (5); (406); 0.32 (1); 0.64 (2). 0.64; ≥12.8 (5); 0.64 (1); 1.28 (2). 1.28; ≥15.36 (6); 1.28 (1); 12.8 (10). 2.56; ≥17.92 (7); 2.56 (1); 15.36 (6). 5.12; ≥23.04 (9); 5.12 (1); 20.48 (4). 10.24; ≥33.28 (13); 10.24 (1); 30.72 (3). NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section X of TS 24.008 [34].. Otpion 2: eDRX_IDLE cycle length [s]; DRX cycle length [s]; PTW length [s] (number of 2.56s periods); Tdetect,NB_Intra_EC [s] (number of DRX cycles); Tmeasure,NB_Intra_EC [s] (number of DRX cycles); Tevaluate,NB_intraEC [s] (number of DRX cycles). 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76; 0.32; ≥12.8 (5); (406); 0.32 (1); 10.24 (32). 0.64; ≥12.8 (5); 0.64 (1); 10.24 (16). 1.28; ≥15.36 (6); 1.28 (1); 12.8 (10). 2.56; ≥17.92 (7); 2.56 (1); 15.36 (6). 5.12; ≥23.04 (9); 5.12 (1); 20.48 (4). 10.24; ≥33.28 (13); 10.24 (1); 30.72 (3). NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section X of TS 24.008 [34].. Recommendations for 2nd round: Discuss the options above in the 2nd round. Issue 1-3-5: Measurement of Serving with WUS : Tentative agreements: Use the same requirements as in DRX cycle of 1.28s for shorter DRX cycles of 320/640ms when eDRX is and is not configured. Recommendations for 2nd round: Not needed. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on RRM requirements for R16 enhancement for NB-IoT | Huawei |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2007688 | To be revised |
| R4-2007114 | Agreeable |
| R4-2007685 | Agreeable |
| R4-2007686 | To be revised |
| R4-2007687 | To be revised |
| R4-2007689 | To be revised |
| R4-2007976 | To be revised |

Discussion on 2nd round (if applicable)

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |