3GPP TSG-RAN4 Meeting #95-e	R4-2009006

# Electronic meeting, 25th May – 5th June, 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0639 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on CSI-RS based intra-frequency measurement requirement (Introduction, requirement applicability and number of cell and beams) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CSIRS_L3meas-Core |  |  |  |  |  |  | Date: |  | 2020-5-5 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CSI-RS L3 measurement was introduced in Rel-16, hence the CSI-RS based intra-frequency measurement requirements shall be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce section introduction, requirement applicability and number of cell and beams for CSI-RS based intra-frequency measurement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CSI-RS based intra-frequency measurement requirements are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 9.x.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change #1>>

9.x.2	CSI-RS based intra-frequency measurements

9.x.2.1	Introduction

A measurement is defined as a CSI-RS based intra-frequency measurement provided that:

    - the SCS of CSI-RS resources on the neighbour cell configured for measurement is the same as the SCS of CSI-RS resources on the serving cell configured for measurement, and

    - the CP type of CSI-RS resources on neighbour cell configured for measurement is the same as the CP type of CSI-RS resources on the serving cell configured for measurement, and

  - It is applied for SCS = 60KHz

    - the centre frequency of CSI-RS resources on the neighbour cell configured for measurement is the same as the centre frequency of CSI-RS resource on the serving cell configured for measurement

The UE shall be able to identify new intra-frequency cells and perform CSI-RSRP, CSI-RSRQ and CSI-SINR measurements of identified intra-frequency cells if carrier frequency information is provided by PCell or the PSCell.

The UE can perform intra-frequency CSI-RS based measurements without measurement gaps if

-	the CSI-RS resources are completely contained in the active BWP  of the UE

For intra-frequency CSI-RS based measurements without measurement gaps, UE may cause scheduling restriction as specified in clause 9.x.2.5.

CSI-RS based measurements are configured along with a measurement timing configuration (CMTC) per carrier, which provides periodicity, duration and offset information on a window of up to 5ms where the measurements of CSI-RS on the configured intra-frequency carrier are to be performed. Up to two measurements window periodicity may be configured per intra-frequency measurement object.

9.x.2.2	Requirements applicability

The requirements in clause 9.x.2 apply, provided:

-	All CSI-RS resources in the same MO are configured with the same csi-rs-MeasurementBW.

-	The BW of the CSI-RS on the neighbor cell is within the active BWP of the UE

-	The CSI-RS resources and the associated SSB of the cell being identified or measured are detectable.

An intra-frequency cell shall be considered detectable when for each relevant CSI-RS and associated SSB:

-	CSI-RSRP related side conditions given in clauses 10.1 and 10.1 for FR1 and FR2, respectively, for a corresponding Band,

-	CSI-RSRQ related side conditions given in clauses 10.1 and 10.1 for FR1 and FR2, respectively, for a corresponding Band,

-	CSI-SINR related side conditions given in clauses 10.1 and 10.1 for FR1 and FR2, respectively, for a corresponding Band,

-	CSI_RP and CSI Ês/Iot according to Annex B.2.2 for a corresponding Band.

-	SS-RSRP related side conditions given in clauses 10.1.2 and 10.1.3 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-RSRQ related side conditions given in clauses 10.1.7 and 10.1.8 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-SINR related side conditions given in clauses 10.1.12 and 10.1.13 for FR1 and FR2, respectively, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.2 for a corresponding Band.

9.x.3	Number of cells and number of CSI-RS

9.x.3.1	Requirements for FR1

9.x.3.2	Requirements for FR2

<< End of Change #1>>