3GPP TSG-RAN4 #96-e	R4-2009663

Online, 17th – 28th Aug 2020

Title:	Testability issue with OoBB for FR1 EN-DC UE (3)

Source:			Anritsu Corporation

Agenda Item:			4.2.3.3

Document for:			Approval

# 1.	Introduction

Testability issue with OoBB for FR1 EN-DC UE was raised at first in #94-e meeting in February [5]. Though the group continued discussion for consecutive 3 meetings, we have not reached the conclusion and rather found an issue that our basic assumption is not aligned among companies. In this contribution we summarize the current status of discussions and try to create a common test assumption for this requirement.

# 2.	Discussion

2.1 Open issues

There are three remaining issues after the discussions of #95-e (2020 May). Issues and alternatives based on the previous discussions among companies are as follows.

Issue 1)	Assumption of UL power level whose DL is being tested is unclear

alternative 1-1: PCMAX_L,c – 14 dB (original proposal in [8][9])

alternative 1-2: PCMAX_L,c – 7 dB (condition defined in TS38.101-1 [1] as inter-band CA case)

alternative 1-3: PCMAX_L,c – 29 dB (preference of one UE vendor and a compromise value by one TE vendor)

Issue 2)	Assumption of UL power level for the source of IMD is unclear

alternative 2-1: PCMAX_L,c – 4 dB (current test condition)

alternative 2-2: PCMAX_L,c – 7 dB (condition defined in TS38.101-1 [1] as inter-band CA case)

Issue 3)	Test condition for 2UL CA and its self-interference

There are already test conditions of OoBB with 2UL inter-band CA cases in both E-UTRA (TS36.101 [12]) and NR (TS 38.101-1). Transmitter power level for both UL carriers is 7 dB below PCMAX_L,c for each serving cell.

However, there is a concern of the self-interference due to intermodulation products between the CW interfering signal and the lower frequency band UL carrier [10][11]. Therefore there is a need to confirm the influence of the 2nd or 3rd order IMD corresponds to the configured UL carrier frequency with transmitter power level 7 dB below PCMAX_L,c  in a case 2UL carrier exist simultaneously.

2.2 Supposed UE operation mode

As for issue 1) and 2) above, we would also point another aspect.

Issue 4) 	Assumption with regards to the UE operation mode and applicable absolute UL power for OoBB test case is unclear.

A study after the #95-e meeting and some offline discussions revealed another issue that a supposed UE operation mode is not clear. As the EN-DC UE currently has multiple unique operation modes such as single uplink operation (SUO) and dynamic power sharing (DPS) as well as the existing operation, there can be multiple combinations of UL/DLs depending on the actual UL power level while same configuration is applied to the UE.

Case 1) 2UL/2DL with equal UL power between E-UTRA and NR

Case 2) 1UL/2 (or 1) DL with Single UL operation (SUO)

Case 3) 2UL/2DL with imbalanced UL power between E-UTRA and NR
              (Similar to 1UL/2DL from IMD point of view)

Case 3’) 2UL/2DL with imbalanced UL power between E-UTRA and NR with Dynamic Power Sharing (DPS) capability

Also figure 2.2-1 and 2.2-2 depict relationship between two ULs under SUO and DPS modes.

Figure 2.2-1: UL power relationship under single uplink operation (SUO)

Figure 2.2-2: UL power relationship under dynamic power sharing (DPS)

Observation 1: Multi-RAT DC UEs have multiple capabilities of operation modes. Thus we need to create a common test assumption also taking these factors and applicable absolute UL power into account as well as the combination of duplex modes.

When we consider TRx RF requirements such as 38.101-1, 2 and 3, it is preferred that the conditions are as generic as possible irrespective of any UE operation modes, antenna designs, UE front end design and duplex modes, etc. So it is not preferred the requirements are separated depending on each mode, etc.

Observation 2: It is preferred that TRx RF requirements are defined as general as possible irrespective of any UE operation modes, UE design and duplex modes.

Therefore considering a balance between the already existing test conditions and idea of choosing the worst (severest) condition, alternative 2-1: PCMAX_L,c – 4 dB is the appropriate assumption of UL power level for the source of IMD unless the assumed absolute UL power level is changed.

Proposal 1: Apply PCMAX_L,c – 4 dB as UL power level for the source of IMD unless the assumed absolute UL power is changed.

For an assumption of UL power level whose DL is being tested, it depends on whether we take the issue 3) in clause 2.1 above into account. From our precedent with E-UTRA that we agreed once to follow the proposals in [10][11], 1UL test configurations is sufficient to test out the receiver minimum performance as demanded by both 1UL and 2UL CA out-of-band blocking requirements for E-UTRA. Therefore the assumption of UL power level whose DL is being tested for EN-DC UE should be decided taking into account of the balance between the analysis of 2nd or 3rd order IMD impact for 2UL/2DL configuration and the TE dynamic range. (i.e. In between PCMAX_L,c – 14 dB and PCMAX_L,c – 29 dB.)

Proposal 2: Choose the UL power level whose DL is being tested taking into account of the balance between the analysis of 2nd or 3rd order IMD impact for 2UL/2DL configuration and the TE dynamic range. (i.e. In between PCMAX_L,c – 14 dB and PCMAX_L,c – 29 dB.)

- Conclusion

In this contribution we summarized the current status of the EN-DC OoBB testability discussions and tried to create a common test assumption for this requirement.

Observation 1: Multi-RAT DC UEs have multiple capabilities of operation modes. Thus we need to create a common test assumption also taking these factors and applicable absolute UL power into account as well as the combination of duplex modes.

Observation 2: It is preferred that TRx RF requirements are defined as general as possible irrespective of any UE operation modes, UE design and duplex modes.

Proposal 1: Apply PCMAX_L,c – 4 dB as UL power level for the source of IMD unless the assumed absolute UL power is changed.

Proposal 2: Choose the UL power level whose DL is being tested taking into account of the balance between the analysis of 2nd or 3rd order IMD impact for 2UL/2DL configuration and the TE dynamic range. (i.e. In between PCMAX_L,c – 14 dB and PCMAX_L,c – 29 dB.)

# 4.	References

[1] TS 38.101-1, “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”, V16.4.0, 2020-06

[2] TS 38.101-3, “User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”, V16.4.0, 2020-06

[3] TS 38.213, “NR; Physical layer procedures for control”, 16.2.0, 2020-06

[4] TS 38.306, “User Equipment (UE) radio access capabilities”, 16.1.0, 2020-07

[5] R4-2000439, “Testability issue with OoBB for FR1 EN-DC UE”, Anritsu Corp., RAN4 #94-e, Online

[6] R4-2003327, “Testability issue with OoBB for FR1 EN-DC UE (2)”, Anritsu Corp., RAN4 #94-e-bis, Online

[7] R4-2008936, “Email discussion summary for [95e][105] NR_NewRAT_UE_RF_Part_4”, Moderator (Apple Inc.), RAN4 #95-e, Online

[8] R4-2003328, “Draft CR to out-of-band blocking for DC in FR1”, Anritsu corp., RAN4 #94-e-bis, Online

[9] R4-2006146, “CR to out-of-band blocking for DC in FR1”, Anritsu corp., RAN4#95-e, Online

[10] R4-150958, “2UL inter-band CA out-of-band blocking test consideration”, Mediatek, RAN4 #74, Athens, Greece

[11] R4-152187, “2UL inter-band CA out-of-band blocking test consideration”, Mediatek, RAN4 #74bis, Rio de Janeiro, Brazil

[12] TS 36.101, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”, V16.6.0, 2020-06