3GPP TSG-RAN WG4 Meeting #96-e	R4-2009680

Electronic Meeting, 17 August – 28 August, 2020

Source:	ZTE

Title:	transmit timing requirements of IAB-MTs in CA scenarios

Agenda item:	7.4.3.2

Document for:	Discussion

# 1	Introduction

- In RAN4 95-e, the following agreements are captured in WF [1]:

WF on CA scenarios in IAB-MT timing requirements - Define transmit timing requirements of IAB-MTs, that are supporting local area IAB-MT class, in CA scenarios - The transmit timing requirements of IAB-MTs, that are supporting local area IAB-MT class, in CA scenarios can reuse the related requirements of Rel-15 UEs - FFS: Whether to define transmit timing requirements of IAB-MTs, that are supporting wide area IAB-MT class, in CA scenarios. - Down select from following options. - Define requirements - Supported by: Nokia, Qualcomm and Samsung - Do not define requirements - Supported by: Ericsson

- The discussion in this paper is based on the agreements above.

2	Transmit timing requirements for IAB MTs

During RAN4 94-e-bis meeting, the WF [2] from RF session was to only limit the requirements for local area (LA) IAB-MTs, other types FFS. The agreements are copied below:

For the multi-band, multi-carrier and CA requirements, IAB-MT requirements follow BS approach for all wide area IAB-MT class only, other IAB-MT class FFS.

During RAN4 95-e meeting, there was no further agreements in the RF session, so we were at the same place as the end of RAN4 94-e-bis. This is why we think for now, we should not define transmit timing requirements for wide area IAB-MTs. The agreements in the RF session were clear and straightforward which is to follow BS approach, which means UE-like transmit timing requirements are not needed.

Do not define transmit timing requirements for wide area IAB-MTs.

3	Conclusion

- Proposal 1: Do not define transmit timing requirements for wide area IAB-MTs.

References

- R4-2008595

- R4-2005482