3GPP TSG-RAN WG4 Meeting #96-e	R4-2009764

# Electronic Meeting, 17th Aug – 28th Aug 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0929 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on measurement relaxation requirements for UEs under power saving mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav-Core |  |  |  |  |  | Date: |  |  | 2020-07-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The CR (R4-2009133) agreed in RAN4#95e meeting was not fully captured the agreement in agreed WF (R4-2009265). In addition, some further revision is needed based on the RAN2’s latest agreement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add some missing conditions and scenarios for measurement relaxation requirement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE measurement behaviour during power saving mode is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 4.2.2.8, 4.2.2.9, and 4.2.2.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change #1>>

4.2.2.8 	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion

4.2.2.8.1 	Introduction

This section contains the requirements for measurements on intra-frequency NR cells when the UE is configured any of the following relaxed measurement critera:

- Relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.X.1 in [1],

- Relaxed measurement criterion for UE not-at-cell edge defined in clause 5.2.4.X.2 in [1],

- Both low mobility criterion and not-at-cell edge criterion as defined in clauses 5.2.4.X.1 and 5.2.4.X.2 in [1] respectively.

4.2.2.8.2 	Measurements for UE fulfilling low mobility criterion

This clause contains requirements for measurements on intra-frequency NR cells provided that:

- T331 timer is not running for EMR measurements on intra-frequency NR carrier and

- UE is configured with low mobility criterion, or

- UE is configured with both low mobility criterion and not-at-cell edge criteria but has fulfilled only the low mobility criterion.

When Srxlev > SIntraSearchP and Squal > SIntraSearchQ, then the UE may not search for and measure intra-frequency cells.

When Srxlev ≤ SIntraSearchP or Squal ≤ SIntraSearchQ then the UE shall search for and measure intra-frequency cells in preparation for possible reselection. The requirements defined in clause 4.2.2.3 apply for this section except that:

-	Tdetect,NR_Intra as specified in Table 4.2.2.8.1-1.

-	Tmeasure,NR_Intra as specified in Table 4.2.2.8.1-1.

-	Tevaluate,NR_Intra as specified in Table 4.2.2.8.1-1.

Table 4.2.2.8.1-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x M2 x K1 (36 x N1 x M2 x K1) | 1.28 x N1 x M2 x K1 (4 x N1 x M2 x K1) | 5.12 x N1 x M2 x K1 (16 x N1 x M2 x K1) |
| 0.64 |  | 5 | 17.92 x N1 x K1 (28 x N1 x K1) | 1.28 x N1 x K1 (2 x N1 x K1) | 5.12 x N1 x K1 (8 x N1 x K1) |
| 1.28 |  | 4 | 32 x N1 x K1 (25 x N1 x K1) | 1.28 x N1 x K1 (1 x N1 x K1) | 6.4 x N1 x K1 (5 x N1 x K1) |
| 2.56 |  | 3 | 58.88 x N1 x K1 (23 x N1 x K1) | 2.56 x N1 x K1 (1 x N1 x K1) | 7.68 x N1 x K1 (3 x N1 x K1) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. Note 2: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. Note 3: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the low mobility. |  |  |  |  |  |

4.2.2.8.3 	Measurements for UE fulfilling not-at-cell edge criterion

This clause contains requirements for measurements on intra-frequency NR cells provided that:

- T331 timer is not running for EMR measurements on intra-frequency NR carrier and

- UE is configured with not-at-cell edge criterion, or

- UE is configured with both low mobility criterion and not-at-cell edge criteria but has fulfilled only the not-at-cell edge criterion.

When Srxlev > SIntraSearchP and Squal > SIntraSearchQ,  then the UE may not search for and measure intra-frequency cells.

When Srxlev ≤ SIntraSearchP or Squal ≤ SIntraSearchQ then the UE shall search for and measure intra-frequency cells in preparation for possible reselection. The requirements defined in clause 4.2.2.3 apply for this section except that:

-	Tdetect,NR_Intra as specified in Table 4.2.2.8.3-1.

-	Tmeasure,NR_Intra as specified in Table 4.2.2.8.3-1.

-	Tevaluate,NR_Intra as specified in Table 4.2.2.8.3-1.

Table 4.2.2.8.3-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x M2 x K1 (36 x N1 x M2 x K1) | 1.28 x N1 x M2 x K1 (4 x N1 x M2 x K1) | 5.12 x N1 x M2 x K1 (16 x N1 x M2 x K1) |
| 0.64 |  | 5 | 17.92 x N1 x K1 (28 x N1 x K1) | 1.28 x N1 x K1 (2 x N1 x K1) | 5.12 x N1 x K1 (8 x N1 x K1) |
| 1.28 |  | 4 | 32 x N1 x K1 (25 x N1 x K1) | 1.28 x N1 x K1 (1 x N1 x K1) | 6.4 x N1 x K1 (5 x N1 x K1) |
| 2.56 |  | 3 | 58.88 x N1 x K1 (23 x N1 x K1) | 2.56 x N1 x K1 (1 x N1 x K1) | 7.68 x N1 x K1 (3 x N1 x K1) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. Note 2: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. Note 3: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the not-at-cell edge criterion. |  |  |  |  |  |

4.2.2.8.4 	Measurements for UE fulfilling low mobility and not-at-cell edge criteria

This clause contains requirements for measurements on intra-frequency NR cells provided that:

- T331 timer is not running for EMR measurements on intra-frequency NR carrier, and

- UE is configured with both low mobility criterion and not-at-cell edge criterion, and has also fulfilled both criteria

In this case the UE is not required to meet Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra as defined in Table 4.2.2.3-1.

<< End of Change #1>>

<< Start of Change #2>>

4.2.2.9 	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion

4.2.2.9.1 	Introduction

This section contains the requirements for measurements on inter-frequency NR cells when the UE is configured with any of following relaxed measurement criteria:

- Relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.X.1 in [1],

- Relaxed measurement criterion for UE not-at-cell edge defined in clause 5.2.4.X.2 in [1],

- Both low mobility criterion and not-at-cell edge criterion as defined in clauses 5.2.4.X.1 and 5.2.4.X.2 in [1] respectively.

4.2.2.9.2 	Measurements for UE fulfilling low mobility criterion

This clause contains requirements for measurements on inter-frequency NR cells provided that:

- T331 timer is not running for EMR measurements on inter-frequency NR carrier, and

- UE is configured with low mobility criterion, or

- UE is configured with both low mobility criterion and not-at-cell edge criterion, and UE has fulfilled only the low mobility criterion

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. The requirements defined in clause 4.2.2.4 apply for this section except that:

-	Tdetect,NR_Inter as specified in Table 4.2.2.9.2-1.

-	Tmeasure,NR_Inter as specified in Table 4.2.2.9.2-1.

-	Tevaluate,NR_Inter as specified in Table 4.2.2.9.2-1.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ,

- UE may not search for and measure inter-frequency layers of equal or lower priority.

- If the UE is configured with highPriorityMeasRelax [2] then the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search seconds where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60.

- If the UE is not configured with highPriorityMeasRelax [2] then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

Table 4.2.2.9.2-1: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Inter [s] (number of DRX cycles) | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x 1.5 x K1 (36 x N1 x 1.5 x K1) | 1.28 x N1 x 1.5 x K1 (4 x N1 x 1.5 x K1) | 5.12 x N1 x 1.5 x K1 (16 x N1 x 1.5 x K1) |
| 0.64 |  | 5 | 17.92x N1 x K1 (28 x N1 x K1) | 1.28 x N1 x K1 (2 x N1 x K1) | 5.12 x N1 x K1 (8 x N1 x K1) |
| 1.28 |  | 4 | 32 x N1 x K1 (25 x N1 x K1) | 1.28 x N1 x K1 (1 x N1 x K1) | 6.4 x N1 x K1 (5 x N1 x K1) |
| 2.56 |  | 3 | 58.88 x N1 x K1 (23 x N1 x K1) | 2.56 x N1 x K1 (1 x N1 x K1) | 7.68 x N1 x K1 (3 x N1 x K1) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. Note 2: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the low mobility. |  |  |  |  |  |

4.2.2.9.3 	Measurements for UE fulfilling not-at-cell edge criterion

This clause contains requirements for measurements on inter-frequency NR cells provided that:

- T331 timer is not running for EMR measurements on inter-frequency NR carrier, and

- UE is configured with not-at-cell edge criterion, or

- UE is configured with both low mobility criterion and not-at-cell edge criterion, but has fulfilled only the not-at-cell edge criterion

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. The requirements defined in clause 4.2.2.3 apply for this section except that:

-	Tdetect,NR_Inter as specified in Table 4.2.2.9.3-1.

-	Tmeasure,NR_Inter as specified in Table 4.2.2.9.3-1.

-	Tevaluate,NR_Inter as specified in Table 4.2.2.9.3-1.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ,

- UE may not search for and measure inter-frequency layers of equal or lower priority.

- When UE is only configured with not-at-cell edge criterion, and it is fulfilled, then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 regardless of whether the UE is configured with highPriorityMeasRelax [2] or not.

- When UE is configured with both low mobility criterion and not-at-cell edge criterion, if combineRelaxedMeasCondition [2] is not configured and only the criteria of not-at-cell-edge is fulfilled, the UE shall search for and measure inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 regardless of whether the UE is configured with highPriorityMeasRelax [2] or not.

Table 4.2.2.9.3-1: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Inter [s] (number of DRX cycles) | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x 1.5 x K1 (36 x N1 x 1.5 x K1) | 1.28 x N1 x 1.5 x K1 (4 x N1 x 1.5 x K1) | 5.12 x N1 x 1.5 x K1 (16 x N1 x 1.5 x K1) |
| 0.64 |  | 5 | 17.92x N1 x K1 (28 x N1 x K1) | 1.28 x N1 x K1 (2 x N1 x K1) | 5.12 x N1 x K1 (8 x N1 x K1) |
| 1.28 |  | 4 | 32 x N1 x K1 (25 x N1 x K1) | 1.28 x N1 x K1 (1 x N1 x K1) | 6.4 x N1 x K1 (5 x N1 x K1) |
| 2.56 |  | 3 | 58.88 x N1 x K1 (23 x N1 x K1) | 2.56 x N1 x K1 (1 x N1 x K1) | 7.68 x N1 x K1 (3 x N1 x K1) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. Note 2: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the not-at-cell edge criterion. |  |  |  |  |  |

4.2.2.9.4 	Measurements for UE fulfilling low mobility and not-at-cell edge criterion

This clause contains requirements for measurements on intra-frequency NR cells provided that:

- T331 timer is not running for EMR measurements on inter-frequency NR carrier, and

- UE is configured with both low mobility criterion and not-at-cell edge criterion, and

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, and when both the criteria of low mobility and not-at-cell-edge are fulfilled, regardless whether combineRelaxedMeasCondition [2] is configured or not,

- UE may not search for and measure inter-frequency layers of equal or lower priority.

- If highPriorityMeasRelax [2] is configured, UE may choose not search for and measure inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60.

- Otherwise, the UE shall search for and measure inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, and when both the criteria of low mobility and not-at-cell-edge are fulfilled, regardless whether combineRelaxedMeasCondition [2] is configured or not,

- UE may not search for and measure inter-frequency layers of equal or lower priority.

- If highPriorityMeasRelax [2] is configured, UE may choose not search for and measure inter-frequency layers of higher, equal or lower priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60.

- Otherwise,

    - the UE shall search for and measure inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

    - UE may not search for and measure inter-frequency layers of equal or lower priority.

<< End of Change #2>>

<< Start of Change #3>>

4.2.2.10 	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion

4.2.2.10.1 	Introduction

This section contains the requirements for measurements on inter-RAT E-UTRAN cells when the UE is configured with any of following relaxed measurement critera:

- Relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.X.1 in [1],

- Relaxed measurement criterion for UE not-at-cell edge defined in clause 5.2.4.X.2 in [1],

- Both low mobility criterion and not-at-cell edge criterion as defined in clauses 5.2.4.X.1 and 5.2.4.X.2 in [1] respectively.

4.2.2.10.2 	Measurements for UE fulfilling low mobility criterion

This clause contains requirements for measurements on inter-RAT E-UTRAN cells provided that:

- T331 timer is not running for EMR measurements on inter-RAT E-UTRAN, and

- UE is configured with low mobility criterion, or

- UE is configured with both low mobility criterion and not-at-cell edge criterion, but has fulfilled only the low mobility criterion.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ,

- UE may not search for and measure inter-RAT frequency layers of equal or lower priority.

- If the UE is configured with highPriorityMeasRelax [2] then the UE shall search for inter-RAT frequency layers of higher priority at least every K2*Thigher_priority_search seconds where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60.

- If the UE is not configured with highPriorityMeasRelax [2] then the UE shall search for inter-RAT frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-RAT frequency layers of higher, equal or lower priority in preparation for possible reselection. The requirements defined in clause 4.2.2.4 apply for this section except that:

-	Tdetect,EUTRAN as specified in Table 4.2.2.10.2-1.

-	Tmeasure,EUTRAN as specified in Table 4.2.2.10.2-1.

-	Tevaluate,EUTRAN as specified in Table 4.2.2.10.2-1.

Table 4.2.2.10.2-1: Tdetect,EUTRAN, Tmeasure,EUTRAN, and Tevaluate,EUTRAN

| DRX cycle length [s] | Tdetect,EUTRAN [s] (number of DRX cycles) | Tmeasure,EUTRAN [s] (number of DRX cycles) | Tevaluate,EUTRAN [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 x K1 (36 x K1) | 1.28 x K1 (4 x K1) | 5.12 x K1 (16 x K1) |
| 0.64 | 17.92 x K1 (28 x K1) | 1.28 x K1 (2 x K1) | 5.12 x K1 (8 x K1) |
| 1.28 | 32 x K1 (25 x K1) | 1.28 x K1 (1 x K1) | 6.4 x K1 (5 x K1) |
| 2.56 | 58.88 x K1 (23 x K1) | 2.56 (1 x K1) | 7.68 x K1 (3 x K1) |
| Note 1: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the low mobility. |  |  |  |

4.2.2.10.3 	Measurements for UE fulfilling with not-at-cell edge criterion

This clause contains requirements for measurements on inter-RAT E-UTRAN cells provided that:

- T331 timer is not running for EMR measurements on inter-RAT E-UTRAN, and

- UE is configured with not-at-cell edge criterion, or

- UE is configured with both low mobility criterion and not-at-cell edge criterion, but UE has fulfilled only the not-at-cell edge criterion

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ,

- UE may not search for and measure inter-RAT frequency layers of equal or lower priority.

- When UE is only configured with not-at-cell edge criterion, and it is fulfilled, then the UE shall search for inter-RAT frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 regardless of whether the UE is configured with highPriorityMeasRelax [2] or not.

- When UE is configured with both low mobility criterion and not-at-cell edge criterion, if combineRelaxedMeasCondition [2] is not configured and only the criteria of not-at-cell-edge is fulfilled, the UE shall search for and measure inter-RAT frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 regardless of whether the UE is configured with highPriorityMeasRelax [2] or not.

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-RAT frequency layers of higher, equal or lower priority in preparation for possible reselection. The requirements defined in clause 4.2.2.3 apply for this section except that:

-	Tdetect,EUTRAN as specified in Table 4.2.2.10.3-1.

-	Tmeasure,EUTRAN as specified in Table 4.2.2.10.3-1.

-	Tevaluate,EUTRAN as specified in Table 4.2.2.10.3-1.

Table 4.2.2.10.3-1: Tdetect,EUTRAN, Tmeasure,EUTRAN, and Tevaluate,EUTRAN

| DRX cycle length [s] | Tdetect,EUTRAN [s] (number of DRX cycles) | Tmeasure,EUTRAN [s] (number of DRX cycles) | Tevaluate,EUTRAN [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 11.52 x K1 (36 x K1) | 1.28 x K1 (4 x K1) | 5.12 x K1 (16 x K1) |
| 0.64 | 17.92 x K1 (28 x K1) | 1.28 x K1 (2 x K1) | 5.12 x K1 (8 x K1) |
| 1.28 | 32 x K1 (25 x K1) | 1.28 x K1 (1 x K1) | 6.4 x K1 (5 x K1) |
| 2.56 | 58.88 x K1 (23 x K1) | 2.56 x K1 (1 x K1) | 7.68 x K1 (3 x K1) |
| Note 1: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the low mobility. |  |  |  |

4.2.2.10.4 	Measurements for UE fulfilling low mobility and not-at-cell edge criterion

This clause contains requirements for measurements on inter-RAT E-UTRAN cells provided that:

- T331 timer is not running for EMR measurements on inter-RAT E-UTRAN, and

- UE is configured with both low mobility criterion and not-at-cell edge criterion, and has also fulfilled both criteria.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, and when both the criteria of low mobility and not-at-cell-edge are fulfilled, regardless whether combineRelaxedMeasCondition [2] is configured or not,

- UE may not search for and measure inter-RAT frequency layers of equal or lower priority.

- If highPriorityMeasRelax [2] is configured, UE may choose not search for and measure inter-RAT frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60.

- Otherwise, the UE shall search for and measure inter-RAT frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, and when both the criteria of low mobility and not-at-cell-edge are fulfilled, regardless whether combineRelaxedMeasCondition [2] is configured or not,

- UE may not search for and measure inter-RAT frequency layers of equal or lower priority.

- If highPriorityMeasRelax [2] is configured, UE may choose not search for and measure inter- RAT frequency layers of higher, equal or lower priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60.

- Otherwise,

    - the UE shall search for and measure inter-RAT frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

    - UE may not search for and measure inter-RAT frequency layers of equal or lower priority.

<< End of Change #3>>