3GPP TSG-RAN WG4 Meeting # 96-e 	R4-2009829

Electronic Meeting, 17-28 Aug., 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.886 | CR | 0002 | rev | - | Current version: | 16.0.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Switching period for NR V2X in ITS band |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL |  |  |  |  |  | Date: |  |  | 2020-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The time mask for TDM operation between NR SL and LTE SL at n47 should be introduced in 38.886. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the time mask for TDM operation between NR V2X and LTE V2X in section 8.1.7.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The time mask for TDM operation between NR SL and LTE SL would be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

8.1.7.3		Additional Time mask for TDM operation between NR SL and LTE SL at n47

When a NR V2X UE is operated with TDM between NR SL and LTE SL at n47 without dual PA capability, the maximum UL switching time is defined as 150 us and SL reception interruption is allowed during UL switching time masks in Figure 8.1.7.3-1 and Figure 8.1.7.3-2 shall apply.

RAN4 will not specify the RF specific requirements during the switched period.

Figure 8.1.7.3-1: NR V2X to LTE V2X switching time mask at n47 without dual PA capability

Figure 8.1.7.3-1: LTE V2X to NR V2X switching time mask at n47 without dual PA capability

<End of Change 1>