3GPP TSG-RAN WG4 Meeting #96-e 	R4-2009855

Electronic Meeting, 17-28 Aug., 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	On NR Rel-16 HST BS demodulation UL timing adjustment requirements and simulation results

Agenda item:	7.15.3.2.3

Document for:	Discussion

Introduction

During the last RAN4#95-e meeting, some progress was made on the topic of NR Rel-16 HST BS demodulation UL requirements [1].

Remaining issues are captured in the corresponding WF [2]. The major open topics being:

- Additional scenario X.

- Manufacturer declarations w.r.t. scenario X.

- Organization of HST requirements for UL TA 500kph in specifications.

We remind that the general manufacturer declaring for UL TA were agreed along with the PUSCH declarations [2].

In this contribution we will express our views on the captured open issues and open new discussions, if necessary.
Additionally, we include the results of our various simulation campaigns directly in this Tdoc.

Discussion on open issues

Here we discuss open issues, left over from the last meeting.

UL TA additional scenario X

This discussion was relatively controversial in the last meeting [2]:

- Additional scenario “X” - Option 1: Specify requirements for scenario X. - Option 2: Do not specify scenario “X”. - No consensus, continue discussion. Companies are encourage to bring more analysis the necessity and un-necessity of introducing this test cases and make decisions in Q3 2020. - Scenario “X” implicit test passing - Discuss after additional scenario “X” introduction is decided.

We previously observed that the agreed 70% relative TPUT KPI does not represent any challenge to any implementation [6]. Hence, we don’t think that much is lost by not testing the 120kph scenario. Though, we would still prefer to have the test case independent of the HST scenario, as UL TA was not actually tested in Rel-15.

UL TA is never tested outside the HST context.

RAN4 to introduce UL TA scenario X.

UL TA manufacturer declarations w.r.t. scenario X

The same observations and arguments as in the previous section are applicable.

RAN4 to introduce UL TA scenario X, which is to be tested independent of the HST support declaration.

UL TA organization of requirements for 500kph in specifications

The UL TA specification is the last remaining topic with uncertainty w.r.t. specification writing [2]:

- Organization of HST requirements for UL TA 500kph in specifications - Option 1: Requirements for different scenarios captured in same table. - Option 2: Requirements for different scenarios captured in separate tables. - Option 3: Capture the 500kph UL TA scenario in the same table as the 350kph UL TA scenario.

Ultimately only option 1 and option 3 had proponents in the last meeting [1] and both options are viable in our opinion.
However, option 1 would make the addition of further scenarios easier in the future.

Both option 1 and 3 are viable, but option 1 is slightly preferred.

Simulation results

We include our simulation results for this meeting hereunder.

Parameters

Unless otherwise stated in the following, the simulation setup follows the HST UL TA baseline as last outlined in [3] and amended with agreements in [4], and is now captured in the specification as [5]:

Table 1: UL TA HST baseline configuration

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Uplink-downlink allocation for TDD |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| Channel bandwidth |  | 15 kHz SCS: 10 MHz 30 kHz SCS: 40 MHz |
| MCS |  | 16 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | DM-RS position (l0) | 2 |
|  | Additional DM-RS position | pos2 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 for moving UE NID0=1, nSCID =1 for stationary UE |
| Time domain resource assignment | PUSCH mapping type | Both A and B |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 10MHz CBW: 25 RB for each UE 40MHz CBW: 50 RB for each UE |
|  | Starting PRB index | Moving UE: 0 Stationary UE: 25 for 10 MHz CBW, and 50 for 40 MHz CBW |
|  | Frequency hopping | Disabled |
| SRS resource allocation | Slots in which sounding RS is transmitted (Note 1) | For FDD: slot #1 in radio frames For TDD: - last symbol in slot #3 in radio frames for 15KHz - last symbol in slot #7 in radio frames for 30KHz |
|  | SRS resource allocation | 15 kHz SCS: CSRS = 11, BSRS =0, for 40 RB 30 kHz SCS: CSRS = 21, BSRS =0, for 80 RB |
| Note 1. The transmission of SRS is optional. And the transmission comb and SRS periodic are configured as KTC = 2, and TSRS = 10 respectively. |  |  |

The specific scenarios (X, Y, Z) are then defined as follows:

Table 2: UL TA HST baseline scenarios

| Parameter | Scenario X | Scenario Y | Scenario Z |
| --- | --- | --- | --- |
| Channel model | Stationary UE: AWGN Moving UE: TDLC300-400 | Stationary UE: AWGN Moving UE: AWGN | Stationary UE: AWGN Moving UE: AWGN |
| UE speed | 120 km/h | 350 km/h | 500 km/h |
| CP length | Normal |  |  |
| A | 15 kHz: 10 s 30 kHz: 5 s | 15kHz: 10 µs 30kHz: 5 µs | 15 kHz: 10 s 30 kHz: 5 s |
| Δω | 15 kHz: 0.04 s-1 30 kHz: 0.08 s-1 | 15kHz: 0.13 s-1 30kHz: 0.26 s-1 | 15 kHz: 0.18 s-1 30 kHz: 0.36 s-1 |

Scenario Z

Here are our simulation results for the newly introduced scenario Z cases:

Table 3: PUSCH UL TA, 500kph, CP-OFDM, 1T2R, Scenario Z: simulation results summary

| PUSCH transmission | SRS transmission | CBW | SCS | DMRS | l0 | MCS | A | Δω | PUSCH mapping type | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Ideal, 70% TPUT |  |
| FDD | Disabled | 10MHz | 15KHz | 1+1+1 | 2 | 16 | 16us | 0.18s-1 | Type A | 5.70 |  |
|  |  | 10MHz | 15KHz | 1+1+1 | 0 | 16 | 16us | 0.18s-1 | Type B | 5.69 |  |
|  | Disabled | 40MHz | 30KHz | 1+1+1 | 2 | 16 | 5us | 0.36s-1 | Type A | 5.67 |  |
|  |  | 40MHz | 30KHz | 1+1+1 | 0 | 16 | 5us | 0.36s-1 | Type B | 5.67 |  |

Conclusion

In this contribution we have provided our views on various open UL TA HST issues. In particular, the addition of scenario X and corresponding additional manufacturer declarations, as well as, the specification organization.
We have made the following observations and proposals:

UL TA additional scenario X

- UL TA is never tested outside the HST context.

- RAN4 to introduce UL TA scenario X.

UL TA manufacturer declarations w.r.t. scenario X

RAN4 to introduce UL TA scenario X, which is to be tested independent of the HST support declaration.

UL TA organization of requirements for 500kph in specifications

Both option 1 and 3 are viable, but option 1 is slightly preferred.

- References

- R4-2009042, Email discussion summary for [95e][322] NR_HST_Demod_BS, Moderator (Nokia, Nokia Shanghai Bell), RAN4#95-e.

- R4-2008821, WF on Rel-16 NR HST PUSCH BS demodulation requirements, Nokia, Nokia Shanghai Bell, RAN4#95-e.

- R4-1915886, Way forward on NR HST PUSCH demodulation requirements, Huawei, HiSilicon, RAN4#93.

- R4-2002405, WF on Rel-16 NR HST BS demodulation requirements, Nokia, Nokia Shanghai Bell, RAN4#94-e.

- TS 38.104, V16.4.0, NR; Base Station (BS) radio transmission and reception.

- R4-2003895, On NR Rel-16 HST BS demodulation UL timing adjustment requirements, Nokia Shanghai Bell, RAN4#94-bis-e.