3GPP TSG-RAN WG4 Meeting #96-e 	R4-2009857

Electronic Meeting, 17-28 Aug., 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	On NR Rel-16 high reliability and low latency BS demodulation requirements and simulation results

Agenda item:	7.8.1.2.3

Document for:	Discussion

Introduction

In the RAN4#95-e meeting, good progress was made concerning ultra-low BLER BS demodulation requirements [1]. In particular the TDRA and FDRA setting for both high reliability and low latency tests were almost completely agreed upon.

Some remaining issues are captured in the corresponding WF [2]. The major open topics being:

- HR:

- PUSCH aggregation factor for 15 kHz SCS.

- Whether to clarify the safety statement.

- Test applicability rule for FR1 and FR2 if BS declare to support both FR1 and FR2.

- LL:

- Test applicability rule for FR1 and FR2 if BS declare to support both FR1 and FR2.

In this contribution we will express our views on the captured open issues and open new discussions, if necessary.

Additionally, we include the results of our various simulation campaigns directly in this Tdoc.

Discussion on open high reliability requirement issues

Here we discuss open issues, left over from the last meeting.

PUSCH aggregation factor for 15 kHz SCS

In RAN4#95-e, no conclusion could be reached on the aggregation factor to be employed, when using the 3D1S1U TDD pattern (i.e., 15 kHz SCS) [2]:

- PUSCH aggregation factor for 15 kHz SCS: - Option 1: No requirement for 15 kHz SCS. - Option 2: Only configure n2 for FDD and no requirement for TDD. - Option 3: Configure n2 for FDD and n8 for TDD with note. - Note: The intention of this configuration is to have two effective transmissions of the transport block. To achieve this for the standard TDD pattern captured in this table, a value of n8 is necessary, while for FDD a value of n2 is necessary.

From our understanding of the email discussions [1], both proponents and opponents of “n8” are well aware that a configuration of “n2 or n4” would result in a single repetition of the PUSCH slot.
The background of this circumstance was (e.g.) explained in our last discussion paper [3]:

TDD pattern: DDDSU AggregationFactor: n4 pusch-AggregationFactor ENUMERATED {n2,n4,n8} PUCCH-FormatConfig > nrofSlots ENUMERATED {n2,n4,n8}

Observing that the text/use case of this requirement is focused on high reliability, we see it necessary to choose a PUSCH aggregation that increases the reliability, i.e., that results in repeated TBs.
Optimized implementations might use the aggregation for performance improvement (over HARQ), even in cases where the coherence is low between the repetitions.

Optimized implementations might use the aggregation for performance improvement (over HARQ), even in cases where the coherence is low between the repetitions.

RAN4 to chose to configure n2 for FDD and n8 for TDD with note.

Whether to clarify the safety statement

RAN4#95-e captured the open issue as follows [2]:

- Whether to clarify the safety statement - Option 1: No need to specify any safety statements in specification - Option 2: Yes - Option 1a: Since the URLLC features of 5G NR will potentially be used in safety critical applications, the ultimately chosen statistical testing methodology for testing of these features must be verified by an independent body of experts/statisticians, before requirements and test can be used as basis for safety critical implementations. All statistical analysis and discussions provided in this meeting are to be taken as a best effort and is not to be taken as due diligence - Option 1b: If the URLLC features of 5G NR would be used in safety or mission critical applications, the ultimately chosen statistical testing methodology for testing of these features must be verified by an independent body of experts/statisticians. It is also important to bear in mind that the demodulation requirements do not take account of all aspects of system operation (for example RF, transmitter, internal interfaces, higher layer protocol software etc.).

As usual, we would like to clarify that the passing of RAN4 performance requirements alone is not sufficient grounds to use URLLC features in safety critical environments.
A straightforward decision of the URLLC group on this matter would be appreciated in this meeting, so we can stop discussing this matter at each meeting.

RAN4 to take a recorded majority decision on to inclusion of a safety statement clarification.

Test applicability rule for FR1 and FR2 if BS declare to support both FR1 and FR2

RAN4#95-e captured the open issue as follows [2]:

- Test applicability rule for FR1 and FR2 if BS declare to support both FR1 and FR2: - Option 1: Tests shall be done for either FR1 or FR2 - Option 2: Tests shall be done for both , and only 1 SCS will be tested for each frequency band with test applicability rule. - Option 3: Tests shall be done for both, all supported SCS will be tested for FR1 and only 1 SCS need to be tested for FR2. - SCS/BW for FR2 (only if FR2 is defined) - 60 KHz: - Option 1: 50 MHz - Option 2: 50 MHz and 100 MHz - 120 KHz - Option 1: 100 MHz - Option 2: 50 MHz and 100 MHz

As we argued in the ultra-low BLER agenda item, it is our observation that FR2 and high reliability do not make a very practical use case.
However, here the reliability target (1% BLER) is much closer to achievable values, hence our opinion is much less strict:

FR2 and high reliability do not make a very practical use case.

Discussion on open low latency requirement issues

Here we discuss open issues, left over from the last meeting.

Test applicability rule for FR1 and FR2 if BS declare to support both FR1 and FR2

RAN4#95-e captured the open issue as follows [2]:

- BS URLLC demodulation requirements for PUSCH mapping Type B - Test applicability rule for FR1 and FR2 if BS declare to support both FR1 and FR2: - Option 1: Tests shall be done for either FR1 or FR2 - Option 2: Tests shall be done for both , and only 1 SCS will be tested for each frequency band with test applicability rule. - Option 3: Tests shall be done for both, all supported SCS will be tested for FR1 and only 1 SCS need to be tested for FR2. - SCS/CBW for FR2 (only if FR2 is agreed) - 60 KHz: - Option 1: 50 MHz - Option 2: 50 MHz and 100 MHz - 120 KHz - Option 1: 100 MHz - Option 2: 50 MHz and 100 MHz

For low latency, the use case of FR2 is immediately evident.
However, we expect products implementing such a solution to be very specialized; potentially FR2 only solutions. Common hardware should not need to test both FR1 and FR2.

RAN4 to introduce low latency requirements for both FR1 and FR2, along with applicability rule that only one SCS will be tested for each frequency band with test applicability rule.

Simulation results high reliability

Parameters

The WF contains a great summary of the basic simulation assumptions [2]:

Table 1: Simulation parameters.

| Parameter |  | Value |
| --- | --- | --- |
| Frequency range |  | FR1 |
| Transform precoding |  | Disabled |
| Antenna configuration |  | 1x2, ULA Low |
| PUSCH configuration | Mapping type | Type A and Type B |
|  | Starting symbol (S) | 0 |
|  | Length (L) | 14 |
|  | PUSCH aggregation factor | n2 for 30 kHz SCS FFS 15 kHz SCS |
| PUSCH DMRS configuration | DMRS Type | Type 1 |
|  | DM-RS duration | Single-symbol DM-RS |
|  | Number of additional DMRS | 1 |
| Propagation condition |  | TDLB100-400 |
| MCS Table |  | Table 3, MCS 5 |
| SCS and BW |  | 15 kHz for 5/10 MHz 30 kHz for 10/40 MHz |
| Frequency domain resource |  | Full Bandwidth |
| TDD pattern |  | 15 kHz SCS: 3D1S1U, S=10:2:2 30 kHz SCS: 7D1S2U, S=6:4:4 |
| Maximum number of HARQ transmissions |  | 4 |
| Testing metric |  | Target BLER: 10-2 (Calculate the target BLER after all transmission) |

Results

Table 2: PUSCH BS demod high reliability testing simulation results.

| MCS level | SCS/CBW | Aggregation Factor | Mapping Type | SNR @ target BLER 1e-2 (dB) |
| --- | --- | --- | --- | --- |
| Table 3, MCS5 | 15 kHz, 10 MHz | n1 | TypeA | -10.2 |
|  |  |  | TypeB | -9.9 |
|  |  | n2 | TypeA | -13.4 |
|  |  |  | TypeB | -13.2 |

Remark: We see a slight difference between Type A and B PUSCH TDRA, hence we don’t feel comfortable to combine both into a single field, as requested by the template.

Simulation results low latency

Parameters

The WF contains a great summary of the basic simulation assumptions [2]:

Table 3: Simulation parameters.

| Parameter |  | Value |
| --- | --- | --- |
| Frequency range |  | FR1 |
| Transform precoding |  | Disabled |
| Antenna configuration |  | 1x2, ULA Low |
| PUSCH configuration | PUSCH mapping type | Type B |
|  | Start symbol | 0 |
|  | Allocation length | 2 |
|  | PUSCH aggregation factor | 1 |
| PUSCH DMRS configuration | DMRS Type | Type 1 |
|  | DM-RS duration | Single-symbol DM-RS |
|  | Number of additional DMRS | 0 |
| SCS and BW |  | 15 kHz for 5/10 MHz 30 kHz for 10/40 MHz |
| TDD pattern |  | 15 kHz SCS: 3D1S1U, S=10:2:2 30 kHz SCS: 7D1S2U, S=6:4:4 |
| MCS Table |  | Table 3, MCS 10 |
| Propagation condition |  | TDLC300-100 Low |
| Maximum number of HARQ transmissions |  | 1 |
| Frequency domain resource assignment |  | Full bandwidth For MCS10 |
| Test metric |  | 70% TP |

Results

Table 4: PUSCH BS demod low latency testing simulation results.

| Test | Symbol length | waveform | Tx/Rx | PUSCH mapping type | SCS and BW | MCS level | Propagation condition | DMRS | Nokia |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Ideal | Impairment |
| Test 2.1 | 2os | CP-OFDM | 1T2R | type B | 15kHz, 10MHz | Table 3, MCS10 | TDLC300-100 | DMRS 1+0 | -2.3 | 0.2 |
| Test 2.2 | 2os | CP-OFDM | 1T2R | type B | 30kHz, 40MHz | Table 3, MCS10 | TDLC300-100 | DMRS 1+0 | -2.5 | 0 |

Conclusion

In this contribution we have provided our views on various open high reliability and low latency (e)URLLC issues. In particular, PUSCH aggregation factors, safety statements, and test applicability rules (esp. for FR2). Additionally, we have presented the results of our simulation campaigns.
We have made the following observations and proposals:

PUSCH aggregation factor for 15 kHz SCS

- Optimized implementations might use the aggregation for performance improvement (over HARQ), even in cases where the coherence is low between the repetitions.

- RAN4 to chose to configure n2 for FDD and n8 for TDD with note.

Whether to clarify the safety statement

RAN4 to take a recorded majority decision on to inclusion of a safety statement clarification.

Test applicability rule for FR1 and FR2 if BS declare to support both FR1 and FR2 (high reliability)

FR2 and high reliability do not make a very practical use case.

Test applicability rule for FR1 and FR2 if BS declare to support both FR1 and FR2 (low latency)

RAN4 to introduce low latency requirements for both FR1 and FR2, along with applicability rule that only one SCS will be tested for each frequency band with test applicability rule.

- References

- R4-2008888, Email discussion summary for [95e][318] NR_L1enh_URLLC_Demod_Part2, Moderator (Huawei, HiSilicon), RAN4#95-e.

- R4-2008810, Way forward on NR URLLC BS performance requirements, Huawei, HiSilicon, RAN4#95-e.

- R4-2006062, On NR Rel-16 relaxed high reliability and low latency BS demodulation requirements, Nokia, Nokia Shanghai Bell, RAN4#95-e.