3GPP RAN WG4 Meeting #96-e		R4-2009942

Electronic meeting, 17th – 28th August 2020

Agenda item:	7.1.2.1

Source:	Apple Inc.

Title:	NR-U MPR for PC5

WI/SI:	NR_unlic-Core

Release:	Rel-16

Document for:	Decision

# 1	Introduction

This contribution presents simulation results for NR-U with a 5GHz WiFi PA model. The simulations were conducted according to the latest agreements. A proposal is made for single CC MPR.

# 2	Discussion

Simulations have been done for NR-U single carrier. The simulations model was obtained from a 5GHz WiFi PA. The PA was calibrated with the agreed QPSK DFT-s-OFDM waveform in [1]. The detailed list of the setup is provided in the following:

- Power Class: PC5

- Calibration: QPSK DFT-s-OFDM 100RB3 with 1dB backoff

- ACLR: 27dB

- SEM: As agreed in [2]

- EVM: PA+Image budget used

- Image rejection: -28dBc

- Carrier leakage rejection: -28dBc

The PA was simulated with the listed impairments and the results evaluated with the given spectral requirements. The 0dB reference was obtained by finding the maximum inband power with 1MHz aggregation bandwidth. The mask was applied accordingly. Before identifying the reference level, the carrier leakage was removed to avoid an offset in the     NR-U SEM mask. Simulations were conducted for full carrier and interlace operation. The waveform setups are found in Table1. All setups were simulated with QPSK, 16QAM, 64QAM and 256QAM. The results of the simulations are provided in Figure1.

|  | ID | Waveform | BW | RB Setup | SCS |
| --- | --- | --- | --- | --- | --- |
| Full Allocation Single CC | 1 | DFT-s-OFDM | 20 | 100RB3 | 15 |
|  | 2 | DFT-s-OFDM | 20 | 100RB0 | 15 |
|  | 3 | DFT-s-OFDM | 20 | 50RB0 | 30 |
|  | 4 | DFT-s-OFDM | 40 | 216RB0 | 15 |
|  | 5 | DFT-s-OFDM | 80 | 216RB0 | 30 |
|  | 6 | CP-OFDM | 20 | 106RB0 | 15 |
|  | 7 | CP-OFDM | 20 | 51RB0 | 30 |
|  | 8 | CP-OFDM | 40 | 216RB0 | 15 |
|  | 9 | CP-OFDM | 80 | 217RB0 | 30 |
| Interlaced Allocation Single CC | 10 | DFT-s-OFDM | 20 | 1RB0 every 10RBs (10x) | 15 |
|  | 11 | DFT-s-OFDM | 40 | 1RB0 every 10RBs (20x) | 15 |
|  | 12 | DFT-s-OFDM | 80 | 1RB0 every 5RBs (40x) | 30 |
|  | 13 | CP-OFDM | 20 | 1 RB0 every 10RBs (10x) | 15 |
|  | 14 | CP-OFDM | 40 | 1RB0 every 10RBs (22x) | 15 |
|  | 15 | CP-OFDM | 80 | 1RB0 every 5RBs (44x) | 30 |

Table1: Signal Setup

Figure1: Simulation results

Summing up the simulation a proposal for NR-U MPR is drafted and can be found in Table2.

| Modulation |  | MPR (dB) |  |
| --- | --- | --- | --- |
|  |  | Full RB allocations | Partial RB allocations |
| DFT-s-OFDM | PI/2 BPSK | 1.5 | 2.0 |
|  | QPSK | 1.5 | 2.5 |
|  | 16 QAM | 2.0 | 3.0 |
|  | 64 QAM | 3.5 | 3.5 |
|  | 256 QAM | 5.0 | 5.0 |
| CP-OFDM | QPSK | 3.5 | 3.5 |
|  | 16 QAM | 4.0 | 4.0 |
|  | 64 QAM | 5.0 | 4.5 |
|  | 256QAM | 7.0 | 6.5 |

Table2: NR-U MPR

Proposal: Define MPR for NR-U Single Carrier according to Table2.

# 3	Conclusions

This paper contributes to the NR-U MPR discussion. Several simulations for single carrier have been conducted and the following proposal is made.

| Modulation |  | MPR (dB) |  |
| --- | --- | --- | --- |
|  |  | Full RB allocations | Partial RB allocations |
| DFT-s-OFDM | PI/2 BPSK | 1.5 | 2.0 |
|  | QPSK | 1.5 | 2.5 |
|  | 16 QAM | 2.0 | 3.0 |
|  | 64 QAM | 3.5 | 3.5 |
|  | 256 QAM | 5.0 | 5.0 |
| CP-OFDM | QPSK | 3.5 | 3.5 |
|  | 16 QAM | 4.0 | 4.0 |
|  | 64 QAM | 5.0 | 4.5 |
|  | 256QAM | 7.0 | 6.5 |

Table2: NR-U MPR

Proposal: Define MPR for NR-U Single Carrier according to Table2.

# 4	References

- R4-2005221, “WF on NR-U UE Tx Requirement”, RAN4#94-bis-e, Skyworks Solutions Inc, Nokia, Ericsson

- R4-1915979, “WF on NR-U spectrum emission mask”, RAN4#93, Nokia, CableLabs, Charter

- R4-2008436, “WF on NR-U MPR”, RAN4#95-e, MediaTek Inc., Skyworks Solution Inc.