3GPP TSG-RAN4 Meeting #96-e	R4-2009954

# Online,  17th Aug 2020 - 28th Aug 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-01 | CR | 0020 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to capture WRC-19 impact on NR bands in TR38.817-01 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The technical background and outcome of the RAN4 discussion of the impact of the WRC-19 decision on FR2 band requirements needs to be captured in the Rel-15 technical report on NR RF to provide a convenient reference. RAN4 reached a common understanding of all applicable regulatory requirements, the necessary A-MPR to comply with these requirements, and the NS signaling structure over multiple working group meetings. This summary collects all relevant agreements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1: introduce a reference to the WRC-19 decision to Clause 2 Change 2: introduce the relevant technical background on the common understanding of all applicable regulatory requirements, the necessary A-MPR to comply with these requirements, and the NS signaling structure to the new Clause 7.5.4.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The impact of the WRC-19 decision on RAN4 RF specifications will not be clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.5.4.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The text proposal in this CR was drafted based on the following RAN4 contributions: R4-2000091, R4-2005738, R4-2006788, R4-2006789, R4-2006329, R4-2009141. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< start of change 1 >>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Recommendation ITU-R M.1036-5 (10/2015), “Frequency arrangements for implementation of the terrestrial component of International Mobile Telecommunications (IMT) in the bands identified for IMT in the Radio Regulations (RR)”.

[3]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[4]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[5]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[6]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[7]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[8]	3GPP TS 38.211: "NR; Physical channels and modulation".

[9]	3GPP TR 36.942: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios".

[10]	3GPP TR 38.803: "Study on new radio access technology: Radio Frequency (RF) and co-existence aspects".

[11]	Void.

[12]	47 CFR Part 30, "UPPER MICROWAVE FLEXIBLE USE SERVICE, §30.202   Power limits", FCC.

[13]	ITU Provisional Final Acts WRC-19,  Resolution 750 (REV.WRC-19), Table 1-1 FN 5 (a) and (b), page 349 (https://www.itu.int/dms_pub/itu-r/opb/act/R-ACT-WRC.13-2019-PDF-E.pdf)

<< end of change 1 >>

<< start of change 2 >>

## 7.5	Output RF spectrum emissions

### 7.5.1	Occupied bandwidth

### 7.5.2	Spectrum emission mask

### 7.5.3	Adjacent Channel Leakage ratio

### 7.5.4	Spurious emission

#### 7.5.4.1	Protection of earth exploration-satellite service

The International Telecommunication Union World Radiocommunication Conference 2019 (WRC-19) created an internationally recognized 24.25 to 27.5 GHz band aiming at primary mobile allocation for licensed/IMT networks deployments. In addition to it, the limits of unwanted emission power from active service stations in that band were specified to protect 23.6-24 GHz EESS (passive) band.

A summary of the WRC-19 agreement related to EESS unwanted emission limits is given below [13]:

For 5G Base stations:

-	-33 dBW in any 200 MHz of the EESS (passive) band for IMT/5G base stations brought into use before 1st September 2027.

-	-39 dBW in any 200 MHz of the EESS (passive) band for IMT/5G base stations brought into use after 1st September 2027.

-	The -39 dBW/200 MHz limit will not apply to 5G base stations which have been brought into use prior to this date.

-	For those base stations, the limit of -33 dBW/200 MHz will continue to indefinitely apply after this date.

For 5G User equipment:

-	-29 dBW in any 200 MHz of the EESS (passive) band for IMT/5G user equipment brought into use before 1st September 2027.

-	-35 dBW in any 200 MHz of the EESS (passive) band for IMT/5G user equipment brought into use after 1st September 2027.

-	The -35 dBW/200 MHz limit will not apply to UEs which have been brought into use prior to this date.

-	For those UEs, the limit of -29 dBW/200 MHz will continue to indefinitely apply after this date.

NOTE 1: these out-of-band emission limits are equally applicable to all license holders within the band.

The relation between "active service band" and 3GPP bands is illustrated in Figure 7.5.4.1-1 below. NR band n258 frequency ranges from 24.25 to 27.5 GHz, which is identical to the aforementioned WRC active service band. Furthermore, 3GPP band n257 partially overlaps with "active service band", and thus ideally should be also considered while analysing potential specification impact.

Figure 7.5.4.1-1: EESS passive band, active service band, and 3GPP bands n257 and n258.

Combining the WRC-19 outcome with the existing spurious emissions requirements in TS38.101-2, Table 7.5.4.1-1 summarizes the common understanding of emissions requirements globally and regionally.

Table 7.5.4.1-1:

| Region | Requirement 1 | Requirement 2 | Requirement 3 | Requirement 4 | Requirement 5 | Requirement 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Global, US, Japan | Protected range: spurious Band applicability: all Limit: -13 dBm/MHz | Protected range: 23.6 - 24.0 GHz Band applicability: n258, n257 Limit: +1 dBm/200 MHz | Protected range: 23.6 - 24.0 GHz Band applicability: n258, n257 Limit: -5 dBm/200 MHz | N/A | Protected range: 36.0 to 37.0 GHz Band applicability: n260, n259 Limit: +7 dBm/1000 MHz | Protected range: 36.0 to 37.0 GHz Band applicability: n260, n259 Limit: -13 dBm/1 MHz |
| Europe |  |  |  | Protected range: 7.25 GHz ≤ f ≤ 2nd harmonic Band applicability: all bands Limit: -10 dBm/100 MHz |  |  |

The A-MPR needed to comply with these requirements is summarized in Table 7.5.4.1-2.

Table 7.5.4.1-2: Summary of A-MPR needed to comply with the regulatory requirements in Table 7.5.4.1-1

| Band | Requirement | A-MPR for PC1 | A-MPR for PC2 | A-MPR for PC3 | A-MPR for PC4 |
| --- | --- | --- | --- | --- | --- |
| n258 | (Case A) Protected range: 23.6 - 24.0 GHz Limit: +1 dBm/200 MHz | 3.0 if Offset frequency < BWchannel, 0.0 otherwise | 0 | 0 | 0 |
| n257 |  | 0 (absorb as general requirement) | 0 (absorb as general requirement) | 0 (absorb as general requirement) | 0 (absorb as general requirement) |
| n258 | (Case B) Protected range: 23.6 - 24.0 GHz Limit: -5 dBm/200 MHz | 7.0 if Offset frequency < BWchannel, 6.0 otherwise | 1.0 if Offset frequency < BWchannel, 0.0 otherwise | 1.0 if Offset frequency < BWchannel, 0.0 otherwise | 1.0 if Offset frequency < BWchannel, 0.0 otherwise |
| n257 |  |  |  |  |  |
| n260 | (Case C) Protected range: 36.0 to 37.0 GHz Limit: +7 dBm/1000 MHz and -13 dBm/1 MHz | >0 for low edge allocations (see below) | >0 for low edge allocations (see below) | >0 for low edge allocations (see below) | >0 for low edge allocations (see below) |
| n259 |  | 0 (absorb as general requirement) | 0 (absorb as general requirement) | 0 (absorb as general requirement) | 0 (absorb as general requirement) |

The A-MPR requirements can be defined in more detailed according to the Case listed in Table 7.5.4.1-1.

For Case A:

-	Protected range: 23.6 - 24.0 GHz

-	Limit: +1 dBm/200 MHz

-	For Power Class 1:

For intra-band UL CA the following rule for AMPR (dB) applies:

AMPR = AMPRCC

Where:

AMPRCC is 3.0 if Offset frequency < BWchannel, 0.0 otherwise

Offset frequency is defined as the frequency from 24.0 GHz to the lower edge of the channel

-	For Power Class 2, 3, 4:

AMPR shall be 0 dB

For Case B:

-	Protected range: 23.6 - 24.0 GHz

-	Limit: -5 dBm/200 MHz

-	For Power Class 1:

For intra-band UL CA the following rule for AMPR (dB) applies:

AMPR = AMPRCC

Where:

AMPRCC is 7.0 if Offset frequency < BWchannel, 6.0 otherwise

Offset frequency is defined as the frequency from 24.0 GHz to the lower edge of the channel

-	For Power Class 2, 3, 4:

For intra-band UL CA the following rule for AMPR (dB) applies:

AMPR = AMPRCC

Where:

AMPRCC is 1.0 if Offset frequency < BWchannel, 0.0 otherwise

Offset frequency is defined as the frequency from 24.0 GHz to the lower edge of the channel

For Case C:

-	Protected range: 36.0 - 37.0 GHz

-	Limits: 7 dBm/1000 MHz and -13 dBm/1 MHz

-	For Power Class 1:

For power class 1, AMPR shall be 0.0 dB unless the allocation meets the following conditions to qualify as a low-edge allocation:

RBstart < BWchannel/10 MHz

RBstart ≤ ⎣LCRB/2⎦

Offset frequency < 10% of BWchannel

Where:

Offset frequency is defined as the frequency from 37.0 GHz to the lower channel edge

The AMPR (dB) for low-edge allocations is shown in Table 7.5.4.1-3 below.

Table 7.5.4.1-3: A-MPR for low-edge allocations (Case C with PC1)

| PC1 | BWchan (MHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 | 100 | 200 | 400 |
| A-MPR(dB) | 16.0 | 14.0 | 7.0 | 6.0 |

-	For Power Class 2, 3, 4:

For power class 3, AMPR shall be 0.0 dB unless the allocation meets the following conditions to qualify as a low-edge allocation:

RBstart < BWchannel/10 MHz

RBstart ≤ ⎣LCRB/2⎦

Offset frequency < 10% of BWchannel

Where:

Offset frequency is defined as the frequency from 37.0 GHz to the lower channel edge

The AMPR (dB) for low-edge allocations is shown in Table 7.5.4.1-4 below.

Table 7.5.4.1-4: A-MPR for low-edge allocations (Case C with PC2, PC3, PC4)

| PC2, PC3, PC4 | BWchan (MHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 | 100 | 200 | 400 |
| A-MPR(dB) | 12.0 | 10.0 | 3.0 | 2.0 |

Because of the delayed implementation of Requirement 3 from Table 7.5.4.1-1, RAN4 will incorporate the new NS value structure in a future release, such that the timing of UEs achieving certification according to that release and the timing of the WRC-19 regulation coming into effect is aligned.

To assist such future discussions, Table 7.5.4.1-5 below captures the recommended NS value structure.

Table 7.5.4.1-5: Summary of NS values aligned with the WRC-19 outcome

|  | n257 | n258 | n259 | n260 |
| --- | --- | --- | --- | --- |
| NS_200 | 1dBm/200MHz into general |  | 7dBm/1GHz into general |  |
| NS_201 | -5dBm/200MHz | -5dBm/200MHz |  |  |
| NS_202 | -10dBm/100MHz (Harmonic) + 1dBm/200MHz | -10dBm/100MHz (Harmonic) + 1dBm/200MHz |  |  |
| NS_203 |  | 1dBm/200MHz |  |  |
| NS_204 |  |  |  | 7dBm/1GHz + -13dBm/MHz |

In order for the UE to comply with all relevant regulations associated with the FR2 bands which it supports, RAN4 should mandate that a UE designed against Rel-X shall support all NS values defined in the Rel-X version of the specification and which correspond to the bands which the UE supports.

<< end of change 2 >>