3GPP TSG-RAN4#96e	R4-2010232

17th Aug – 28th Aug 2020

Source: 	Nokia

Title: 					TP for TR 37.717.21-11: DC_2A-48A_n5A

Agenda Item: 			10.4.2

Document for:	Approval

# 1	Introduction

This is a text proposal to introduce DC_2A-48A_n5A into TR 37.717.21-11 Dual Connectivity of 2 bands LTE inter-band CA and 1 NR band.

# 2 	Text proposal

************************************Start of TP********************************************

## 5.x	DC_2-48_n5

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-48A_n5A | DC_2A_n5A DC_48A_n5A |

### 5.x.2	Co-existence studies

When uplink is DC_2A_n5A there is IMD3 interfering band 48 downlink.

When uplink is DC_48A_n5A there is IMD3 interfering band 2 downlink.

### 5.x.3	∆TIB and ∆RIB values

It is proposed to take the maximum relaxation values from DC_2-n5 and DC_48-n5.

Table 5.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-48_n5 | 2 | 0.6 |
|  | 48 | 0.8 |
|  | n5 | 0.3 |

Table 5.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-48_n5 | 2 | 0.2 |
|  | 48 | 0 |
|  | n5 | 0.5 |

### 5.x.4	Reference sensitivity exceptions

It is prosed to re-use the IMD3 MSD from already specified configurations DC_1A-5A_n78A and DC_1A-42A_n28A which are similar to DC_2A-48A_n5A.

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A-48A_n5A | 2 | 1870 | 5 | 25 | 1950 | 18.1 | IMD3 |
|  | 48 | 3610 | 10 | 50 | 3610 | N/A | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | 2 | 1890 | 5 | 25 | 1970 | N/A | N/A |
|  | 48 | 3570 | 5 | 25 | 3570 | 15.7 | IMD3 |
|  | n5 | 840 | 5 | 25 | 885 | N/A | N/A |

************************************End of TP********************************************