3GPP TSG-RAN WG4 Meeting #96-e              	R4-2010274

Electronic meeting, 17th Aug – 28th Aug, 2020

Agenda item:	10.27.1

Source:	Samsung

Title:	Discussion the test model in DSS

Document for:	Discussion

# 1	Introduction

DSS (dynamic spectrum sharing): DSS is an important feature for migration from LTE to NR. Especially for operators don’t have enough spectrum for NR operations, the DSS technology is a key to success to enhance their NR coverage using sub 6 LTE spectrum. It can allows for sharing existing spectrum between LTE and NR carriers, thus enabling smoother transmission from LTE and faster adoption of NR. To running LTE and NR within the same frequency hand, such as band 40/n40, band48/n48, and band38/n38 [1][2][3], either FDD or TDD, the related system parameters should be designed properly, such as sync rater. Meanwhile, the related test model should be considered to guarantee requirement.

In this contribution, the view on test model for DSS is presented.

# 2	Discussion

2. 1 Motivation

As per dynamic spectrum sharing, in case of NR co-exist with LTE, NR and LTE can interfere each other. When deploying NR systems in a certain LTE site, new NR DUs shall be combined with LTE DUs for the DSS.  Regarding resource block-level sharing, frequency alignment between LTE and NR is necessary to avoid inter-subcarrier interference.  Regarding resource slot-level sharing, where the NR DUs combined with LTE DUs in a time domain, NR and LTE can interfere each other at the presence of timing alignment error between NR and LTE systems.  As indicated as figure1, the NR symbols are advanced by LTE, which results in the SIR degradation seriously due to ISI. The SIR degradation can be explained as table 1 with different value of Advance

For current multi-RAT BS, the EVM and frame alignment error are tested separately. It seems the current test should be enough to guarantee the related with requirement.

While considering an ecosystem, such as open-RAN system, especially the DUs and RUs provided by different vendors, it is not clear that how RU combines NR and LTE data after IFFT operation in the time domain. In case that there is some timing error/frame error existed, the system performance will be degraded.

Figure 1 NR symbols are advanced by LTE,

| Advance (ns) | SIR (eval) |
| --- | --- |
| 50 | 31.2 |
| 100 | 29.3 |
| 150 | 26.6 |
| 200 | 25.9 |
| 250 | 24.5 |
| 300 | 23.9 |
| 350 | 23.0 |
| 400 | 22.5 |
| 450 | 21.9 |
| 500 | 21.6 |

Table 1 NR symbols are advanced by LTE

Consider there is no related test scenarios for DSS specified in RAN4, to guarantee the basic performances for more complex system performance, we see a need to define the new test models for DSS, including LTE and NR channels in the same slot or different slots, to measure the related with timing error.

Proposal 1: The test model should be introduced for DSS to measurement timing/framing alignment error

Proposal 2:  Define the minimum requirement of timing/framing alignment error for DSS scenarios

2. 2 Deployment scenario

Enabling co-existence of existing LTE deployment, there are several deployment scenario for DSS can be considered as follows:

Scenario 1 (FDM):

Figure 1: Frequency domain scenario 1 for NR/LTE DSS with different BW and same center frequency

Figure 2: Frequency domain scenario 1 for NR/LTE DSS with different BW and different center frequency

For frequency domain scenario, NR and LTE can share with different BW under the same center frequency and different center frequency.  In LTE, the related DC tone is reserved. There is tone mismatch for high frequency band due to DC empty

Scenario 2 (TDM):

For time domain scenario, NR and LTE can share different slots. Typically, there are 3 slot types for DSS, with as LTE slots, NR slots and NR slots for MBSFN subframe. As indicated in Frgure3, LTE channel/signals for test model are transmitted in LTE slot, and LTE CRS is transmitted in NR slots. For slot#5, LTE sync signals are allocated in LTE center RBs. With NR slots for MBSFN subframe, in a first symbol, LTE control channels/signals are transmitted. NR channels/signals are transmitted in the other symbols without LTE CRS.

Figure 3: Time domain scenario for NR/LTE DSS

Also, it can support with NR/LTE FDM with the same slots.

Figure 3: Time domain scenario for NR/LTE DSS

Figure 4: Time domain scenario for NR/LTE DSS

2. 3 Configuration for Test Model

Existing specification for Rel-15, different Test models for BS type 1-C and BS type 1-H are introduced to test the key aspect of BS transmitter characteristic, such as BS output power, EVM.

As for concrete test model for DSS, we suggest to reuse the existed test models as indicted in Annex, where the related physical channel parameters should be changed for fulfilling the DSS deployment scenario.

Proposal 3: Common physical channel parameters for PDCCH for BS type 1-C and BS type 1-H test models for DSS

| Parameter | Value |
| --- | --- |
| # of symbols used for control channel | 2 |
| Starting symbol number for control channel | 1 |
| # of CCEs allocated to PDCCH | 1 |
| Starting RB location for PDCCH | 0 |
| # of available REGs | 6 |
| Aggregation level | 1 |
| # of RBs not allocated for PDCCH in each symbol | NRB – 3 |
| Ratio of PDCCH EPRE to DM-RS EPRE | 0 dB |
| Boosting level of control channel | 0 dB |

Proposal 4: Common physical channel parameters for PDSCH for BS type 1-C and BS type 1-H test models for DSS

| Parameter | Value |
| --- | --- |
| Mapping type | PDSCH mapping type A |
| dmrs-TypeA-Position for the first DM-RS symbol | Pos3 |
| dmrs-AdditionalPosition for additional DM-RS symbol(s) | 1(Note) |
| dmrs-Type for comb pattern | Configuration type 1 |
| maxLength | 1 |
| Ratio of PDSCH EPRE to DM-RS EPRE | 0 dB |
| Note: The position of additional DMRS is 12 with RateMatchPattern LTE-CRS |  |

Proposal 5: Common physical channel parameters for PDSCH by RNTI for BS type 1-C and BS type 1-H test models for DSS

| Parameter | Value |
| --- | --- |
| PDSCH nRNTI=0 |  |
| Starting symbol | 1 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=1 |  |
| Starting symbol | 1 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=2 |  |
| Starting symbol | 3 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| Starting PRB location | 0 |
| Number of PRBs | 3 |

Proposal 6: Common physical channel parameter for LTE CRS and MBSFN for DSS with considering NR and LTE deployed in the same or different center frequency, respectively

| Parameter | Value |
| --- | --- |
| carrierFreqDL | FFS |
| CarrierBandwidthDL | FFS on {1,4, 3,5,10,15,20MHz} |
| NrofCRS-Ports | [1] |
| v-Shift | (LTE_PCID%6) |
| Mbsfn-SubframeConfig |  |
| RadioframeallocationPeriod | [1] |
| RadioframeallocationOffset | [0] |

# 3 Conclusion

In this contribution, the view on the Test model is discussed.

Proposal 1: The test model should be introduced for DSS to measurement timing/framing alignment error

Proposal 2:  Define the minimum requirement of timing/framing alignment error for DSS scenarios

Proposal 3: Common physical channel parameters for PDCCH for BS type 1-C and BS type 1-H test models for DSS

| Parameter | Value |
| --- | --- |
| # of symbols used for control channel | 2 |
| Starting symbol number for control channel | 1 |
| # of CCEs allocated to PDCCH | 1 |
| Starting RB location for PDCCH | 0 |
| # of available REGs | 6 |
| Aggregation level | 1 |
| # of RBs not allocated for PDCCH in each symbol | NRB – 3 |
| Ratio of PDCCH EPRE to DM-RS EPRE | 0 dB |
| Boosting level of control channel | 0 dB |

Proposal 4: Common physical channel parameters for PDSCH for BS type 1-C and BS type 1-H test models for DSS

| Parameter | Value |
| --- | --- |
| Mapping type | PDSCH mapping type A |
| dmrs-TypeA-Position for the first DM-RS symbol | Pos3 |
| dmrs-AdditionalPosition for additional DM-RS symbol(s) | 1(Note) |
| dmrs-Type for comb pattern | Configuration type 1 |
| maxLength | 1 |
| Ratio of PDSCH EPRE to DM-RS EPRE | 0 dB |
| Note: The position of additional DMRS is 12 with RateMatchPattern LTE-CRS |  |

Proposal 5: Common physical channel parameters for PDSCH by RNTI for BS type 1-C and BS type 1-H test models for DSS

| Parameter | Value |
| --- | --- |
| PDSCH nRNTI=0 |  |
| Starting symbol | 1 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=1 |  |
| Starting symbol | 1 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=2 |  |
| Starting symbol | 3 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| Starting PRB location | 0 |
| Number of PRBs | 3 |

Proposal 6: Common physical channel parameter for LTE CRS and MBSFN for DSS with considering NR and LTE deployed in the same or different center frequency, respectively

| Parameter | Value |
| --- | --- |
| carrierFreqDL | FFS |
| CarrierBandwidthDL | FFS on {1,4, 3,5,10,15,20MHz} |
| NrofCRS-Ports | [1] |
| v-Shift | (LTE_PCID%6) |
| Mbsfn-SubframeConfig |  |
| RadioframeallocationPeriod | [1] |
| RadioframeallocationOffset | [0] |

# References

- [1] RP- 192427 WID on LTE/NR spectrum sharing in band 48/n48 frequency range

- [2] RP-201362 New WI proposal: LTE/NR spectrum sharing in Band 40/n40

- [3] RP-201314 New WI proposal: LTE/NR spectrum sharing in Band 38/n38

# Annex

- Test Model for DSS

- Figure 5: TM 1.1/3.1/3.1a for DSS

- Figure 6: TM 1.2/3.2/3.3 for DSS

- Figure 7: TM 2.0 for DSS