3GPP TSG-RAN4 Meeting #96-e	R4-2010463

# Electronic Meeting, 17th Aug 2020 - 28th Aug 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5660 | rev | - | Current version: | 15.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of applicability for MTC UE capable of 64QAM DL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC4-Perf |  |  |  |  |  | Date: |  |  | 2020-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | No applicability rule is specified for PDSCH demodulation requirements with 64QAM for MTC UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add note PDSCH demodulation requirements with 64QAM for MTC UE is applicable for MTC UE capable of ce-PDSCH-64QAM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is misleading |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.11.1.1.3.1 and 8.11.1.2.3.1. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521-1... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

##### 8.11.1.1.3	Transmit diversity performance (Cell-Specific Reference Symbols)

8.11.1.1.3.1	Minimum Requirement 2 Tx Antenna Port supporting narrowband transmission

The requirements are specified in Table 8.11.1.1.3.1-2, with the addition of the parameters in Table 8.11.1.1.3.1-1 and Table 8.11.1.1.3.1-1a, and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of transmit diversity (SFBC) with 2 transmitter antennas.

Table 8.11.1.1.3.1-1: Test Parameters for Transmit diversity performance (FRC)

| Parameter |  | Unit | Test 1 (Note 3) | Test 2 (Note 3) | Test 2a (Note 3) | Test 3 (Note 3) | Test 4 (Note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 | -3 | -3 | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) |
|  |  | dB | 0 | 0 | 0 | 0 | 0 |
|  | δ | dB | 3 | 3 | 3 | 3 | 3 |
| at antenna port |  | dBm/15kHz | -98 | -98 | -98 | -98 | -98 |
| Coverage enhancement mode |  |  | CE Mode B | CE Mode A | CE Mode A | CE Mode B | CE Mode B |
| PDSCH transmission mode |  |  | 2 | 2 | 2 | 2 | 2 |
| OFDM starting symbol (startSymbolBR) |  |  | 2 | 2 | 2 | 2 | 2 |
| Maximum number of repetitions (for PDSCH (pdsch-maxNumRepetitionCEmodeA/ pdsch-maxNumRepetitionCEmodeB)) |  |  | Not configured | Not configured | Not configured | Not configured | Not configured |
| PDSCH repetition number |  |  | 64 | 1 | 1 | 32 | 16 |
| Frequency hopping (mpdcch-pdsch-HoppingConfig) |  |  | Enabled | Disabled | Disabled | Enabled | Enabled |
| Frequency hopping offset (mpdcch-pdsch-HoppingOffset) |  |  | 1 | N/A | N/A | 1 | 1 |
| Frequency hopping interval (interval-FDD) |  | ms | 16 | N/A | N/A | 8 | 4 |
| MPDCCH transmission duration (mPDCCH-NumRepetition) |  | ms | 64 | 1 | 1 | 32 | 8 |
| MPDCCH repetition number |  |  | 64 | 1 | 1 | 32 | 8 |
| Number of narrowbands for frequency hopping (mpdcch-pdsch-HoppingNB) |  |  | 4 | N/A | N/A | 4 | 4 |
| Starting subframe configuration for MPDCCH (mpdcch_startSF_UESS) |  |  | 2.5 | 1 | 1 | 2.5 | 4 |
| Narrowband for MPDCCH (mpdcch_Narrowband) |  |  | 7 | 0 | 0 | 7 | 7 |
| MPDCCH aggregation level |  |  | 24 | 8 | 8 | 24 | 24 |
| CRS muting outside UE RF bandwidth (crs-IntfMitigEnabled) |  |  | Disabled | Disabled | Enabled | Disabled | Disabled |
| Number of PRBs for CRS transmission (crs-IntfMitigNumPRBs) |  |  | N/A | N/A | 6 | N/A | N/A |
| Note 1: . Note 2: For each test, DC subcarrier puncturing shall be considered. Note 3: Test 1, test 3 and test 4 are applicable for UE supporting CE Mode B. Test 2 is applicable for UE not supporting CE Mode B. Test 2a is applicable for UE not supporting CE Mode B and UE capable of ce-CRS-IntfMitig. Note 4: If not otherwise stated, the values in this table refer to parameters in TS 36.211 [4] or/and TS 36.213 [6] as appropriate. |  |  |  |  |  |  |  |

Table 8.11.1.1.3.1-1a: Test Parameters for Transmit diversity performance (FRC)

| Parameter | Unit | Test 5 | Test 6 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
|  | δ | dB | 3 |
| at antenna port | dBm/15kHz | -98 | -98 |
| Coverage enhancement mode |  | CE Mode A | CE Mode A |
| PDSCH transmission mode |  | 2 | 2 |
| OFDM starting symbol (startSymbolBR) |  | 2 | 2 |
| Maximum number of repetitions (for PDSCH (pdsch-maxNumRepetitionCEmodeA/ pdsch-maxNumRepetitionCEmodeB)) |  | Not configured | Not configured |
| PDSCH repetition number |  | 1 | 1 |
| Frequency hopping (mpdcch-pdsch-HoppingConfig) |  | Disabled | Disabled |
| Frequency hopping offset (mpdcch-pdsch-HoppingOffset) |  | N/A | N/A |
| Frequency hopping interval (interval-FDD) | ms | N/A | N/A |
| MPDCCH transmission duration (mPDCCH-NumRepetition) | ms | 4 | 1 |
| MPDCCH repetition number |  | 4 | 1 |
| Number of narrowbands for frequency hopping (mpdcch-pdsch-HoppingNB) |  | N/A | N/A |
| Starting subframe configuration for MPDCCH (mpdcch_startSF_UESS) |  | 1 | 1 |
| Narrowband for MPDCCH (mpdcch_Narrowband) |  | 0 | 0 |
| MPDCCH aggregation level |  | 8 | 8 |
| CRS muting outside UE RF bandwidth (crs-IntfMitigEnabled) |  | Disabled | Disabled |
| Activation of 64QAM for non-repeated PDSCH in CE Mode A (ce-PDSCH-64QAM-Config) |  | Disabled | Enabled |
| Note 1: . Note 2: For each test, DC subcarrier puncturing shall be considered. Note 3: If not otherwise stated, the values in this table refer to parameters in TS 36.211 [4] or/and TS 36.213 [6] as appropriate. Note 4: Test 6 is applicable for UE capable of ce-PDSCH-64QAM. |  |  |  |

Table 8.11.1.1.3.1-2: Minimum performance Transmit Diversity (FRC)

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10MHz QPSK 1/10 | R.81 FDD | OP.2 FDD | ETU1 | 2x1 Low | 70 | -13.5 | M1 |
| 2 | 10MHz 16QAM 1/2 | R.79 FDD | OP.2 FDD | EPA5 | 2x1 Low | 70 | 9.4 | M1 |
| 2a | 10MHz 16QAM 1/2 | R.79 FDD | OP.2 FDD | EPA5 | 2x1 Low | 70 | 9.6 | M1 |
| 3 | 10MHz QPSK 1/10 | R.81-1 FDD | OP.2 FDD | ETU1 | 2x2 Low | 70 | -13.3 | ≥1 |
| 4 | 10MHz QPSK 1/10 | R.81-2 FDD | OP.2 FDD | ETU1 | 2x4 Low | 70 | -13.3 | ≥1 |
| 5 | 10MHz QPSK 1/3 | R.103 FDD | OP.2 FDD | EPA200 | 2x1 Low | 70 | 3.8 | M1 |
| 6 | 10MHz 64QAM 0.4 | R.104 FDD | OP.2 FDD | EPA5 | 2x1 Low | 70 | 12.9 | M1 |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

##### 8.11.1.2.3	Transmit diversity performance (Cell-Specific Reference Symbols)

8.11.1.2.3.1	Minimum Requirement 2 Tx Antenna Port supporting narrowband transmission

The requirements are specified in Table 8.11.1.2.3.1-2, with the addition of the parameters in Table 8.11.1.2.3.1-1 and Table 8.11.1.2.3.1-1a, and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of transmit diversity (SFBC) with 2 transmitter antennas.

Table 8.11.1.2.3.1-1: Test Parameters for Transmit diversity performance (FRC)

| Parameter |  | Unit | Test 1 (Note 3) | Test 2 (Note 3) | Test 2a (Note 3) | Test 3 (Note 3) | Test 4 (Note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 | -3 | -3 | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) | -3 (Note 1) |
|  |  | dB | 0 | 0 | 0 | 0 | 0 |
|  | δ | dB | 3 | 3 | 3 | 3 | 3 |
| at antenna port |  | dBm/15kHz | -98 | -98 | -98 | -98 | -98 |
| ACK/NACK feedback mode |  |  | Multiplexing | Multiplexing | Multiplexing | Multiplexing | Multiplexing |
| PDSCH transmission mode |  |  | 2 | 2 | 2 | 2 | 2 |
| Coverage enhancement mode |  |  | CE Mode B | CE Mode A | CE Mode A | CE Mode B | CE Mode B |
| PDSCH transmission mode |  |  | 2 | 2 | 2 | 2 | 2 |
| OFDM starting symbol (startSymbolBR) |  |  | 2 | 2 | 2 | 2 | 2 |
| Maximum number of repetitions (for PDSCH (pdsch-maxNumRepetitionCEmodeA/ pdsch-maxNumRepetitionCEmodeB)) |  |  | Not configured | Not configured | Not configured | Not configured | Not configured |
| PDSCH repetition number |  |  | 64 | 1 | 1 | 32 | 16 |
| Frequency hopping (mpdcch-pdsch-HoppingConfig) |  |  | Enabled | Disabled | Disabled | Enabled | Enabled |
| Frequency hopping offset (mpdcch-pdsch-HoppingOffset) |  |  | 1 | N/A | N/A | 1 | 1 |
| Frequency hopping interval (interval-TDD) |  | ms | 20 | N/A | N/A | 20 | 10 |
| MPDCCH transmission duration (mPDCCH-NumRepetition) |  | ms | 64 | 1 | 1 | 32 | 8 |
| MPDCCH repetition number |  |  | 64 | 1 | 1 | 32 | 8 |
| Number of narrowbands for frequency hopping (mpdcch-pdsch-HoppingNB) |  |  | 4 | N/A | N/A | 4 | 4 |
| Starting subframe configuration for MPDCCH (mpdcch-startSF-UESS) |  |  | 8 | 1 | 1 | 8 | 10 |
| Narrowband for MPDCCH (mpdcch_Narrowband) |  |  | 7 | 0 | 0 | 7 | 7 |
| MPDCCH aggregation level |  |  | 24 | 8 | 8 | 24 | 24 |
| CRS muting outside UE RF bandwidth (crs-IntfMitigEnabled) |  |  | Disabled | Disabled | Enabled | Disabled | Disabled |
| Number of PRBs for CRS transmission (crs-IntfMitigNumPRBs) |  |  | N/A | N/A | 6 | N/A | N/A |
| Note 1: . Note 2: For each test, DC subcarrier puncturing shall be considered. Note 3: Test 1, test 3 and test 4 are applicable for UE supporting CE Mode B. Test 2 is applicable for UE not supporting CE Mode B. Test 2a is applicable for UE not supporting CE Mode B and UE capable of ce-CRS-IntfMitig. Note 4: If not otherwise stated, the values in this table refer to parameters in TS 36.211 [4] or/and TS 36.213 [6] as appropriate. |  |  |  |  |  |  |  |

Table 8.11.1.2.3.1-1a: Test Parameters for Transmit diversity performance (FRC)

| Parameter |  | Unit | Test 5 | Test 6 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 (Note 1) |
|  |  | dB | 0 | 0 |
|  | δ | dB | 3 | 3 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| ACK/NACK feedback mode |  |  | Multiplexing | Multiplexing |
| PDSCH transmission mode |  |  | 2 | 2 |
| Coverage enhancement mode |  |  | CE Mode A | CE Mode A |
| PDSCH transmission mode |  |  | 2 | 2 |
| OFDM starting symbol (startSymbolBR) |  |  | 2 | 2 |
| Maximum number of repetitions (for PDSCH (pdsch-maxNumRepetitionCEmodeA/ pdsch-maxNumRepetitionCEmodeB)) |  |  | Not configured | Not configured |
| PDSCH repetition number |  |  | 1 | 1 |
| Frequency hopping (mpdcch-pdsch-HoppingConfig) |  |  | Disabled | Disabled |
| Frequency hopping offset (mpdcch-pdsch-HoppingOffset) |  |  | N/A | N/A |
| Frequency hopping interval (interval-TDD) |  | ms | N/A | N/A |
| MPDCCH transmission duration (mPDCCH-NumRepetition) |  | ms | 4 | 1 |
| MPDCCH repetition number |  |  | 4 | 1 |
| Number of narrowbands for frequency hopping (mpdcch-pdsch-HoppingNB) |  |  | N/A | N/A |
| Starting subframe configuration for MPDCCH (mpdcch-startSF-UESS) |  |  | 1 | 1 |
| Narrowband for MPDCCH (mpdcch_Narrowband) |  |  | 0 | 0 |
| MPDCCH aggregation level |  |  | 8 | 8 |
| CRS muting outside UE RF bandwidth (crs-IntfMitigEnabled) |  |  | Disabled | Disabled |
| Activation of 64QAM for non-repeated PDSCH in CE Mode A (ce-PDSCH-64QAM-Config) |  |  | Disabled | Enabled |
| Note 1: . Note 2: For each test, DC subcarrier puncturing shall be considered. Note 3: If not otherwise stated, the values in this table refer to parameters in TS 36.211 [4] or/and TS 36.213 [6] as appropriate. Note 4: Test 6 is applicable for UE capable of ce-PDSCH-64QAM. |  |  |  |  |

Table 8.11.1.2.3.1-2: Minimum performance Transmit Diversity (FRC)

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10MHz QPSK 1/10 | R.81 TDD | OP.2 TDD | ETU1 | 2x1 Low | 70 | -14.0 | M1 |
| 2 | 10MHz 16QAM 1/2 | R.79 TDD | OP.2 TDD | EPA5 | 2x1 Low | 70 | 9.6 | M1 |
| 2a | 10MHz 16QAM 1/2 | R.79 TDD | OP.2 TDD | EPA5 | 2x1 Low | 70 | 9.8 | M1 |
| 3 | 10MHz QPSK 1/10 | R.81-1 TDD | OP.2 TDD | ETU1 | 2x2 Low | 70 | -13.3 | ≥1 |
| 4 | 10MHz QPSK 1/10 | R.81-2 TDD | OP.2 TDD | ETU1 | 2x4 Low | 70 | -13.3 | ≥1 |
| 5 | 10MHz QPSK 1/3 | R.103 TDD | OP.2 TDD | EPA200 | 2x1 Low | 70 | 3.5 | M1 |
| 6 | 10MHz 64QAM 0.4 | R.104 TDD | OP.2 TDD | EPA5 | 2x1 Low | 70 | 13.5 | M1 |

------------------------------------------------------------- End of change ------------------------------------------------------------