3GPP TSG-RAN WG4 Meeting # 96-e 															R4-2010486

Electronic Meeting, 17 August – 28 August, 2020

Source: 	Huawei, HiSilicon

Title: 	Downlink co-existence simulation results on 6.425-7.125GHz and 10.0-10.5GHz for urban macro and indoor hotspot

Agenda Item:	11.1.3.2

Document for:	Approval

Introduction

The SI on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz was approved at TSG RAN#87-e [1]. Transmitter and receiver characteristics for the NR UE and BS need to be studied and evaluated. As for ACLR and ACS, RAN4 need to perform the adjacent channel co-existence simulations on both 6.425-7.125GHz and 10.0-10.5GHz.

In last meeting, the simulation assumptions are approved in WF [2] and TP [3]. In this paper, we’d like to provide the adjacent channel downlink co-existence simulation results on both 6.425-7.125GHz and 10.0-10.5GHz for urban macro and indoor hotspot.

Discussion

Simulation results on 6.425-7.125GHz

Based on the current specifications, we set 45dB ACLR for BS and 33dB ACS for UE as baseline. The corresponding ACIR is about 32.7dB. When we simulate the scenario, different delta-ACIR X values were considered in order to find the reasonable requirements. The CDF figures of SINR and throughput for Macro scenario are shown below.

Figure 1 The CDF figures of SINR and throughput for Macro scenario

The urban macro simulation results of the downlink throughput loss of the victim UE with different delta-ACIR values are shown at 7GHz centre frequency in table 1

Table 1 The urban macro simulation results of the downlink throughput loss at 7GHz centre frequency

| delta-ACIR X [dB] | 0 | -1 | -2 | -3 |
| --- | --- | --- | --- | --- |
| Average throughput (7GHz) baseline | 466.6591472 | 466.0256152 | 467.6262642 | 463.2999863 |
| Average throughput (7GHz) | 461.4606274 | 460.5681709 | 460.8944972 | 454.8474735 |
| Average throughput loss (7GHz) | 1.11% | 1.17% | 1.44% | 1.82% |
| 5%-tile throughput (7GHz) baseline | 103.5958843 | 112.7570174 | 110.5259045 | 105.8749672 |
| 5%-tile throughput (7GHz) | 99.24795288 | 107.0513429 | 103.811823 | 97.6744638 |
| 5%-tile throughput loss (7GHz) | 4.20% | 5.06% | 6.07% | 7.75% |

The CDF figures of SINR and throughput for Indoor hotpot scenario are shown below.

Figure 2 The CDF figures of SINR and throughput for Indoor hotpot scenario

The indoor hotpot simulation results of the downlink throughput loss of the victim UE with different delta-ACIR values are shown at 7GHz centre frequency in table 2

Table 2 The indoor hotpot simulation results of the downlink throughput loss at 7GHz centre frequency

| delta-ACIR X [dB] | 0 | -2 | -4 | -6 |
| --- | --- | --- | --- | --- |
| Average throughput (7GHz) baseline | 501.7083 | 501.6906614 | 502.0155946 | 499.4085242 |
| Average throughput (7GHz) | 500.0025 | 499.558 | 498.6154 | 493.0911 |
| Average throughput loss (7GHz) | 0.34% | 0.43% | 0.68% | 1.26% |
| 5%-tile throughput (7GHz) baseline | 167.2007 | 165.7132217 | 166.9807175 | 165.4564 |
| 5%-tile throughput (7GHz) | 167.1902 | 165.6541 | 166.8976 | 165.1538 |
| 5%-tile throughput loss (7GHz) | 0.01% | 0.04% | 0.05% | 0.18% |

It can be found that the Urban Macro is the restricted scenario instead of the indoor hotpot. Based on the simulation results, urban macro scenario can be restricted to 5% throughput loss with downlink delta-ACIR of -1dB at 7GHz. The corresponding ACIR value is 31.7 dB which is less stringent than current FR1 requirements. If we keep 33dB ACS for UE at 7GHz, 36~40 dB ACLR for BS can be achieved to meet 31.7 dB ACIR.

Observation 1: When downlink ACIR is set to 31.7dB at 7GHz, the urban macro and indoor scenario can be restricted to 5% DL throughput loss.

Simulation results on 10.0-10.5GHz

The CDF figures of SINR and throughput for Macro scenario on 10.0-10.5GHz are shown below.

Figure 3 The CDF figures of SINR and throughput for Macro scenario at 10GHz

The urban macro simulation results of the downlink throughput loss of the victim UE with different delta-ACIR values are shown at 10GHz centre frequency in table 3

Table 3 The urban macro simulation results of the downlink throughput loss at 10GHz centre frequency

| delta-ACIR X [dB] | 0 | -1 | -2 | -3 |
| --- | --- | --- | --- | --- |
| Average throughput (10GHz) baseline | 457.5910423 | 459.0642451 | 455.4940226 | 458.3263305 |
| Average throughput (10GHz) | 452.1292008 | 453.1403575 | 449.3607213 | 450.7480241 |
| Average throughput loss (10GHz) | 1.19% | 1.29% | 1.35% | 1.65% |
| 5%-tile throughput (10GHz) baseline | 92.20054204 | 80.69748647 | 87.6480235 | 89.05865874 |
| 5%-tile throughput (10GHz) | 90.29398594 | 78.21200614 | 83.27198421 | 83.0038 |
| 5%-tile throughput loss (10GHz) | 2.07% | 3.08% | 4.99% | 6.80% |

The CDF figures of SINR and throughput for Indoor hotpot scenario are shown below.

Figure 4 The CDF figures of SINR and throughput for Indoor hotpot scenario at 10GHz

The indoor hotpot simulation results of the downlink throughput loss of the victim UE with different delta-ACIR values are shown at 10GHz centre frequency in table 4

Table 4 The indoor hotpot simulation results of the downlink throughput loss at 10GHz centre frequency

| delta-ACIR X [dB] | 0 | -2 | -4 | -6 |
| --- | --- | --- | --- | --- |
| Average throughput (10GHz) baseline | 497.910349 | 500.4417595 | 497.6593374 | 500.6863797 |
| Average throughput (10GHz) | 497.643915 | 497.1837634 | 494.1940229 | 496.699647 |
| Average throughput loss (10GHz) | 0.05% | 0.65% | 0.70% | 0.80% |
| 5%-tile throughput (10GHz) baseline | 165.5580639 | 165.3603094 | 162.5645362 | 165.9941696 |
| 5%-tile throughput (10GHz) | 165.5572363 | 165.354215 | 162.3308392 | 165.4927392 |
| 5%-tile throughput loss (10GHz) | 0.00% | 0.00% | 0.14% | 0.30% |

It can be found that the Urban Macro is the restricted scenario instead of the indoor hotpot at 10GHz as well. Based on the simulation results, urban macro scenario can be restricted to 5% throughput loss with downlink delta-ACIR of -2dB at 10GHz. The corresponding ACIR value is 30.7 dB which is less stringent than current FR1 requirements. If we keep 33dB ACS for UE at 10GHz, 36~38 dB ACLR for BS can be achieved to meet 30.7 dB ACIR.

Observation 2: When downlink ACIR is set to 30.7dB at 10GHz, the urban macro and indoor scenario can be restricted to 5% DL throughput loss.

Analysis

For UE DL receiver, there is no technical restriction to achieve 33dB ACS at 7GHz and 10GHz. However, more antenna elements and larger channel bandwidth probably make it difficult to implement larger ACLR value for BS. It seems to be much better to specify one value for both 7GHz and 10GHz. Thus, It’s proposed to specify 36 dB ACLR for BS and 33dB ACS for UE on both 6.425-7.125GHz and 10.0-10.5GHz. The corresponding ACIR is 31.2dB which is the average value of 30.7 and 31.7.

Proposal: It’s proposed to specify 36 dB ACLR for BS and 33dB ACS for UE on both 6.425-7.125GHz and 10.0-10.5GHz.

Summary

Based on the simulation results and analysis, all the observations and proposals are listed below:

Observation 1: When downlink ACIR is set to 31.7dB at 7GHz, the urban macro and indoor scenario can be restricted to 5% DL throughput loss.

Observation 2: When downlink ACIR is set to 30.7dB at 10GHz, the urban macro and indoor scenario can be restricted to 5% DL throughput loss.

Proposal: It’s proposed to specify 36 dB ACLR for BS and 33dB ACS for UE on both 6.425-7.125GHz and 10.0-10.5GHz.

- References

- RP-200513, “New SI proposal: Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz”, Ericsson, Huawei, HiSilicon.

- R4-2008927, WF on Simulation Assumptions for the SI on 6.425-7.125GHz and 10.0-10.5GHz, Nokia, Ericsson, ZTE

- R4-2008928, TP to TR 38.9xx: System level simulation methodology and assumptions for study on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz, Nokia, Nokia Shanghai Bell