3GPP TSG-RAN WG4 Meeting #96-e								          R4-2010497

Electronic Meeting, 17-28 Aug., 2020

Source: 	Huawei, HiSilicon

Title: 	Discussion on NR-U UE ACLR and MPR evaluation

Agenda Item:	7.1.2.1

Document for:	Approval

Introduction

In last RAN4#95-e meeting, for PC3 ACLR two options has been discussed and the following value is agreed [4].

  - ACLR = [28 to 30] dB for PC3 in NR-U

And, way forward on NR-U MPR was agreed in [5]. Tentative agreements are reached for PC5 MPR values.

In this contribution, we provide proposal for NR-U PC3 ACLR and some simulation results on the MPR.

Discussion

PC3 ACLR

The 27 dB ACLR for PC5 in NR-U was agreed in [1], however, the requirement for PC3 is still an open issue.  In NR, the ACLR requirement is defined as below

Table 6.5.2.4.1-2: NR ACLR requirement

|  | Power class 2 | Power class 3 |
| --- | --- | --- |
| NR ACLR | 31 dB | 30 dB |

It can be found that the ACLR requirement difference between PC2 and PC3 is only 1dB in licensed band. It is reasonable to reuse the same principle in NR-U, i.e. the ACLR for PC3 in NR-U should be specified to 28dB. Besides, during the study on LAA/eLAA, impact of LAA to Wi-Fi compared to the Wi-Fi + Wi-Fi scenario had been evaluated in [3], and it can be observed that 26.3dB ACLR can guarantee a fair coexistence with WiFi, which means that 28dB ACLR could also meet the requirement. Furthermore there should be no co-existence issue since we agreed to use the generic mask also applied to WIFI.

The corresponding NR-U ACLR requirement in following table 1 should be supported.

Table 1: NR-U ACLR requirement

|  | Power class 3 | Power class 5 |
| --- | --- | --- |
| NR-U ACLR | 28 dB | 27 dB |

Proposal 1: ACLR for PC3 in NR-U should be specified to 28dB.

MPR evaluation

Simulated MPR is shown as below. The SEM is the general mask and ACLR= 27 dB for PC5, using the assumptions agreed in [1].

| Full allocation (15kHz) |  | 20M | 40M | 60M | 80M | 100M | Max |
| --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | CP-OFDM | 3.2 | 3.2 |  |  |  | 3.2 |
|  | DFT-S | 1.2 | 1.6 |  |  |  | 1.6 |
| Full allocation (30kHz) |  | 20M | 40M | 60M | 80M | 100M |  |
| QPSK | CP-OFDM | 3 | 3.2 | 3.2 | 3.2 | 3.3 | 3.3 |
|  | DFT-S | 1.2 | 1.2 | 1.6 | 1.3 | 1.3 | 1.6 |
| Full allocation (15kHz) |  | 20M | 40M | 60M | 80M | 100M |  |
| 16QAM | CP-OFDM | 3.2 | 3.3 |  |  |  | 3.3 |
|  | DFT-S | 1.8 | 2.1 |  |  |  | 2.1 |
| Full allocation (30kHz) |  | 20M | 40M | 60M | 80M | 100M |  |
| 16QAM | CP-OFDM | 3 | 3.2 | 3.3 | 3.3 | 3.3 | 3.3 |
|  | DFT-S | 1.7 | 1.8 | 2.1 | 2 | 2 | 2.1 |
| Interlace 0 (15kHz) |  | 20M | 40M | 60M | 80M | 100M |  |
| QPSK | CP-OFDM | 3.4 | 3.4 |  |  |  | 3.4 |
|  | DFT-S | 2.3 | 2.4 |  |  |  | 2.4 |
| Interlace 0 (30kHz) |  | 20M | 40M | 60M | 80M | 100M |  |
| QPSK | CP-OFDM | 3.5 | 3.3 | 3.2 | 3.2 | 3.2 | 3.5 |
|  | DFT-S | 2.3 | 2.3 | 2.3 | 2.3 | 2 | 2.3 |
| Interlace 0 (15kHz) |  | 20M | 40M | 60M | 80M | 100M |  |
| 16QAM | CP-OFDM | 3.4 | 3.3 |  |  |  | 3.4 |
|  | DFT-S | 2.7 | 2.7 |  |  |  | 2.7 |
| Interlace 0 (30kHz) |  | 20M | 40M | 60M | 80M | 100M |  |
| 16QAM | CP-OFDM | 3.3 | 3.3 | 3.3 | 3.2 | 3.2 | 3.3 |
|  | DFT-S | 2.7 | 2.7 | 2.6 | 2.4 | 2.4 | 2.7 |

Based on our study, we propose to update the MPR proposal as below.

| Modulation |  | MPR (dB) |  |
| --- | --- | --- | --- |
|  |  | Full RB allocations | Partial RB allocations |
| DFT-S-OFDM | QPSK | [1.6] | [2.5] |
|  | 16QAM | [2.1] | [3.0] |
| CP-OFDM | QPSK | [3.5] | [3.5] |
|  | 16QAM | [4.0] | [4.0] |

Conclusions

In this contribution, we discussed the remaining issues on UE Tx requirement in NR-U, and the following proposal was made:

Proposal 1: ACLR for PC3 in NR-U should be specified to 28dB.

References

R4-2005221 WF on NR-U UE Tx requirements

38.101-1 V16.0.0  Base Station (BS) radio transmission and reception

TR 36.889 Study on Licensed-Assisted Access to Unlicensed Spectrum

R4-2009165, WF on NR-U PC3 ACLR and in-band emissions, Huawei, HiSilicon

R4-2008436, WF on NR-U MPR, MediaTek Inc., Skyworks Solution Inc.