3GPP TSG-WG RAN4 Meeting #96-e	R4-2010522

# Online, 17th – 28th August, 2020

Source: 		Nokia, Nokia Shanghai Bell

Title: 			Band number and System parameters of 47 GHz band

Agenda Item: 	10.24.3 [NR_47GHz_Band]

Document for:	Approval

# Introduction

The work item to specify a new band (47.2-48.2 GHz) was approved in RAN#88e meeting [1]. In this document, the band definition and system parameters for this new band is discussed.

# Discussion

Band definition

The new band 47.2-48.2 GHz is within the range of FR2 (24250 – 52600 MHz). The smallest band number available in FR2 is n262, therefore, it is proposed n262 is used for this new band. The duplex is TDD as specified in WID.

Table 1: The new FR2 TDD operating band 47.2-48.2 GHz

| Band number | UL | DL | Duplex mode |
| --- | --- | --- | --- |
| n262 | 47.2 – 48.2 GHz | 47.2 – 48.2 GHz | TDD |

Proposal 1: n262 is used for the new FR2 TDD band 47.2-48.2 GHz.

Channel Bandwidth

The channel bandwidths are already clarified in WID [1] shown in Table 2. Though the channelization in FCC rules is 100 MHz, it is not precluded to use 50 MHz channel bandwidth. Allocation block size is still unknown in other administrations. For maximum flexibility, it is reasonable to assume that the minimum channel bandwidth is 50 MHz also for this band like the other FR2 bands.

Table 2: NR channel bandwidth for NR band n262

| NR band |  | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Band number | data SCS(kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n262 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |

Channel Raster

The channel raster for n262 is presented in Table 3.

Table 3: NR-ARFCN for NR band n262

| NR Operating Band | ΔFRaster (kHz) | Uplink and Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- |
| n262 | 60 | 2399166 – <1> – 2415832 |
|  | 120 | 2399167 – <2> – 2415831 |

Sync Raster

The sync raster and burst pattern for n262 is presented in Table 3.

Table 4: SS raster for NR band n262

| NR Operating Band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n262 | 120 kHz | Case D | 23586 – <1> – 23641 |
|  | 240 kHz | Case E | 23588 – <2> – 23640 |
| NOTE 1: SS Block pattern is defined in subclause 4.1 in TS 38.213. |  |  |  |

# Conclusion

In this contributions, the band definition and system parameters for the new FR2 TDD band 47.2-48.2 GHz are proposed.

# Reference

[1] RP-201232	New WID: introduction of NR 47 GHz band, T-Mobile USA, Dish Network, RAN#88e

[2] R4-2010521	Regulatory Background of 47 GHz band, Nokia, RAN#96e