3GPP TSG-RAN WG4 Meeting # 96-e 	R4-2010714

# Electronic Meeting, 17 – 28 Aug., 2020

Source:	OPPO

Title:	On measurement requirements for CSI-RS L3 measurement

Agenda Item:	7.14.1.4

Document for:	Discussion

Introduction

In last RAN4 #95e meeting, a WF on CSI-RS based L3 measurement capability and requirements [1] was approved. And in last RAN 88e meeting, a Rel-16 WI Exception for CSI-RS based L3 measurement was approved including UE measurement requirement.

- Scope of requirement

  - whether UE is required to perform Rx beam sweeping for CSI-RS based L3 measurement

- Cell identification requirement

  - whether to introduce 2 different requirements for with index and without index

  - the tuning time of inter-frequency GAP

  - time-domain restriction on CSI-RS resources configuration

  - note: CMTC for CSI-RS L3 measurement is out of scope

- Scheduling restriction on the following cases

  - when UE is not able to support mixed numerology of data and CSI-RS L3 mobility

  - when UE performs CSI-RS intra-frequency measurements in a TDD band

  - when UE performs RX beam sweeping

- The collision case between L1 measurement of serving cell and CSI-RS L3 measurement of neighbour cell

- CSSF for CSI-RS based measurement within measurement gap and outside of measurement gap.

In this contribution, we further discuss the remaining measurement requirements for CSI-RS L3 measurement.

Discussion

- Whether UE is required to perform Rx beam sweeping for CSI-RS based L3 measurement

In our view, no additional Rx beam sweeping is needed for CSI-RS configured with QCL-ed Type D to the associated SSB. The scaling factor of Rx beam sweeping has been already considered for SSB based Rx beam sweep for target cell search.

It is not a common case that the CSI-RS resources are QCL-ed with different associated SSBs when the multiple CSI-RS resources from different cells are transmitted in the same OFDM symbols in one MO. Even in this case, UE can conduct Rx beam sweeping for all associated SSBs with different beam direction together. Furthermore, such scenario could also occur for SSB-based measurement when different SSBs from different cells are transmitted in the same OFDM symbols in one MO. It depends on UE’s implementation for Rx beam. So it is suggested not to be specified in Rel-16. More discussion can be done in future release if necessary.

Therefore, it is proposed that UE is not required to perform dedicated Rx beam sweeping for CSI-RS based L3 measurement.

Proposal 1：UE is not required to perform dedicated Rx beam sweeping for CSI-RS based L3 measurement in Rel-16.

- Cell identification requirement

- whether to introduce 2 different requirements for with index and without index

- the tuning time of inter-frequency GAP

- time-domain restriction on CSI-RS resources configuration

- note: CMTC for CSI-RS L3 measurement is out of scope

As agreed in last meeting, UE shall be able to at least identify a new detectable CSI-RS based inter frequency cell within T CSI-RS_identify_inter_with_index,

T CSI-RS_identify_inter_with_index = (TPSS/SSS_sync + T CSI-RS_measurement_period_inter + TSSB_time_index) ms

If UE has already detected the associatedSSB of the target cell and deriveSSB-IndexFromCell is indicated, the CSI-RS based measurement shall only include TCSI-RS_measurement_period_inter.

In our view, UE should first try to detect the associated SSB before measuring the CSI-RS. The requirement can follow TPSS/SSS_sync and TSSB_time_index for SSB-based measurement. TPSS/SSS_sync is the time period used in PSS/SSS detection, and TSSB_time_index is the time period used to acquire the index of the SSB being measured, which are determined

- according to TPSS/SSS_sync_intra and TSSB_time_index_intra given in clause 9.2.5 for SSB based intra-frequency measurements, or

- according to TPSS/SSS_sync_inter and TSSB_time_index_inter given in clause 9.3.4 for SSB based inter-frequency measurement.

Proposal 2: Do not introduce 2 different requirements for with index and without index.

- the tuning time of inter-frequency GAP

If the target cell is the first time to be measured, the AGC adjustment time be considered, i.e., 3 samples can be reused. The tuning time of inter-frequency GAP should be also considered, which can be equal to the gap switch time for measuring the inter-frequency SSBs.

Proposal 3: The tuning time of inter-frequency GAP is equal to the gap switch time for measuring inter-frequency SSBs.

- Time-domain restriction on CSI-RS resources configuration

As discussed in last meeting, RAN4 agreed to introduce the restriction on time-domain for CSI-RS resource in Rel-16 and leaving CMTC (CSI-RS Measurement Timing Configuration) to be discussed in Rel-17 enhancement. And two options are provided to be further decided in this meeting.

- Option 1: to limit all CSI-RS resources for L3 measurement should be configured within 5ms window. And CSI-RS periodicities for L3 measurement can be [10,20,40] ms.

- Option 2: to confine CSI-RS resources within SMTC of the associatedSSB and the corresponding periodicity should follow SMTC periodicity.

In our view, either option has both pros and cons. In option 1, the configuration of CSI-RS is flexible, but it is difficult for UE to fast search the starting point of measurement window without configured “offset”. As for option 2, it is easier for UE to search the measurement window through SMTC configuration in order to reduce the complexity, but the flexibility of CSI-RS configuration would be impacted. Thus, a trade-off between flexibility and complexity is needed. From UE perspective, it is more acceptable to take complexity reduction to fulfil more stable performance. So it is proposed to confine CSI-RS resources within SMTC of the associatedSSB in Rel-16. And any improvement i.e., CMTC can be introduced in Rel-17.

Besides, according to RAN1’s agreement, we suggest up to 1 CSI-RS periodicity should be configured for per intra-frequency or inter- frequency layer in Rel-16. And  the maximum of SMTC periodicity configured for CSI-RS should no more than 40ms.

Proposal 4: Support to confine CSI-RS resources within SMTC of the associatedSSB and the corresponding periodicity should follow SMTC periodicity≤40ms.

- Scheduling restriction

The requirements for scheduling restriction are only defined for CSI-RS L3 measurement without gaps. Regarding the scheduling restriction was introduced for SSB based measurement without gaps, the similar issues can be considered for CSI-RS L3 measurement [4].

- Collision with UL transmission and DL measurement for TDD band on FR1

- Mix-numerology between data/SSB of serving cell and CSI-RS of neighbour cell

Proposal 5: When UE performs CSI-RS intra-frequency measurements in a FR1 TDD band, UE is not expected to transmit and receive on X data OFDM symbols impacted by CSI-RS resource symbol to be measured, i.e., X=2.

Proposal 6: If UE is not able to support mixed numerology of data and CSI-RS L3 mobility, the following scheduling restrictions apply due to intra-frequency CSI-RS based L3 measurement:

    - if the associatedSSB is configured, UE is not expected to transmit or receive on Y data OFDM symbols impacted by CSI-RS resource symbol to be measured, i.e., Y=2.

    - if the associatedSSB is not configured, no requirements apply.

Conclusion

In this contribution, we propose our views as follows.

Proposal 1：UE is not required to perform dedicated Rx beam sweeping for CSI-RS based L3 measurement in Rel-16.

Proposal 2: Do not introduce 2 different requirements for with index and without index.

Proposal 3: The tuning time of inter-frequency GAP is equal to the gap switch time for measuring inter-frequency SSBs.

Proposal 4: Support to confine CSI-RS resources within SMTC of the associatedSSB and the corresponding periodicity should follow SMTC periodicity≤40ms.

Proposal 5: When UE performs CSI-RS intra-frequency measurements in a FR1 TDD band, UE is not expected to transmit and receive on X data OFDM symbols impacted by CSI-RS resource symbol to be measured, i.e., X=2.

Proposal 6: If UE is not able to support mixed numerology of data and CSI-RS L3 mobility, the following scheduling restrictions apply due to intra-frequency CSI-RS based L3 measurement:

    - if the associatedSSB is configured, UE is not expected to transmit or receive on Y data OFDM symbols impacted by CSI-RS resource symbol to be measured, i.e., Y=2.

    - if the associatedSSB is not configured, no requirements apply.

References

- R4-2009009, WF on CSI-RS based L3 measurement capability and requirements, OPPO

- RP-201340, Rel-16 WI Exception for Core part: RRM requirement for CSI-RS based L3 measurement in NR, CATT