3GPP TSG- <TSG/WG> Meeting #96-e	R4-2010823

# Electronic Meeting, 17-28 Aug, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR for 38.101-1 NR V2X FRC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduce NR-V2X FRC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New section A.7 is added for NR-V2X for differenent SCS and modulation orders. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR-V2X FRC isn’t defined in the Rel-16 spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7, A.7.1, A.7.2, A.7.3, A.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

# A.7 	V2X reference measurement channels

## A.7.1 General

The measurement channels in the following clauses are defined for NR V2X sidelink.

## A.7.2 FRC for maximum input level for QPSK

For V2X transmission over PC5, Table A.7.2-1, Table A.7.2-2 and Table A.7.2-3 are applicable for measurements on the Receiver Characteristics with the exception of Maximum input level.

Table A.7.2-1 Fixed reference channel for V2X receiver requirements (SCS 15 kHz, QPSK)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  | 52 | 106 | 160 | 216 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 9 | 9 | 9 | 9 |
| MCS Index |  | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| Maximum number of HARQ transmissions |  | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 1,2,3,4,5,6,7,8,9 | Bits | 3904 | 8192 | 12552 | 16896 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 | CBs | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | CBs | 1 | 1 | 2 | 3 |
| Binary Channel Bits per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | Bits | 13008 | 27264 | 41520 | 56304 |
| Max. Throughput averaged over 1 frame | Mbps | 3.514 | 7.373 | 11.297 | 15.206 |
| NOTE 1: There are 14 symbols in a SL slot. Considering AGC and GAP symbol, 12 symbols are allocated to PSSCH transmission. NOTE 2: 10RBs and 3 symbols allocated to PSCCH transmission NOTE 3: 2symbols allocated to PSSCH DMRS. FDM between DMRS and PSSCH is supported NOTE 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 5: Slot i is slot index per frame |  |  |  |  |  |

Table A.7.2-2 Fixed reference channel for V2X receiver requirements (SCS 30 kHz, QPSK)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 |
| Subcarrier spacing configuration |  | 1 | 1 | 1 | 1 |
| Allocated resource blocks |  | 24 | 51 | 78 | 106 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 10 | 10 | 10 | 10 |
| MCS Index |  | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| Maximum number of HARQ transmissions |  | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | Bits | 1736 | 3840 | 6016 | 8192 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 | CBs | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | CBs | 1 | 1 | 1 | 1 |
| Binary Channel Bits per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | Bits | 5616 | 12744 | 19872 | 27264 |
| Max. Throughput averaged over 1 frame | Mbps | 1.5624 | 3.456 | 5.414 | 7.373 |
| NOTE 1: There are 14 symbols in a SL slot. Considering AGC and GAP symbol, 12 symbols are allocated to PSSCH transmission. NOTE 2: 10RBs and 3 symbols allocated to PSCCH transmission NOTE 3: 2symbols allocated to PSSCH DMRS. FDM between PSSCH DMRS and PSSCH is supported NOTE 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 5: Slot i is slot index per frame |  |  |  |  |  |

Table A.7.2-3 Fixed reference channel for V2X receiver requirements (SCS 60 kHz, QPSK)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 60 | 60 | 60 | 60 |
| Subcarrier spacing configuration |  | 2 | 2 | 2 | 2 |
| Allocated resource blocks |  | 11 | 24 | 38 | 51 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 10 | 10 | 10 | 10 |
| MCS Index |  | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| Maximum number of HARQ transmissions |  | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | Bits | 672 | 1736 | 2792 | 3840 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 2 | 2 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 | CBs | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | CBs | 1 | 1 | 1 | 1 |
| Binary Channel Bits per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | Bits | 2184 | 5616 | 9312 | 12744 |
| Max. Throughput averaged over 1 frame | Mbps | 0.605 | 1.5624 | 2.513 | 3.456 |
| NOTE 1: There are 14 symbols in a SL slot. Considering AGC and GAP symbol, 12 symbols are allocated to PSSCH transmission. NOTE 2: 10RBs and 3 symbols allocated to PSCCH transmission NOTE 3: 2symbols allocated to PSSCH DMRS. FDM between PSSCH DMRS and PSSCH is supported NOTE 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 5: Slot i is slot index per frame |  |  |  |  |  |

## A.7.3 FRC for maximum input level for 64QAM

For V2X transmission over PC5, Table A.7.3-1, Table A.7.3-2 and Table A.7.3-3 are applicable for Maximum input level when the maximum modulation order is 64QAM.

Table A.7.3-1 Fixed reference channel for V2X receiver requirements (SCS 15 kHz, 64QAM)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  | 52 | 106 | 160 | 216 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 9 | 9 | 9 | 9 |
| MCS Index |  | 24 | 24 | 24 | 24 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| Maximum number of HARQ transmissions |  | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 1,2,3,4,5,6,7,8,9 | Bits | 29192 | 61480 | 94248 | 127080 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 | CBs | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | CBs | 4 | 8 | 12 | 16 |
| Binary Channel Bits per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | Bits | 39024 | 81792 | 124560 | 168912 |
| Max. Throughput averaged over 1 frame | Mbps | 26.273 | 55.332 | 84.823 | 114.37 |
| NOTE 1: There are 14 symbols in a SL slot. Considering AGC and GAP symbol, 12 symbols are allocated to PSSCH transmission. NOTE 2: 10RBs and 3 symbols allocated to PSCCH transmission NOTE 3: 2symbols allocated to PSSCH DMRS. FDM between PSSCH DMRS and PSSCH is supported NOTE 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 5: Slot i is slot index per frame |  |  |  |  |  |

Table A.7.3-2 Fixed reference channel for V2X receiver requirements (SCS 30 kHz, 64QAM)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 |
| Subcarrier spacing configuration |  | 1 | 1 | 1 | 1 |
| Allocated resource blocks |  | 24 | 51 | 78 | 106 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 10 | 10 | 10 | 10 |
| MCS Index |  | 24 | 24 | 24 | 24 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| Maximum number of HARQ transmissions |  | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | Bits | 12808 | 28680 | 45096 | 61480 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 | CBs | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | CBs | 2 | 4 | 6 | 8 |
| Binary Channel Bits per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | Bits | 16848 | 38232 | 59616 | 81792 |
| Max. Throughput averaged over 1 frame | Mbps | 11.527 | 25.812 | 40.586 | 55.332 |
| NOTE 1: There are 14 symbols in a SL slot. Considering AGC and GAP symbol, 12 symbols are allocated to PSSCH transmission. NOTE 2: 10RBs and 3 symbols allocated to PSCCH transmission NOTE 3: 2symbols allocated to PSSCH DMRS. FDM PSSCH between DMRS and PSSCH is supported NOTE 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 5: Slot i is slot index per frame |  |  |  |  |  |

Table A.7.3-3 Fixed reference channel for V2X receiver requirements (SCS 60 kHz, 64QAM)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 60 | 60 | 60 | 60 |
| Subcarrier spacing configuration |  | 2 | 2 | 2 | 2 |
| Allocated resource blocks |  | 11 | 24 | 38 | 51 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 10 | 10 | 10 | 10 |
| MCS Index |  | 24 | 24 | 24 | 24 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| Maximum number of HARQ transmissions |  | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | Bits | 4864 | 12808 | 21000 | 28680 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 | CBs | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | CBs | 1 | 2 | 3 | 4 |
| Binary Channel Bits per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | Bits | 6552 | 16848 | 27936 | 38232 |
| Max. Throughput averaged over 1 frame | Mbps | 4.3776 | 11.527 | 18.9 | 25.812 |
| NOTE 1: There are 14 symbols in a SL slot. Considering AGC and GAP symbol, 12 symbols are allocated to PSSCH transmission. NOTE 2: 10RBs and 3 symbols allocated to PSCCH transmission NOTE 3: 2symbols allocated to PSSCH DMRS. FDM between PSSCH DMRS and PSSCH is supported NOTE 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 5: Slot i is slot index per frame |  |  |  |  |  |

## A.7.4 FRC for maximum input level for 256QAM

For V2X transmission over PC5, Table A.7.4-1, Table A.7.4-2 and Table A.7.4-3 are applicable for Maximum input level when the 256QAM is supported.

Table A.7.4-1 Fixed reference channel for V2X receiver requirements (SCS 15 kHz, 256QAM)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  | 52 | 106 | 160 | 216 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 9 | 9 | 9 | 9 |
| MCS Index |  | 23 | 23 | 23 | 23 |
| MCS Table for TBS determination | 256QAM |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| Maximum number of HARQ transmissions |  | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 1,2,3,4,5,6,7,8,9 | Bits | 40976 | 83976 | 129128 | 176208 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 | CBs | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | CBs | 5 | 10 | 16 | 21 |
| Binary Channel Bits per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | Bits | 52032 | 109056 | 166080 | 225216 |
| Max. Throughput averaged over 1 frame | Mbps | 36.878 | 75.578 | 116.22 | 158.59 |
| NOTE 1: There are 14 symbols in a SL slot. Considering AGC and GAP symbol, 12 symbols are allocated to PSSCH transmission. NOTE 2: 10RBs and 3 symbols allocated to PSCCH transmission NOTE 3: 2symbols allocated to PSSCH DMRS. FDM between PSSCH DMRS and PSSCH is supported NOTE 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 5: Slot i is slot index per frame |  |  |  |  |  |

Table A.7.4-2 Fixed reference channel for V2X receiver requirements (SCS 30 kHz, 256QAM)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 |
| Subcarrier spacing configuration |  | 1 | 1 | 1 | 1 |
| Allocated resource blocks |  | 24 | 51 | 78 | 106 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 10 | 10 | 10 | 10 |
| MCS Index |  | 23 | 23 | 23 | 23 |
| MCS Table for TBS determination | 256QAM |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| Maximum number of HARQ transmissions |  | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | Bits | 17424 | 39936 | 61480 | 83976 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 | CBs | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | CBs | 3 | 5 | 8 | 10 |
| Binary Channel Bits per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | Bits | 22464 | 50976 | 79488 | 109056 |
| Max. Throughput averaged over 1 frame | Mbps | 15.682 | 35.942 | 55.332 | 75.578 |
| NOTE 1: There are 14 symbols in a SL slot. Considering AGC and GAP symbol, 12 symbols are allocated to PSSCH transmission. NOTE 2: 10RBs and 3 symbols allocated to PSCCH transmission NOTE 3: 2symbols allocated to PSSCH DMRS. FDM between PSSCH DMRS and PSSCH is supported NOTE 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 5: Slot i is slot index per frame |  |  |  |  |  |

Table A.7.4-3 Fixed reference channel for V2X receiver requirements (SCS 60kHz, 256QAM)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 60 | 60 | 60 | 60 |
| Subcarrier spacing configuration |  | 2 | 2 | 2 | 2 |
| Allocated resource blocks |  | 11 | 24 | 38 | 51 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 10 | 10 | 10 | 10 |
| MCS Index |  | 23 | 23 | 23 | 23 |
| MCS Table for TBS determination | 256QAM |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| Maximum number of HARQ transmissions |  | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | Bits | 6784 | 17424 | 29192 | 39936 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 | CBs | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | CBs | 1 | 3 | 4 | 5 |
| Binary Channel Bits per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | Bits | 8736 | 22464 | 37248 | 50976 |
| Max. Throughput averaged over 1 frame | Mbps | 6.1056 | 15.682 | 26.273 | 35.942 |
| NOTE 1: There are 14 symbols in a SL slot. Considering AGC and GAP symbol, 12 symbols are allocated to PSSCH transmission. NOTE 2: 10RBs and 3 symbols allocated to PSCCH transmission NOTE 3: 2symbols allocated to PSSCH DMRS. FDM between PSSCH DMRS and PSSCH is supported NOTE 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 5: Slot i is slot index per frame |  |  |  |  |  |

<End of Change>